| ##### Vessel: |  | ##### Place |  |
| --- | --- | --- | --- |
| ##### Auditor: |  | ##### Date: |  |
| ##### Qualification and Experience of Auditor: |  |  |  |

Notes: - Bridge Procedure Audit Checklist is required to be completed by Master within 15 days of joining the vessel and end of each quarter (31st March, 30th June, 30th Sept & 31st Dec). - If the end of quarter date is within 15 days of last audit completed by the newly joined master, audit can be skipped and same to be notified to the concerned Marine Superintendent. - When used as an Addendum to the HSSEQ Internal Audit checklist, Non-conformity or observations raised shall be recorded in Shore Form A1 and A2. In such case, make reference to Form A1 and A2 in section “audit results” at the end of this checklist. - After completion of audit de-briefing meeting to be carried out and audit results discussed.

Opening meeting Prior to commencement of the audit a pre-audit meeting was held between the Auditor & Auditee’s. The purpose and terms of the audit was defined, and the arrangement of the audit were reviewed and agreed. The following persons attended the opening meeting: Date: ____________ From (Time): ____________ To (Time): ______________ 1 Name / Rank: ___________________________________ 2 Name / Rank: ___________________________________ 3 Name / Rank: ___________________________________ 4 Name / Rank: ___________________________________

| ##### Date of last audit: |  | ##### Place: |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ##### Have all the observations / NC noted during the last internal audit / office / master closed? (If Nil observation raised, mark as NA) All the observations / NC are required to be physically verified that they are closed. |  |  | ##### Yes | ##### No | ##### NA |
| ##### Pending / Open Observations (if any) 1. 2. Notes: Open Observations to be included in the Audit results. |  |  |  |  |  |

| ##### 1 | ##### Passage planning – Critical operations | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 1.01 | Is passage plan prepared by 2/O on Latest revision of NAV-01? | FOM 3.6 |  |  |
| 1.02 | Is passage plan verified and approved by the Master? | FOM 3.6 |  |  |
| 1.03 | Is passage plan verified by other members of the bridge team? | FOM 3.6 |  |  |
| 1.04 | Is the passage plan briefing done by the bridge team and date recorded in Form No.NAV-01? | FOM 3.3 |  |  |
| 1.05 | Is the passage plan comprehensive & cover full voyage berth to berth? - Master declaration as per stated in checklist A has been signed by master - Passage plan checklist A and B completed including marking of Environmental sensitive areas - Additional remarks on anticipated Weather & Sea conditions, Traffic conditions, Ship operations etc & Master’s Comments on same are available and voyage specific not generic. - All departure & arrival port information, Berth to Pilot & Pilot to Berth Plan is available. - Implementation checklist has been completed by duty officers prior to change of charts. - Passage plan checklist C completed including Safety settings sheet for Departure, Sea passage and Arrival port. - Form NAV 02 has been completed - Form NAV 02A has been completed | FOM 3.6 /NAV-01/ NAV-02/ NAV-02A |  |  |
| 1.06 | Is the passage plan realistic with regards to Bridge level being recorded? | FOM 3.9 |  |  |
| 1.07 | Confirm that appropriate entries have been made for each leg corresponding to: - Bridge watch level – Is it realistic - Position Fixing and verification frequency & method are as per company procedure - Maximum Speed - T & P Notice – Have same been plotted - NAVTEX Warnings – Have same been plotted T & P notices and NAVTEX warnings are plotted within 15 NM either side of the track. - NAV Aids, restrictions, weather, traffic density etc. - Safety setting selected in the waypoint sheet is realistic with regards to the leg. | NAV-01 |  |  |
| 1.08 | The following should be marked on the chart/ENC, where it enhances safe navigation: - Parallel indexing (not from floating objects unless they have been first checked for position); - Chart changes (not applicable for ECDIS ships) - Methods and frequency of position fixing; - Prominent navigation and radar marks; - No-go areas (the excessive marking of no-go areas should be discouraged) - Landfall targets and lights, Clearing lines and bearings; - Transits, heading marks and leading lines; - Significant tides or current; - Safe speed and necessary speed alterations; - Changes in machinery status; - Minimum under keel clearance; - Positions where the echo sounder should be activated; - Crossing and high-density traffic areas; - Safe distance off; - Anchor clearance; - Contingency plans; - Abort positions; - Wheel over position - VTS and reporting points, etc. - Tidal streams | NAV-01 |  |  |
| 1.09 | Has the UKC been calculated & latest revision of Form NAV-02 filled up for every Port? Note: Heel to be taken into account for small vessels specially LPG vessels which are tender ship and may list while turning, check for heel under other allowance in NAV-02. | FOM 3.33 |  |  |
| 1.10 | Is total tide being used for calculating UKC? |  |  |  |
| 1.11 | Is appendix deviation being completed for any deviation from the planned passage? | NAV-01 |  |  |
| 1.12 | Is risk assessment and contingency plan for each leg completed for arrival and departure port? | NAV-01 |  |  |

| ##### 2 | ##### Position fixing | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 2.01 | Is position fixing frequency / method marked on each chart / ENC and as per the passage plan? | FOM 3.6 |  |  |
| 2.02 | Position fixes have been cross - checked using independent sources of information. (Radar fixes & GPS fix, Radar ranges & Visual bearings etc) Note: For vessels with ECDIS as primary means of navigation is 2nd GPS used for position verification, where no Radar / Visual fix is available. | FOM 3.6 |  |  |
| 2.03 | Prior plotting GPS position, the corrections for satellite derived position (based on WGS 84), as applicable have been considered. Verify values of Dilution of precision (DOP) and Signal to noise ratio (SNR) are recorded in NAV-21. | FOM 3.23 Section 10 |  |  |
| 2.04 | Are officers using parallel index to track vessels movement? | FOM 3.6 |  |  |
| 2.05 | Is position fixing on passage plan realistic For paper charts – distance from nearest danger (shoal, less depth, land) / speed = time/2 In case the distance is such that the calculated time is less than 10 minutes, has parallel indexing been used? For ECDIS Vessels: Ref. FOM 3.6 Section 8.1 | FOM 3.6 |  |  |
|  | ##### Remarks |  |  |  |

| ##### 3 | ##### Following details are displayed, and all Officers familiar with the contents | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 3.01 | Manoeuvring characteristics (IMO Res A601(15)) and Wheelhouse poster (IMO Res A 601(15) & BPG). Note: Ensure OOW understand that they have Unrestricted access to M/Eng. |  |  |  |
| 3.02 | Auto to Manual Steering Change over procedure, Emergency Steering Changeover procedures / block diagrams. Are all navigating officers and ratings aware of the change-over procedure from Auto to Manual steering and also the method of engaging NFU mode of steering and the meaning of and reason for this mode? Are all navigating officers and A/B's aware of the method of engaging and using emergency steering? | FOM 3.23 Section 5 |  |  |
| 3.03 | Master’s Standing orders posted, countersigned by all officers for ‘read & understood’. Company standing Orders (FOM 3.8) displayed on bridge, OOW aware of company minimum CPA/TCPA policy? Master’s Standing orders should not conflict with SMS. | FOM 3.9 & FOM 3.8 |  |  |
| 3.04 | Poster 004 - Bridge Poster. |  |  |  |
| 3.05 | Poster 007 – Smoke comparison chart (Entries recorded daily). |  |  |  |
| 3.06 | Poster 024 – Steaming watch condition & tasks. |  |  |  |
| 3.07 | Deviation card – (Do the errors being recorded in compass error book broadly agree with deviation card) – Last date done ________. | FOM 3.2.3 Section 6 |  |  |
| 3.08 | Bridge Watch Schedule as per ILO format – This schedule to also include Lookout timings. | FOM 3.8 |  |  |
| 3.09 | Squat table for open and confined waters on various speeds and draft. | FOM 3.33 |  |  |
| 3.10 | Access to SMS Procedures confirm Master aware that during blackout situation ECP Procedures to follow from hard copy files. |  |  |  |
| 3.11 | Certificate for auto supplier for Charts/ENC & Publications |  |  |  |
| 3.12 | Poster 10/10A/B (Emergency Plan) as applicable and Muster list |  |  |  |
| 3.13 | Are the following displays posted as required: - Visibility diagram (If not included in manoeuvring characteristics) - IMO pyrotechnics and emergency signals - IMO guidelines for required boarding arrangements for Pilot - Williamson turn (If not included in manoeuvring characteristics) - Company policy - Poster 13A as assigned to each business unit (Also includes company's open reporting procedures) - Crew list - Lifejacket/ TPA/ EEBD/ immersion suit donning instructions - Radar blind sector diagram posted in a conspicuous location in the vicinity of the Radars |  |  |  |
| 3.14 | Officers are aware of following: - Bridge Access Control - Distraction Policy - VDR Data Download Policy | FOM 3.3 FOM 3.23 |  |  |
|  | ##### Remarks |  |  |  |

| ##### 4 | ##### Logs & records | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 4.01 | Deck log book - Bridge Watch Levels as per Poster 024 being maintained adequately and documented (watch level & look out names are recorded) - Look out being maintained as per the Requirement – - AB 1 – 1000-1400 / 2200 – 0200 - AB 2 – 1400-1800 / 0200 – 0600 - AB 3 – 0600-1000 / 1800 – 2200 - Is manual steering used during periods of river transits and when navigating through restricted waters? The time and position when hand steering was engaged recorded in the logbook / bell book - Fire & Safety round is completed every 2 hourly during hours of darkness between 2000-0600 by the outgoing officer of watch and A/B as per watchkeeping schedule. Verify that the records are maintained in deck logbook. l | FOM 3.15 |  |  |
| 4.02 | Bell book maintained in good order, entries are made such that Following being recorded. - End & Commencement of Sea Passage - Test & Checks prior arrival & Departure as per USCG 164.25 & other appropriate industrial requirements - Pilot on board - Passage of Buoys, important land marks - Main times of mooring/unmooring the vessel (first line ashore, all fast, all clear) - M/E RPM / Pitch (if auto logger not provided) - Con take over/ hand over, Control test, M/E test etc. - Master Pilot information exchange. | FOM 3.15 |  |  |
| 4.01 & 4.02 | Notes: Information which should be recorded includes that concerning position, course and speed, the times and positions when passing waypoints, land or sea marks, weather and sea conditions and incidents and events including pilot embarkation/disembarkation, times of attendance and connection and disconnection of tugs, times of berthing and un berthing, hazardous occurrences and accidents. |  |  |  |
| 4.03 | Are compass errors being taken and logged at least once a watch except when this is not possible due to the state of visibility or weather? Note: A note should be made in the deck log book or compass error log when the error is not obtained. | FOM 3.23 Section 6 |  |  |
| 4.04 | Master’s night order (also known as Bridge order book) being written daily at sea & at anchorage. It should be signed for ‘read & understood’ by all officers. | FOM 3.9 Section 10 |  |  |
| 4.05 | Echo sounder - Date / time and position being marked when the equipment is switched on (especially when passing critical area where UKC is of concern). - To be marked when passing significant land / seamarks. - Critical passage during Arrival and departure print out to be taken and filed. For electronic unit - Are the procedures been posted as to how to retrieve the old information? | FOM 3.23 Section 8 |  |  |
| 4.06 | Course recorder - Date / time marked for prior arrival / departure port - Date / time / position during noon every day - Is the course recorder set to UTC? - Is the unit always kept on during sailing & at anchor? | FOM 3.23 Section 14.3 |  |  |
| 4.07 | Radar Log - Radar performance monitor readings being taken every watch and logged in the Radar log? Are they within the acceptable range? For the ARPAs, is there a notice posted that speed input should only be through the water not ground? Are all officers aware of the reason for this? | FOM 3.23 Section 4 |  |  |
| 4.08 | All logs being maintained in good order and being verified by Master – Compass error log / VHF log / Deck log book / Master standing orders / Night orders / Bell book / GPS / Echo sounder / Anchor Log Book / GMDSS Log Book etc. |  |  |  |
|  | ##### Remarks |  |  |  |

| ##### 5 | ##### Bridge checklists – Completed for each voyage, port | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 5.01 | Bridge & ECDIS Familiarization completed for all officers on-board | NAV-06 |  |  |
| 5.02 | Are the Following checklist completed as applicable 1. Bridge daily tests and checks | NAV-13 |  |  |
|  | 2. Bridge preparation for sea | NAV-07 |  |  |
|  | 3. Bridge embarkation of pilot (To be completed on every occasion pilot boards including change of pilot) | NAV-03 |  |  |
|  | 4. Pilot card (To be completed on every occasion pilot boards including change of pilot) - Min UKC Clearance calculation / NAV-02 also signed by Pilot - Maximum bollard pull from tugs | NAV-04 |  |  |
|  | 5. Master – Pilot information exchange (To be completed on every occasion pilot boards including change of pilot) | NAV-05 |  |  |
|  | 6. Coastal congested water and TSS | NAV-10 |  |  |
|  | 7. Changing over of the watch & ECDIS changeover watch | NAV-16 & NAV-11a |  |  |
|  | 8. Preparation for arrival port | NAV-08 |  |  |
|  | 9. Anchoring and anchor watch | NAV-19 |  |  |
|  | 10. Restricted visibility | NAV-17 |  |  |
|  | 11. Heavy weather or storms | NAV-09 |  |  |
|  | 12. Ice or extreme cold conditions | NAV-18 |  |  |
|  | 13. Helicopter safety checklist | NAV-12 |  |  |
| 5.03 | Deck log book entry is made upon completion of checklists as above. |  |  |  |
|  | ##### Remarks |  |  |  |

| ##### 6 | ##### Charts/ENC & publication – Correction & maintenance | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 6.01 | Are all recommended charts (including largest scale) and publications such as relevant Admiralty Sailing Directions (Pilots) and List of Lights, Tidal Tables Etc available for the current voyage and are they of the latest edition? | FOM 3.6 |  |  |
| 6.02 | Status (Edition / corrections) of Charts / ENC maintained in NP 133 / NP 133C or similar appropriate publication. | FOM 3.29 |  |  |
| 6.03 | All Charts / ENC corrected to the latest notice received on-board NTM ____________ | FOM 3.13 |  |  |
| 6.04 | T & P Notices / AIO – Plotted for the Voyage charts / ENC, File for all areas to be maintained on vessels using paper charts or ECDIS not compatible for AIO. Check NAV-25 for ECDIS T&P correction log is being maintained. | FOM 3.6 |  |  |
| 6.05 | NAVTEX & NAVAREA Warnings – Plotted for the Voyage charts / ENC, File maintained for all areas. - NAVTEX messages if displayed on ECDIS, check that they are read and where required plotted on the ENC (like co-ordinates for gunnery warnings etc.) - NAVTEX & NAVAREA Warnings to be signed by the OOW, navigating officer and if effecting the passage also by the Master. Check NAV-25 – ECDIS T&P correction log is being maintained. | FOM 3.6 |  |  |
| 6.06 | Publications have been corrected to the latest notice received on-board - List of Lights & Fog Signals - List of Radio Signals - Sailing directions - ATT Where electronic publications are fitted. Is primary & backup computer for Admiralty digital publications/e-NPs operational & updated to latest weekly updates received on-board? | FOM 3.6 /FOM 3.13 |  |  |
| 6.07 | All publications maintained as per latest version notification by office. | FOM 3.6 |  |  |
| 6.08 | Vessel operating with paper charts as primary or as back-up to ECDIS, Print – outs of following are maintained: Weekly NTM’s including tracing Cumulative notices |  |  |  |
|  | ##### Remarks |  |  |  |

| ##### 7 | ##### Practical application of bridge procedures |  | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| 7.01 | Are all Navigation equipment in good working condition? Any bridge equipment which is not functioning must be recorded as a non-observance? |  |  |  |
| 7.02 | Is the standard magnetic compass operational, properly maintained and adjusted? - Ship’s heading clearly displayed at the main steering position. - Binnacle lights are operational. - Compass is provided with an azimuth mirror or other means to take bearings. - Compass is adjusted by a qualified compass adjuster - If a period of two years has elapsed since the last adjustment and a record of compass deviations has not been maintained, - Or the recorded deviations are excessive (more than 5 degrees) or when the compass shows physical defects. - A compass deviation card should be prepared by compass adjuster each time the compass is adjusted, if any of the above condition apply, and if adjustment not required then master shall carry out verification annually by carrying out a swing and preparing Deviation card. - Separate deviation cards should be prepared for the standard compass and the transmitting magnetic compass repeater, if fitted. - Is the auto pilot capable of steering on magnetic compass? If so, is this tested at least weekly? | FOM 3.23 Section 6 |  |  |
| 7.03 | Is the gyro compass operating satisfactorily? - Speed and latitude corrections are properly applied. - Gyro maintenance / service records are maintained as per makers instructions - Each of the gyro repeaters, including the ones in the emergency steering position and the engine control room are synchronised. - Where two gyros are fitted, a change-over device is fitted and change-over procedures are posted, and Gyro in use is recorded in the compass error log. | FOM 3.23 Section 6 |  |  |
| 7.04 | Operation of AIS to be checked and verified. - The LOW power as per the instruction manual is 1W or else the Equipment to be switched off. - Are changeover from 12.5W to 1W instruction posted near the AIS - If Low watt then verify from the maker manual if LOW WATT is 1W or 2W. - Is AIS integrated with the Radar and if not can the AIS be seen from the Radar position | FOM 3.23 Section 20 |  |  |
| 7.05 | Is vessel fitted with Weather Routing system ? |  |  |  |
| 7.06 | VDR instruction have been posted near the VDR and are all watch-keeping officers familiar with this? | FOM 3.23 Section 21 |  |  |
| 7.07 | If Chartco fitted are officer fully aware as to the use of the equipment: - Are the daily checks as recommended by Chartco being done and recorded - Are all charts entered in the active folio - Have the officers completed CBT training |  |  |  |
| 7.08 | If fitted with BNWAS: Is the bridge navigational watch alarm system in operation whenever the ship is underway at sea or at anchor? The date, time and position the equipment was switched on / off shall be logged in the ship’s log book. Check last test records of 3-stage alarm and power failure alarm. | FOM 3.23 Section 23 |  |  |
| 7.09 | 1. Are all navigating officers aware of the Company's minimum UKC policy? (Carry out Random checks, comparisons between Passage Plans and Echo Sounder log/ printout). 2. Do officers know what constitutes Open waters / shallow waters? | FOM 3.33 |  |  |
| 7.10 | While transiting the Singapore and Malacca Straits has vessel complied with FOM 3.7 & FOM 3.10 ? |  |  |  |
| 7.11 | Confirm if the Master and Navigating officers aware of: 1. Navigational Hazardous Areas and the associated hazard in these areas 2. Particularly Sensitive Sea Areas 3. Marine Environmental High-Risk Areas (MEHRAs) | FOM 3.5 |  |  |
| 7.12 | Are deck officers aware of the various methods of anchoring? Note: Methods of Anchoring refers to walking Back Anchor, letting go on Brake. | FOM 3.38 |  |  |
| 7.13 | Are anchor watches properly maintained while vessel is at anchor? | FOM 3.9 |  |  |
| 7.14 | Are the Navigating officers aware of the precautions to prevent dragging of anchor and the actions to be taken in case the vessel drags anchor? | FOM 3.3 Section 10, Poster 004 |  |  |
| 7.15 | Are all lights, shapes and signalling equipment in good working order? | FOM 3.23 Section 15, 16 & 17 |  |  |
| 7.16 | Are watch-keeping officers aware of the procedure to be followed in the event of failure of navigational equipment and have Navigational Risk Assessments been carried when appropriate? Note: Master to be informed, Risk assessment to be carried out before entering or leaving port, authorities to be informed, Company to be informed, urgent repairs to be arranged, dispensation to be obtained from Flag if repairs cannot be done and Class to be informed, Entry to be made in TDL E24 or on DSM | FOM 1.1 |  |  |
|  | Remarks: |  |  |  |

| ##### 8 | ##### For vessels fitted with ECDIS | ##### Ref | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
|  | ##### Is the fitted ECDIS ‘type approved’ and included as a part of Cargo Ship Safety Equipment Form E. Note: If fitted for Reference only, instructions are posted stating ONLY REFERENCE AND NOT USED FOR NAVIGATION PURPOSES. |  |  |  |
| 8.01 | ##### Are ECDIS procedures from SMS printed and available for quick reference? The document to be signed for ‘read and understood’ by all navigating Officers |  |  |  |
| 8.02 | ##### Each deck watch keeper is in possession of an ECDIS certificate of training. As per IMO Model Course 1.27, (The operational use of Electronic Chart Display and Information Systems (ECDIS). | FOM 3.29 Section 8 |  |  |
| 8.03 | ##### Each deck watch keeper is in possession of a Type specific ECDIS certificate of training or an evidence of familiarisation of the actual equipment fitted on board. (The Type specific training should be conducted by the manufacturer or the manufactures approved agent or a trainer who has attended such a programme) Has the senior officer gone through the Type-specific equipment training for the SAM and Raytheon Equipment? If for reason shore-based familiarization training cannot be completed, then the following option to be considered prior the officer taking over the 1st watch: - Self Familiarization of equipment with the aid of user manual and a company generated and approved checklist identified within the Bridge procedures. - Instruction on the vessel by a suitable experienced and able bridge officer who has attended the manufactures training. - Shore staffs who have attended the operator’s course to visit the vessel and conduct on board type specific familiarization. | FOM 3.29 Section 8/ NAV-16 |  |  |
| 8.04 | ##### Each deck watch keeper is fully familiar with the different functions of the equipment ##### Familiarization with available functions ##### Familiarization with the Menu structure ##### Recognition of alarms and malfunction indicators and the actions to be taken. ##### Display set up - Setting of Safety values ##### Route monitoring ##### Changing over to back up system ##### Route planning ##### Updating of software. ##### Loading charts and licences - Position plotting using ‘DR Mode’ in case of loss of GPS Signal. |  |  |  |
| 8.05 | ##### Verify Officers are aware of various overlays on ECDIS (RADAR, AIS, NAVTEX etc.) Note: Verify Bridge Team understands danger of over-reliance on the overlays provided. | FOM 3.29 |  |  |
| 8.06 | ##### Is the voyage plan loaded before commencement of the voyage and is operational at all times especially when the ship is in coastal water for both the primary and secondary means? Is the "route check" run on the ECDIS to identify possible hazards, prior commencing the voyage? | FOM 3.6 & FOM 3.29 |  |  |
| 8.07 | ##### Are the following checks being made to avoid over-reliance on ECDIS? - Parallel indexing and use of clearing bearings; - Use of radar to check the accuracy of the charted position by comparing the location of the radar target against the charted symbol; - Visual cross bearings; - Comparison of the signal to noise ratio of the GPS system in use. | FOM 3.29 |  |  |
| 8.08 | ##### If the paper charts are the recognised back-up system, are the paper charts provided, adequate for the areas in which the ship trades and are they fully corrected? | FOM 3.6 |  |  |
| 8.09 | ##### Is the vessels able to comply with all Passage planning requirements as laid out under section 1 on the ECDIS / ECS systems? |  |  |  |
| 8.10 | ##### Are the Sensors correctly installed and aligned? - The alarm is functioning and not muted - There is independent power supply to each individual ECDIS - There is power backup available to ECDIS - The USB drive and/ or DVD drive is functional - There is dedicated USB used for data inputs in the ECDIS Note: Confirm there is no error messages displayed on ECDIS of serious nature, e.g. overdue ENC corrections or failure of IMO mandatory sensor data (GPS/ Gyro/ Log). | FOM 3.29 Section 9.2 |  |  |
| 8.11 | ##### Is the ECDIS software being maintained? | FOM 3.29 Section 9.1 |  |  |
| 8.12 | ##### Are the Electronic charts in use Up-to-date (latest edition and NM)? Note: In areas with no ENC coverage, are paper charts of the largest available scale available on board. Verify if latest Base DVD available on-board. | FOM 3.6 |  |  |
| 8.13 | Are the correct alarm settings in use in relation to vessel's draft? | FOM 3.29 Section 5 |  |  |
| 8.14 | Are all officers aware of the action to be taken in the event of ECDIS failure? | FOM 3.23 Section 10 |  |  |
|  | ##### Remarks |  |  |  |

| ##### 9 | ##### Radio log / GMDSS |  | ##### YES | ##### NO |
| --- | --- | --- | --- | --- |
| ##### 9.01 | ##### Is all GMDSS equipment operating satisfactorily and is the GMDSS Radio Log properly maintained? | ##### FOM 3.23 Section 13 / FOM 3.15 & FOM 3.14 |  |  |
| ##### 9.02 | ##### Vessel’s Name / Call Sign / Inmarsat Ship Station Identity / MMSI no. (Near the GMDSS Equipment). |  |  |  |
| ##### 9.03 | ##### Instructions for operating DSC & Satellite communication in an emergency. |  |  |  |
| ##### 9.04 | ##### Is the GMDSS station kept earthed during cargo, tank cleaning and/or venting operations, and are officers aware of the procedure to do so? ##### Note: this applies to tankers only. |  |  |  |
| ##### 9.04 | ##### Are officers aware of the function of the ship security alert system and how it operates? | ##### FOM 3.23 Section 19 |  |  |
| 9.05 | Is the radio log being maintained correctly? Radio Log book correctly filled and test carried out as per SOLAS AND STCW requirement for GMDSS test procedures - SOLAS IV-17 / STCW A-VIII/2 part3-3 :87-89 / STCW B-VIII/2: 10/12/14/17/33 and the requirement as stated in the Daily, weekly and Monthly checks. | FOM 3.15/ FOM 3.14 |  |  |
| 9.06 | Has the Person in-charge of Radio communication in an emergency being identified and mentioned in the master list / Radio log? |  |  |  |
| 9.07 | Daily / Weekly / Monthly test. On load test and annual discharge of GMDSS batteries (NAV-24) been conducted and logged. | FOM 3.14 |  |  |
| 9.08 | Is pre-sailing test being conducted and being recorded in the radio log book? | FOM 3.14 |  |  |
| 9.09 | Is the radio log book being inspected and signed for each days entry by the Master? | FOM 3.15 |  |  |
| 9.10 | Position updates are made, as applicable for the VHF, DSC & satellite equipment. |  |  |  |
| 9.11 | Confirm by self-test that the following equipment is in good order. SART – on 3 cm Radar EPIRB - Expiry of Hydrostatic release and Self-test functions are known to the officer? 2 Way Radio – on any channel other than Ch 16 NAVTEX | FOM 3.23 Section 13 |  |  |
| 9.12 | Verify all Bridge officers understand and can demonstrate how to initiate a distress alert on all equipment’s and conduct self-test of all equipment (verify the operations of the MF or MF/HF controller by means of a test call on MF and /or HF to a coast station). | FOM 3.23 Section 13.3 |  |  |
| 9.13 | Are radio emergency batteries in a satisfactory condition & fully charged – State if these are maintenance free batteries? If maintenance free batteries confirm that fully charged indicator is in green. | FOM 3.14 |  |  |
| 9.14 | Are the following posted near communication equipment as required? - Copy of the shore-based equipment maintenance agreement. - Radio Station Licence. - Safety Radio Certificate with attachment. - Hard, colour copy of 'GMDSS operating guidance for Ship Masters in Distress situations'. - Vessel-specific Antennae arrangement plan posted? Is the plan updated and current? - Emergency notification and contact numbers (Company, Flag and Port) |  |  |  |

| ##### 11 | ##### Others | ##### YES | ##### NO |
| --- | --- | --- | --- |
| ##### Remarks |  |  |  |

| ##### Evaluation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ##### All Bridge team members who were a part of the Bridge Audit are required to evaluated in regards to below ##### Evaluation from 1 to 5 ##### 5) VERY GOOD 4) GOOD 3) SATISFACTORY 2) FAIR (AVERAGE) 1) POOR |  |  |  |  |  |
|  | ##### Familiarity with navigation policy and Master’s over riding authority | ##### Familiarity with Navigation procedures | ##### Operation of Navigation equipment | ##### Operation of GMDSS / Communication equipment |  |
| ##### Master: | ##### Not Applicable |  |  |  |  |
| ##### C/Officer: |  |  |  |  |  |
| ##### 2/Officer: |  |  |  |  |  |
| ##### 3/officer: |  |  |  |  |  |
| ##### A/Officer: |  |  |  |  |  |

| ##### Audit Result: |  |  |  |  |
| --- | --- | --- | --- | --- |
| ##### S. no. | ##### Type of Deviation A | ##### Finding | ##### Reference B | ##### Due Date C |
|  |  |  |  | 90 Days |
|  |  |  |  | 90 Days |
|  |  |  |  | 90 Days |
|  |  |  |  | 90 Days |
|  |  |  |  | 90 Days |

| De-briefing Completed |  | Date: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Time (From – To): |  |  |  |  |  |  |
| ##### Remarks (Master to enter remarks if applicable): |  |  |  |  |  |  |  |  |
| (Issued By / Auditor) |  |  |  |  | (Master) |  |

##### Notes:

A. Definitions

1. Non-conformity - An observed situation where objective evidence indicates the non-fulfilment of a specified requirement. Observations recorded during a PSC & Flag inspection are recorded as Non conformity

2. Major non-conformity - An identifiable deviation that poses a serious threat to the safety of personnel or the ship or a serious risk to the environment that requires immediate corrective action and includes the lack of effective and systematic implementation of a requirement of ISM Code. If a MNC is recorded during any audit, it is required to inform DPA / advice an immediate corrective action, so to allow the MNC to be graded as Non conformity before the vessel sails out of the port.  While listing the MNC, the immediate corrective action implemented to grade the noted deficiency as a minor non conformity is also required to be noted.

B. Reference

Please cite the SMS Document number where the requirement is found.  Also quote whether related to ISM / ISPS Audit

C. Due Date

Due date shall be 90 days.

D. These observations / deficiencies to be recorded in E24 for follow up.