| NAV-18 |  | Form No.: NAV-18 |
| --- | --- | --- |
|  |  | Revision: 01 Date: 15 Oct 2024 |
|  | Checklist for Operation in extreme winter and ice region | Issued by: DPA Approved by: MD |
|  |  | Page: 1 of 4 |

| Date: |  | Vessel Name: |
| --- | --- | --- |

| # | Item | Yes | No |
| --- | --- | --- | --- |
| Notification Section |  |  |  |
| 1 | Have the following been informed of the ice or extremely cold conditions? |  |  |
| a | All the departments - Deck, Engine, and Saloon. |  |  |
| b | Vessel Manager and Marine Superintendent / Owners / Head Charterers. |  |  |
| 2 | Has company been advised of the following? |  |  |
| a | Status of equipment, stores, spares, PPE, provisions, and FW onboard for the anticipated Voyage. |  |  |
| b | Freezing conditions/ice expected to be encountered. |  |  |
| c | Insurance cover for the intended voyage. Vessel's ice-class notation checked. |  |  |
| d | Loading / discharging / Ballast condition with limitations, if any. |  |  |
| Crew Safety – Preparation and Operational Readiness |  |  |  |
| 3 | Has the following been carried out? |  |  |
| a | Risk assessment for the intended voyage carried out and discussed with Marine Superintendent and Vessel manager. |  |  |
| b | Safety meeting & familiarization with precautions to be taken in expected cold conditions. |  |  |
| c | Adequate warm clothing for crew available. |  |  |
| d | Hypothermia and frostbite first aid care training carried out |  |  |
| e | Crew accommodation heating to be tested and checked for operational readiness |  |  |
| f | Walkway on deck / mooring stations / pilot embarkation area / cargo working area / Survival craft access are clear of snow / ice and adequate rock salt sprinkled. |  |  |
| g | Safety lines rigged on deck. |  |  |

| Navigation – Preparation and Operational Readiness |  |  |  |
| --- | --- | --- | --- |
| 4 | Minimum requirements prior to proceeding into cold or ice-bound areas |  |  |
| a | All information required for proper appraisal of situation and route planning gathered |  |  |
| b | Use of weather routing services. Ice charts obtained and discussed with the office as required. |  |  |
| c | As a minimum, the propeller immersion should be 1 meter below the thickness of the ice. Example - Ice thickness 20 cms, then the tip of the propeller should be 1.2 meters below water line. |  |  |
| d | Icebreaker or tug assistance is called for, as required. Services of Ice advisor as required. |  |  |
| e | Reference made to recommended publications on ice procedures as per FOM 3.36. |  |  |
| f | Ballast condition (max propeller immersion & sea chest immersion) |  |  |
| g | Master and Chief Engineer written instructions issued to officers and crew. |  |  |
| h | Lookouts are posted if ice is considered close by and speeds adjusted accordingly. Check search lights operational. |  |  |
| i | Watertight doors closed and ship prepared for oncoming weather. |  |  |
| j | Heaters for Radars and Fog horns are kept 'ON'. Exposed motors for the horns are covered. |  |  |
| k | Radar scanners kept turning in port. Permission obtained to keep the scanners turning. |  |  |
| l | Increase the frequency of sounding tanks and bilges |  |  |
| m | FOM 3.36 DO's and DON'Ts referred to / complied |  |  |
| Cargo - Preparation and Operational Readiness |  |  |  |
| 5 | Mandatory checks prior to proceeding into cold or ice bound areas. Once completed this checklist should be sent to MS for review. |  |  |
| a | Check that anchor wash valves are open. |  |  |
| b | Drain freshwater lines, fire lines, compressed air line. |  |  |
| c | Provision to conduct cargo hold washing in extreme cold. Including supply of propane heaters from charterers |  |  |
| d | Carry out checks of the deck machinery. Top up the hydraulic oil tank for forward and aft mooring winches |  |  |
| e | Carry out a greasing and lubrication routine of open gears and wires, if found dry. |  |  |
| f | Keep oil warm: Before the deck machinery is to be used, start the hydraulic pumps at least 1 hour in advance and allow at least one winch to run idling at slow speed. This will keep the oil warm by circulating. Hatch cover hydraulic system started in advance. Keep the hydraulic oil in circulation and warm. |  |  |
| g | Cranes to be tried to check the moving parts, particularly, pulleys are moving in sheaves. |  |  |

| h | Check ballast air vents and ensure the floats are clear of icing. |  |  |
| --- | --- | --- | --- |
| i | Try out the lifeboats/rescue boat engines. check the fuel tank level. Check lifeboat engine heater (if fitted) is kept on. Ensure that Fresh water is stored in a way to avoid freezing. Ensure sheaves are in properly greased. Try out the engines regularly. and. Regularly check to see the charging condition of lifeboat / rescue boat engine starting batteries. Check operation of signal light. Ensure painter release mechanism is free. |  |  |
| j | Daily temperature log sent to Vessel manager and Marine Superintendent |  |  |
| k | Cargo holds cleaning instructions received from charterers and discussed with the office. |  |  |
| l | Ensure that hatch cover/cleats/trackways all clear of snow prior to operation of the hatch covers. |  |  |
| Cargo - Preparation and Operational Readiness (Tankers) |  |  |  |
| a | This advisory for ships is in addition to cargo heating procedures listed under: OTM 7.13 (Cargo heating)/CTM 9.8 (Cargo heating and cargo sampling)/FOM 1.3 (Ballast operations) |  |  |
| b | Drain N2 line/Tank cleaning heater/Compressed airline/Foam line/Steam line On Deck & Pumproom |  |  |
| c | Monitor Glands: Change any leaky gaskets |  |  |
| d | Follow correct procedure for FRAMO system (As relevant): In case you have a FRAMO system for cargo as well as deck machinery and Bow thrusters, you need to follow the correct procedure for recirculation and pre warming the oil. |  |  |
| e | Check the steam coils: Check the steam coils for accommodation Air conditioning and drain the lines few days in advance. Check that the air damper for fresh air is free and can be moved when required |  |  |
| f | PV Breaker: Check the liquid inside for temperature tolerance of at least -15oc or more if going to more extreme region. Add additional anti-freeze if required and record it in the deck maintenance log. |  |  |
| g | Carry out an inspection of the Deck Seal and the heating coil. Open steam to see that none of the weld- joints or gaskets or valve glands and flange gaskets are leaking. |  |  |
| h | Ensure each heating coil to the tank is completely empty of water. Keep main deck in/out valve and main steam inlet valve to the tank and return condensate valve shut. Keep all the tank valves, including the drain valves open. |  |  |
| i | Drain the ODME sampling pump and pipeline |  |  |
| j | Ensure cargo stripping pump steam supply from Engine room is shut. |  |  |
| k | Tank cleaning heater: Though not much at risk due to proximity with the E/R bulkhead. It still might be a wise thing to drain the steam lines and also lower the Sea Water side to a controlled extent. |  |  |

| l | IG Line Pressure Sampling Line: Shipyards usually fit a 8-10mm line from the IG line to the pressure switch and indicator. This sometimes gets choked if there is condensate in the line. Please open the line from both ends and blow through to ensure clear. PRECAUTION: Do not blow into the pressure switch. |  |  |
| --- | --- | --- | --- |
| m | Record cargo temperature: Regularly if you are carrying heated cargo. It can be done twice from Cargo Control Console if the temperature indicator is reliable. Cold weather if associated with sprays on deck or rough weather brings down the cargo temperature must faster. |  |  |
| n | PV Valve: Check PV valves to ensure they are free of ice and operating properly. |  |  |

Confirmation that all is in order Master:

Name:

Location: