THIS CHECKLIST IS TO BE COMPLETED BY THE CHIEF OFFICER PRIOR TO ARRIVAL IN PORT AND COMMENCEMENT OF CARGO OPERATIONS WITH A DECK LOG BOOK ENTRY MADE.  ANY DEFECTS FOUND, WHICH CANNOT BE RECTIFIED PRIOR TO ARRIVAL, ARE TO BE REPORTED TO THE MANAGEMENT OFFICE IMMEDIATELY AND PERMISSION OBTAINED TO CONTINUE WITH INTENDED CARGO OPERATIONS

| HAVE THE FOLLOWING BEEN UNDERTAKEN: | ## Initials |
| --- | --- |
| Has the ballast water control procedure as required by port state authority been completed? (ref IMO Res. A.774(18)) |  |
| Can the nominated quantity of the cargo be loaded/discharged? |  |
| Is there sufficient water at the intended berth? (Refer to Fleet Operations Navigation Safety Chapter - “Under Keel Clearance”). |  |
| Are the cargo tanks and lines clean, ready and suitable for the intended cargo? |  |
| IF THE CARGO REQUIRES TO BE HEATED: | ## Initials |
| Have the voyage orders been received? |  |
| Have arrangements been made with the Chief Engineer? |  |
| Has the heating system been tested and found to be tight and operational? |  |
| If heater parts are isolated (e.g. by means of flange etc) is it clear what part is isolated and are the parts clearly identifiable? |  |
| Is the steam condensate return routed through steam observation tank in deck area? |  |
| Are the heating coils of the cargo tanks that do not require heating blanked on both inlet & outlet side and valves lashed in closed position in order to avoid unintended heating? |  |

| HAVE THE FOLLOWING CHECKS BEEN CARRIED OUT: | ## Initials |
| --- | --- |
| Has the cargo plan been prepared? |  |
| Will the stresses and stability of the vessel be within acceptable limits during all stages of the operation and voyage? |  |
| Has all PPE been inspected & in good operational condition for intended use? |  |
| Have the portable spool pieces been fitted and unused connection flanges blanked? |  |
| Have the cargo hoses, spool pieces, flanges been tested to 7bar for 15 minutes? |  |
| Where blanks have been installed on the cargo piping have they been recorded and known to those who supervise and undertake work on the cargo system? Installed spade blinds are clearly tagged on the relevant flange? |  |
| Has the emergency cargo pump stop been tested within 24 hours of expected cargo operations and found satisfactory? |  |
| Are loading / discharging lines properly connected, leak-tested and their condition appears satisfactory? Are all unused manifolds blanked? |  |
| Are all manifolds (both sides) fitted with a pressure gauge and are all gauges calibrated up-to-date? |  |
| Has the inert gas system been tested? |  |
| Are the inert gas cargo tank valves locked in the proper position? |  |
| Has the tank venting system been checked and found to be fully operational? |  |
| Are the cargo tank pressure alarms set correctly as per Tanker Procedures OTM 7.4/ CTM 9.4 ? Prior loading is the accuracy of cargo tank pressure sensor output within required tolerance? OTM 7.4/ CTM 9.4 |  |
| Prior loading is the accuracy of cargo tank pressure sensor output within required tolerance? OTM 7.4/ CTM 9.4. |  |
| Are all flame arrestors in good condition? |  |
| Is the inert gas upriser sump drain checked clear and sump empty? |  |
| Have cargo tanks been purged and found to have oxygen content below the required limits? |  |
| Have all sea suction/overboard valves been air tested or blanked? |  |
| Are sea and overboard discharge valves closed, lashed and locked? |  |
| Have pump trips and pumping equipment been tested and pump purge routines been followed (if applicable)? |  |
| Has Fixed Gas Detection System (if fitted) been calibrated, checked, operational and switched it on ? any non operational status of FGDS notify office via email and telephone with defect raised in DSM /// SMS amendment /// |  |
| Is required portable gas detection equipment available, been checked and found to be calibrated and ready for immediate use? |  |
| Are cargo pipelines and valve annual pressure tests up to date? Date of Test____________ |  |
| Have tank gauging systems, high level and tank overfill alarms been tested, found to be operational and properly set? |  |
| Have all cargo procedures been agreed with the shore including the grade sequence and the rates? |  |
| Is a cargo surveyor attending? |  |
| Are emergency procedures understood with the terminal? |  |
| Is the ship’s Radio and Radar equipment isolated / on low power - 1 watt? |  |
| Is the cargo containment system vapour tight? |  |
| Is a valid vapour tightness certificate available? |  |
| Have all cargo tank domes securing dogs/ butterfly nuts is gas and vapor tight? any leaking COT domes to be reported to office /// SMS Amendment /// |  |

| Have all cargo lines, valves and vent lines been properly set and checked by the Chief Officer and cross checked by another officer prior to manifold being connected to shore line? Initial Check; Cross check. Parcel; Date; Time; Rank; Signature; Date; Time; Rank; Signature. | See Table below. |
| --- | --- |
| Have all hose connections been properly set and checked by the Chief Officer and cross checked by another officer prior to coming operations? Initial Check; Cross check. Parcel; Date; Time; Rank; Signature; Date; Time; Rank; Signature. |  |
| Cargo Lines & COW lines pressure tested to working pressure? |  |
| Drain IGS line before cargo operation |  |
| Are all ball valves connected to the cargo systems closed & positively locked and plugs / caps tightly in place? If these are of cam lock type then, have the dog ears been tie-wrapped? |  |
| Have all spool piece, jumper hoses and shore hose connection flanges on the cargo circuit (including on the offshore side) been shielded with acid spray shields when handling corrosive cargoes? |  |
| Has the SOPEP/SMPEP or VRP been implemented? |  |
| Are the manifold drip trays dry and connections to the spill tank ready to be used? |  |
| Are the manifold save-alls in position, dry and plugged? |  |
| Are all oil tank save-alls in other parts of the vessel clean, dry and plugged? |  |
| Are all deck scuppers plugged? |  |
| Have all safety & firefighting equipment been rigged & are ready for use? |  |
| Have air driven oil spill pumps been positioned aft, properly grounded, operationally tested and is a continuous supply of compressed air available on deck? |  |
| Verify the dump valve is closed/sealed tight and ready for use? |  |
| Have required oil spill cleaning up materials been stationed at the manifold? |  |
| Has the ship/shore checklist been completed? (ref ISGOTT – App A) |  |
| Are material safety data sheets available for each grade? |  |
| Have the butterworth heater valves been closed and blanked (blind off spectacle flange) |  |
| Has the surface of the ballast water been checked for traces of oil prior to ballasting operations? |  |
| Has the water level in Pressure / Vacuum Breaker been checked to be at the correct level? Are the gauge glass shut off valves in good working order and close automatically under spring action? |  |
| Have arrangements been made to monitor the offshore manifolds during starting of cargo operations and while increasing the loading / discharging pressures? |  |

| IF DANGEROUS/TOXIC CARGOES ARE TO BE LOADED/DISCHARGED, HAVE THE NECESSARY PRECAUTIONS BEEN TAKEN? | ## INITIALS |
| --- | --- |
| Are vessel’s superstructure openings closed? |  |
| Is the ship’s air-conditioning set on partial recirculation? |  |
| Has the dangerous/toxic gas concentration control been implemented? |  |
| Is the required antidote onboard and ready for use? Specify antidote ________________ |  |
| Is testing equipment available onboard to test for phenol cargo leaks in the cargo heating system / steam observation tank on deck? (If cargo to be carried is Phenol) |  |

| HAS THE PRE-OPERATION MEETING BEEN CARRIED OUT AND FOLLOWING BEEN UNDERTAKEN? | ## Initials |
| --- | --- |
| Have the planned operations in the port been discussed? (E.g. Loading, discharging, tank cleaning, COW, bunkering, stores / provisions, FW, inspections, crew change etc.) |  |
| Has the Cargo Operations Plan and Manifold arrangement been discussed in detail? |  |
| Has the action to be taken in case of emergency been discussed for various emergencies? (firefighting, spillage, inhalation, ingestion, skin contact and eye contact) |  |
| Has the appropriate Level of Protection (LoP) for the cargo to be carried onboard been identified and PPE requirement been explained in detail for the cargo to be carried? (Chemical Tankers are to refer to FOM 10.5 for identification of cargo specific LoP for various activities). |  |
| Have the antidotes for cargoes to be carried onboard including method of application been explained? (Chemical Tankers) |  |
| Has the safety and pollution prevention equipment to be available explained? |  |
| Has the risk assessment for cargo operations been reviewed and discussed? |  |
| Has the specific terminal information been discussed? (Mooring plan, which side alongside, ship / shore gangway, manifold connection size & No’s, vapour hose, security level etc.) |  |
| Have any special requirements for individual cargoes (i.e. nitrogen purge of tanks and lines, nitrogen blankets for intended cargo, inhibitors, heating etc.) been considered and complied with? |  |

| Signed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Officer |  | Master |  | Date |