THIS CHECKLIST IS TO BE COMPLETED BY THE OFFICER ON WATCH AT LEAST ONCE PER WATCH AND A DECK LOG BOOK ENTRY MADE

| HAVE THE FOLLOWING BEEN UNDERTAKEN: | ## Initials |
| --- | --- |
| The Master's and Chief Officer’s standing and special orders regarding vessel safety, security and cargo operations read and understood? |  |
| Is the vessel securely moored, aligned correctly with the berth and is the deck watch maintained properly? |  |
| Has the vessel draft been visually checked and remote readout compared against it? |  |
| ### Have tidal ranges, tidal or river currents and vessel traffic movements which may affect mooring safety been continuously monitored for unpredicted changes? |  |
| ### Have weather forecasts been obtained at least once each watch from relevant sources, including NAVTEX, the information recorded and relevant changes and trends passed between watch keepers? |  |
| ### Where required has HSSEQ 18 – “Permit to Allow Small Craft Alongside” been completed? |  |
| Are fire wires rigged and maintained above water as agreed with terminal? |  |
| Is the cargo loading and de-ballasting plan being adhered to? |  |
| Has the water in segregated ballast tanks been sampled/visually checked prior to discharge? |  |
| Is the vessel’s stress and stability within the acceptable limits? |  |
| Are cargo surveyors being accompanied by a ship’s officer? |  |
| Are all unused manifolds, bends and spool pieces blanked? |  |
| Is the radio and radar equipment isolated? (Including AIS if required by terminal)? |  |
| Are offside manifold valves pressure gauges installed and the gauge “test” valve open? |  |
| Are all cargo manifolds not in use being monitored for pressure build up? |  |
| Have cargo samples been taken? |  |
| Has cargo heating, if required, been started? |  |
| Are all sea valves closed, lashed and locked with suitable warnings posted? (lock out/tag out)? |  |
| Are all deck scuppers plugged and is the deck clear of any oil or cargo leakages? |  |
| Are all save alls clear, dry and plugged? |  |
| Are ullages checked, and recorded with loading rates being calculated every hour and compared with shore figures? |  |
| Have empty tanks been checked for leakages? |  |
| Is the safety equipment rigged and is protective equipment ready for use? |  |
| Has the inert gas upriser sump drain been checked and found clear and sump empty? |  |
| Is pumproom hydrocarbon and/or toxic vapour (if applicable) monitoring being recorded hourly)? |  |
| Are cargo tank high level, overfill and fixed gas monitoring systems operational? |  |
| Are all cargo lines and valves properly set and cross-checked by a further member of staff? |  |
| Is there a continuous supply of compressed air available on deck, pneumatic spill pumps tested? |  |
| Are the requirements and agreements made within the Ship Shore Safety Check List (SSSCL) being complied with, including communication protocols and repetitive checks? |  |
| The number of crew members on board and the presence of any other persons such as contractors performing work on board. |  |

| Signed: |  |  |  |
| --- | --- | --- | --- |
|  | Officer |  | Date |

A ship’s watch conducted visual monitoring of the water around and behind the vessel for visible sheens, dust, chemicals, abnormal discoloration or foaming, and other indicators of pollutants or constituents of concern originating from the vessel. Attention paid to deck runoff and ballast water. Ref VGP 2013 Routine Visual Inspections (4.1.1)