THIS CHECKLIST IS TO BE COMPLETED BY THE CHIEF OFFICER PRIOR TO DEPARTURE AND A DECK LOGBOOK ENTRY MADE

| HAVE THE FOLLOWING BEEN UNDERTAKEN: | ## Initials |
| --- | --- |
| Have all manifolds, cargo lines, flexible hoses and loading arms been adequately blown/ drained /purged as necessary & pressure released prior disconnecting? |  |
| Have all cargo and ballast system valves been closed and cross-checked by another member of crew? |  |
| Are cargo surveyors being accompanied by a ship’s officer? |  |
| Have all ullages, temperatures and samples been taken & sample retained onboard safely? |  |
| Are all manifolds, spool pieces and bends been blanked? |  |
| Are all save-alls clear of cargo? |  |
| Are all scupper clear of cargo? |  |
| Have cargo calculations been completed, cargo documents agreed and, if required, appropriate protests been issued? |  |
| Has the cargo tanks pre-washing operations, if applicable, been completed? |  |
| Have all tank openings been closed? |  |
| Is the vessel’s stress and stability within permitted limits for intended voyage? |  |
| Is the vessel upright, properly trimmed and ready for sea? |  |
| Have all shore connections including bonding cable been disconnected and are clear of the ship? |  |
| Has the sea chest valves visually confirmed to be closed and lashed? |  |
| Are all vapor locks closed? |  |
| Are all the elbow connected to VECS system removed or valve locked as appropriate? |  |
| Have all cargo hoses and deck gear securely lashed? |  |
| Has all safety, firefighting and oil spill equipment been stored securely away? |  |
| CHIEF OFFICER: |  |
| DATE: |  |