(Revised in line with ISGOTT 6)

| Tanker Name: |  |
| --- | --- |
| Port: |  |
| Berth / Terminal : |  |
| Product(s) to be transferred: |  |

Statements should be answered with the ‘Yes’ box checked.  If that is not possible, an appropriate comment should be written in the remark’s column. If a ‘Yes’ answer is not confirmed, it may be appropriate to delay or cancel the transfer operation. Where either party is not prepared to accept an assigned responsibility, a comment should be left in the remark’s column. It will then need to be decided whether transfer operations can proceed. Any information added to the remark’s column should be clear, any decision about safety of operations should be based on facts and not assumptions or interpretations.

Where a particular item is judged not to apply to the tanker, the terminal or to the planned operation, a note to this effect should be written in the remark’s column.  Also refer to instructions on page 14.

PRE – ARRIVAL CHECKS

| Date and Time completed : |  |
| --- | --- |

| PART 1A. TANKER: CHECKS PRE-ARRIVAL |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 1 | Pre-arrival information is exchanged (6.5, 21.2). | ☐ Yes |  |
| 2 | International shore fire connection is available (5.5, 19.4.3.1). | ☐ Yes |  |
| 3 | Transfer hoses are of suitable construction (18.2). | ☐ Yes |  |
| 4 | Terminal information booklet reviewed (15.2.2). | ☐ Yes |  |
| 5 | Pre-berthing information is exchanged (21.3, 22.3). | ☐ Yes |  |
| 6 | Pressure/vacuum valves and/or high velocity vents are operational (11.1.8). | ☐ Yes |  |
| 7 | Fixed and portable oxygen analysers are operational (2.4). | ☐ Yes |  |

| PART 1B. TANKER: CHECKS PRE-ARRIVAL IF USING AN INERT GAS SYSTEM |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 8 | Inert gas system pressure and oxygen recorders are operational (11.1.5.2, 11.1.11). | ☐ Yes |  |
| 9 | Inert gas system and associated equipment are operational (11.1.5.2, 11.1.11). | ☐ Yes |  |
| 10 | Cargo tank atmospheres’ oxygen content is less than 8% (11.1.3). | ☐ Yes |  |
| 11 | Cargo tank atmospheres are at positive pressure (11.1.3). | ☐ Yes |  |

| PART 2. TERMINAL: CHECKS PRE-ARRIVAL |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 12 | Pre-arrival information is exchanged (6.5, 21.2) | ☐ Yes |  |
| 13 | International shore fire connection is available (5.5, 19.4.3.1, 19.4.3.5) | ☐ Yes |  |
| 14 | Transfer equipment is of suitable construction (18.1, 18.2) | ☐ Yes |  |
| 15 | Terminal information booklet transmitted to tanker (15.2.2) | ☐ Yes |  |
| 16 | Pre-berthing information is exchanged (21.3, 22.3) | ☐ Yes |  |

CHECKS AFTER MOORING

| Date and Time completed : |  |
| --- | --- |

| PART 3. TANKER: CHECKS AFTER MOORING |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 17 | Fendering is effective (22.4.1) | ☐ Yes |  |
| 18 | Mooring arrangement is effective (22.2, 22.4.3) | ☐ Yes |  |
| 19 | Access to and from the tanker is safe (16.4) | ☐ Yes |  |
| 20 | Scuppers and save-alls are plugged (23.7.4, 23.7.5) | ☐ Yes |  |
| 21 | Cargo system sea connections and overboard discharges are secured (23.7.3) | ☐ Yes |  |
| 22 | Very high frequency and ultra-high frequency transceivers are set to low power mode (4.11.6, 4.13.2.2) | ☐ Yes |  |
| 23 | External openings in superstructures are controlled (23.1) | ☐ Yes |  |
| 24 | Pump room ventilation is effective (10.12.2) | ☐ Yes |  |
| 25 | Medium frequency/high frequency radio antennae are isolated (4.11.4, 4.13.2.1) | ☐ Yes |  |
| 26 | Accommodation spaces are at positive pressure (23.2) | ☐ Yes |  |
| 27 | Fire control plans are readily available (9.11.2.5) | ☐ Yes |  |

| PART 4. TERMINAL: CHECKS AFTER MOORING |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 28 | Fendering is effective (22.4.1) | ☐ Yes |  |
| 29 | Tanker is moored according to the terminal mooring plan (22.2, 22.4.3) | ☐ Yes |  |
| 30 | Access to and from the terminal is safe (16.4) | ☐ Yes |  |
| 31 | Spill containment and sumps are secure (18.4.2, 18.4.3, 23.7.4, 23.7.5) | ☐ Yes |  |

PRE – TRANSFER CONFERENCE

| Date and Time completed : |  |
| --- | --- |

| PART 5A. TANKER AND TERMINAL: PRE-TRANSFER CONFERENCE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker Status | Terminal Status | Remarks |
| 32 | Tanker is ready to move at agreed notice period (9.11, 21.7.1.1, 22.5.4) | ☐ Yes | ☐ Yes |  |
| 33 | Effective tanker and terminal communications are established (21.1.1, 21.1.2) | ☐ Yes | ☐ Yes |  |
| 34 | Transfer equipment is in safe condition (isolated, drained and de-pressurised) (18.4.1) | ☐ Yes | ☐ Yes |  |
| 35 | Operation supervision and watchkeeping is adequate (7.9, 23.11) | ☐ Yes | ☐ Yes |  |
| 36 | There are sufficient personnel to deal with an emergency (9.11.2.2, 23.11) | ☐ Yes | ☐ Yes |  |
| 37 | Smoking restrictions and designated smoking areas are established (4.10, 23.10) | ☐ Yes | ☐ Yes |  |
| 38 | Naked light restrictions are established (4.10.1) | ☐ Yes | ☐ Yes |  |
| 39 | Control of electrical and electronic devices is agreed (4.11, 4.12) | ☐ Yes | ☐ Yes | Including galley equipment |
| 40 | Means of emergency escape from both tanker and terminal are established (20.5) | ☐ Yes | ☐ Yes |  |
| 41 | Firefighting equipment is ready for use (5, 19.4, 23.8) | ☐ Yes | ☐ Yes |  |
| 42 | Oil spill clean-up material is available (20.4) | ☐ Yes | ☐ Yes |  |
| 43 | Manifolds are properly connected (23.6.1) | ☐ Yes | ☐ Yes |  |
| 44 | Sampling and gauging protocols are agreed (23.5.3.2, 23.7.7.5) | ☐ Yes | ☐ Yes |  |
| 45 | Procedures for cargo, bunkers and ballast handling operations are agreed (21.4, 21.5, 21.6) | ☐ Yes | ☐ Yes |  |
| 46 | Cargo transfer management controls are agreed (12.1) | ☐ Yes | ☐ Yes |  |
| 47 | Cargo tank cleaning requirements, including crude oil washing, are agreed (12.3, 12.5, 21.4.1) | ☐ Yes | ☐ Yes |  |
| 48 | Cargo tank gas freeing arrangements agreed (12.4) | ☐ Yes | ☐ Yes | See also part 7C |
| 49 | Cargo and bunker slop handling requirements agreed (12.1, 21.2, 21.4) | ☐ Yes | ☐ Yes | See also part 7C |
| 50 | Routine for regular checks on cargo transferred are agreed (23.7.2) | ☐ Yes | ☐ Yes |  |
| 51 | Emergency signals and shutdown procedures are agreed (12.1.6.3, 18.5, 21.1.2) | ☐ Yes | ☐ Yes |  |
| 52 | Safety data sheets are available (1.4.4, 20.1, 21.4) | ☐ Yes | ☐ Yes |  |
| 53 | Hazardous properties of the products to be transferred are discussed (1.2, 1.4) | ☐ Yes | ☐ Yes |  |
| 54 | Electrical insulation of the tanker/terminal interface is effective (12.9.5, 17.4, 18.2.14) | ☐ Yes | ☐ Yes |  |
| 55 | Tank venting system and closed operation procedures are agreed (11.3.3.1, 21.4, 21.5, 23.3.3) | ☐ Yes | ☐ Yes |  |
| 56 | Vapour return line operational parameters are agreed (11.5, 18.3, 23.7.7) | ☐ Yes | ☐ Yes |  |
| 57 | Measures to avoid back-filling are agreed (12.1.13.7) | ☐ Yes | ☐ Yes |  |
| 58 | Status of unused cargo and bunker connections is satisfactory (23.7.1, 23.7.6) | ☐ Yes | ☐ Yes |  |
| 59 | Portable very high frequency and ultra-high frequency radios are intrinsically safe (4.12.4, 21.1.1) | ☐ Yes | ☐ Yes |  |
| 60 | Procedures for receiving nitrogen from terminal to cargo tank are agreed (12.1.14.8) | ☐ Yes | ☐ Yes |  |

| PART 5B. TANKER AND TERMINAL: BULK LIQUID CHEMICALS. Additional for chemical tankers Checks pre-transfer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker Status | Terminal Status | Remarks |
| 61 | Inhibition certificate received (if required) from manufacturer | ☐ Yes | ☐ Yes |  |
| 62 | Appropriate personal protective equipment identified and available (4.8.1) | ☐ Yes | ☐ Yes |  |
| 63 | Countermeasures against personal contact with cargo are agreed (1.4) | ☐ Yes | ☐ Yes |  |
| 64 | Cargo handling rate and relationship with valve closure times and automatic shutdown systems is agreed (16.8, 21.4, 21.5, 21.6) | ☐ Yes | ☐ Yes |  |
| 65 | Cargo system gauge operation and alarm set points are confirmed (12.1.6.6.1) | ☐ Yes | ☐ Yes |  |
| 66 | Adequate portable vapour detection instruments are in use (2.4) | ☐ Yes | ☐ Yes |  |
| 67 | Information on firefighting media and procedures is exchanged (5, 19) | ☐ Yes | ☐ Yes |  |
| 68 | Transfer hoses confirmed suitable for the product being handled (18.2) | ☐ Yes | ☐ Yes |  |
| 69 | Confirm cargo handling is only by a permanent installed pipeline system | ☐ Yes | ☐ Yes |  |
| 70 | Procedures are in place to receive nitrogen from the terminal for inerting or purging (12.1.14.8) | ☐ Yes | ☐ Yes |  |

| PART 5C. TANKER AND TERMINAL: LIQUIFIED GAS. Additional for gas tankers Checks pre-transfer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker Status | Terminal Status | Remarks |
| 71 | Inhibition certificate received (if required) from manufacturer | ☐ Yes | ☐ Yes |  |
| 72 | Water spray system is operational (5.3.1, 19.4.3) | ☐ Yes | ☐ Yes |  |
| 73 | Appropriate personal protective equipment is identified and available (4.8.1) | ☐ Yes | ☐ Yes |  |
| 74 | Remote control valves are operational | ☐ Yes | ☐ Yes |  |
| 75 | Cargo pumps and compressors are operational | ☐ Yes | ☐ Yes |  |
| 76 | Maximum working pressures are agreed between tanker and terminal (21.4, 21.5, 21.6) | ☐ Yes | ☐ Yes |  |
| 77 | Reliquefaction or boil-off control equipment is operational | ☐ Yes | ☐ Yes |  |
| 78 | Gas detection equipment is appropriately set for the cargo (2.4) | ☐ Yes | ☐ Yes |  |
| 79 | Cargo system gauge operation and alarm set points are confirmed (12.1.6.6.1) | ☐ Yes | ☐ Yes |  |
| 80 | Emergency shutdown systems are tested and operational (18.5) | ☐ Yes | ☐ Yes |  |
| 81 | Cargo handling rate and relationship with valve closure times and automatic shutdown systems is agreed (16.8, 21.4, 21.5, 21.6) | ☐ Yes | ☐ Yes |  |
| 82 | Maximum/minimum temperatures/pressures of the cargo to be transferred are agreed (21.4, 21.5, 21.6) | ☐ Yes | ☐ Yes |  |
| 83 | Cargo tank relief valve settings are confirmed (12.11, 21.2, 21.4) | ☐ Yes | ☐ Yes |  |

| PART 6. TANKER AND TERMINAL: AGREEMENTS PRE-TRANSFER. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Part 5. Item | Agreement | Details | Tanker initials | Terminal initials |
| 32 | Tanker manoeuvring readiness | Notice period (maximum) for full readiness to manoeuvre: _____________ Period of disablement (if permitted): _________ |  |  |
| 33 | Security protocols | Security level: _________________ Local requirements: ______________________ _______________________________________ |  |  |
| 33 | Effective tanker/terminal communications | Primary system: _________________________ ___________________________________ Backup system: _________________________ ______________________________________ |  |  |
| 35 | Operational supervision and watchkeeping | Tanker: ________________________________ ___________________________________ Terminal: ______________________________ ______________________________________ |  |  |
| 37/38 | Dedicated smoking areas and naked lights restrictions | Tanker: ________________________________ ___________________________________ Terminal: ______________________________ ___________________________________ |  |  |
| 45 | Maximum wind, current and sea/swell criteria or other environmental factors | Stop cargo transfer: ______________________ Disconnect: _____________________________ Unberth: _______________________________ |  |  |
| 45/46 | Limits for cargo, bunkers and ballast handling | Maximum transfer rates: __________________ Topping-off rates: _______________________ Maximum manifold pressure: ______________ Cargo temperature: ______________________ Other limitations: ________________________ |  |  |
| 45/46 | Pressure surge control | Minimum number of cargo tanks open: ______ Tank switching protocols: _________________ Full load rate: __________________________ Topping-off rate: ________________________ Closing time of automatic valves: ___________ |  |  |
| 46 | Cargo transfer management procedures | Action notice periods: ____________________ Transfer stop protocols: __________________ |  |  |
| 50 | Routine for regular checks on cargo transferred are agreed | Routine transferred quantity checks: ________ |  |  |
| 51 | Emergency signals | Tanker: ________________________________ Terminal: ______________________________ |  |  |
| 55 | Tank venting system | Procedure: ____________________________ ______________________________________ |  |  |

| 55 | Closed operations | Requirements:__________________________ ______________________________________ |  |  |
| --- | --- | --- | --- | --- |
| 56 | Vapour return line | Operational parameters:__________________ ______________________________________ Maximum flow rate: _____________________ |  |  |
| 60 | Nitrogen supply from terminal | Procedures to receive: ___________________ ______________________________________ Maximum pressure: _____________________ Flow rate: _____________________________ |  |  |
| 83 | For gas tanker only: cargo tank relief valve settings | Tank 1: _______________ Tank 2: _______________ Tank 3: _______________ Tank 4: _______________ Tank 5: _______________ Tank 6: _______________ Tank 7: _______________ Tank 8: _______________ Tank 9: _______________ Tank 10: ______________ |  |  |
| XX | Exceptions and additions | Special issues that both parties should be aware of: _______________________________________ _______________________________________ |  |  |

PRE – TRANSFER CHECKS

| Date and Time completed : |  |
| --- | --- |

| PART 7A. GENERAL TANKER: CHECKS PRE-TRANSFER |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 84 | Portable drip trays are correctly positioned and empty (23.7.5) | ☐ Yes |  |
| 85 | Individual cargo tank inert gas supply valves are secured for cargo plan (12.1.13.4) | ☐ Yes |  |
| 86 | Inert gas system delivering inert gas with oxygen content not more than 5% (11.1.3) | ☐ Yes |  |
| 87 | Cargo tank high level alarms are operational (12.1.6.6.1) | ☐ Yes |  |
| 88 | All cargo, ballast and bunker tanks openings are secured (23.3) | ☐ Yes |  |

| PART 7B. TANKER: CHECKS PRE-TRANSFER IF CRUDE OIL WASHING IS PLANNED |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 89 | The completed pre-arrival crude oil washing checklist, as contained in the approved crude oil washing manual, is copied to terminal (12.5.2, 21.2.3) | ☐ Yes |  |
| 90 | Crude oil washing checklists for use before, during and after crude oil washing are in place ready to complete, as contained in the approved crude oil washing manual (12.5.2, 21.6) | ☐ Yes |  |

| PART 7C. TANKER: CHECKS PRIOR TO TANK CLEANING AND/OR GAS FREEING |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 91 | Permission for tank cleaning operations is confirmed (21.2.3, 21.4, 25.4.3) | ☐ Yes |  |
| 92 | Permission for gas freeing operations is confirmed (12.4.3) | ☐ Yes |  |
| 93 | Tank cleaning procedures are agreed (12.3.2, 21.4, 21.6) | ☐ Yes |  |
| 94 | If cargo tank entry is required, procedures for entry have been agreed with the terminal (10.5) | ☐ Yes |  |
| 95 | Slop reception facilities and requirements are confirmed (12.1, 21.2, 21.4) | ☐ Yes |  |

Other checks -

| Are the in-service oxygen and acetylene gas bottles valves closed. Duty engineer’s confirmation received. | ☐ Yes |  |
| --- | --- | --- |

Declaration

We the undersigned have checked the items in the applicable parts 1 to 7 as marked and signed below:

| Part | Checks | Tanker | Terminal |
| --- | --- | --- | --- |
| 1A | Tanker: checks pre-arrival | ☐ | ☐ |
| 1B | Tanker: checks pre-arrival if using an inert gas system | ☐ | ☐ |
| 2 | Terminal: checks pre-arrival | ☐ | ☐ |
| 3 | Tanker: checks after mooring | ☐ | ☐ |
| 4 | Terminal: checks after mooring | ☐ | ☐ |
| 5A | Tanker and terminal: pre-transfer conference | ☐ | ☐ |
| 5B | Tanker and terminal: bulk liquid chemicals. Checks pre-transfer | ☐ | ☐ |
| 5C | Tanker and terminal: liquefied gas. Checks pre-transfer | ☐ | ☐ |
| 6 | Tanker and terminal: agreements pre-transfer | ☐ | ☐ |
| 7A | General tanker: checks pre-transfer | ☐ | ☐ |
| 7B | Tanker: checks pre-transfer if crude oil washing is planned | ☐ | ☐ |
| 7C | Tanker: checks prior to tank cleaning and/or gas freeing | ☐ | ☐ |

In accordance with the guidance in chapter 25 of ISGOTT, we have satisfied ourselves that the entries we have made are correct to the best of our knowledge and that the tanker and terminal are in agreement to undertake the transfer operation.

We have also agreed to carry out the repetitive checks noted in parts 8 and 9 of this checklist, which should occur at intervals of not more than _________ hours for the tanker and not more than ________ hours for the terminal.

If, to our knowledge, the status of any item changes, we will immediately inform the other party.

| Tanker | Terminal |
| --- | --- |
| Name: | Name: |
| Rank: | Rank: |
| Signature: | Signature: |
| Date: | Date: |
| Time: | Time: |

| PART 8. TANKER: REPETITIVE CHECKS DURING AND AFTER TRANSFER (enter date for first check each day - add additional pages if required) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item Ref. | Check | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Remarks |
| Interval time: ………………. Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Inert gas system pressure and oxygen recording operational | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 9 | Inert gas system and all associated equipment are operational | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 11 | Cargo tank atmospheres are at positive pressure | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 18 | Mooring arrangement is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 19 | Access to and from the tanker is safe | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 20 | Scuppers and save-alls are plugged | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 23 | External openings in superstructures are controlled | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 24 | Pump room ventilation is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 28 | Fendering is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 29 | Tanker is ready to move at agreed notice period | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 33 | Communications are effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 35 | Supervision and watchkeeping is adequate | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |

| Item Ref. | Check | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interval time: ………………. Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 | Sufficient personnel are available to deal with an emergency | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 37 | Smoking restrictions and designated smoking areas are complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 38 | Naked light restrictions are complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 39 | Control of electrical devices and equipment in hazardous zones is complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 40,41 42,51 | Emergency response preparedness is satisfactory | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 54 | Electrical insulation of the tanker/terminal interface is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 55 | Tank venting system and closed operation procedures are as agreed | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 85 | Individual cargo tank inert gas valves settings are as agreed | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 86 | Inert gas delivery maintained at not more than 5% oxygen | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 87 | Cargo tank high level alarms are operational | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| Initials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PART 9. TERMINAL: REPETITIVE CHECKS DURING AND AFTER TRANSFER (enter date for first check each day - add additional pages if required) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item Ref. | Check | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Remarks |
| Interval time: ………………. Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Mooring arrangement is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 19 | Access to and from the terminal is safe | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 28 | Fendering is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 32 | Spill containment and sumps are secure | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 33 | Communications are effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 35 | Supervision and watchkeeping is adequate | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 36 | Sufficient personnel are available to deal with an emergency | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 37 | Smoking restrictions and designated smoking areas are complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 38 | Naked light restrictions are complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| Interval time: ………………. Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 | Control of electrical devices and equipment in hazardous zones is complied with | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 40,41 47,51 | Emergency response preparedness is satisfactory | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 54 | Electrical insulation of the tanker/terminal interface is effective | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| 55 | Tank venting system and closed operation procedures are as agreed | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes | ☐ Yes |  |
| Initials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Completion Instructions

Pre – Arrival

The tanker should complete part 1A (and part 1B if using an IG system) and then forward a copy to the terminal for review before arrival. The terminal should complete part 2 and then similarly forward a copy to the tanker for review before arrival.

On completion of the pre-arrival parts, if it is not possible to send a copy of the completed part to the tank and/or terminal, then a message should be sent confirming the time and date of completion to the relevant party before arrival. If there are any outstanding issues not marked ’Yes’ in the status box, this should be explained in the communication.

Checks after mooring

The tanker should complete part 4 and give a copy to the Terminal Representative as soon as possible, but no later than at the pre-transfer conference. The terminal should complete part 4 and give a copy to the tanker as soon as possible, but no later than at the pre- transfer conference.

Checks before transfer – the pre-transfer conference

Tanker and terminal personnel should both complete part 5A as part of the pre-transfer conference. Each party should retain a copy. This requires completion by ALL tankers.

If bulk chemicals are to be transferred, the tanker and terminal personnel should also complete the additional part 5B as part of the pre-transfer conference, and each party should retain a copy (for further information see ICS Tanker Safety Guide Chemicals)

If bulk gases are to be transferred, the tanker and terminal personnel should also complete the additional part 5C as part of the pre-transfer conference, and each party should retain a copy (for further information see ICS Tanker Safety Guide Liquefied Gas)

The tanker and terminal personnel should discuss and agree the content of part 6 (Agreements) which summarizes the detailed operational factors agreed at the pre- transfer conference. A reference copy for personnel on the tanker and in the terminal should be displayed at the relevant control stations. Tanker personnel should also complete the additional pre-transfer checks for all tankers in part 7A immediately before beginning transfer operations. If COW is planned, they should also complete part 7B.

All tankers planning on tank cleaning and/or gas freeing alongside should discuss the intention during the pre-transfer conference and once agreement is reached, provide a copy of part 7C to the terminal before beginning operations

The declaration

When completed, each separate checklist part should be checked off by tanker personnel, terminal personnel, or both, in the relevant boxes on the declaration form.

When all parts are addressed, tanker and terminal representative should agree the intervals at which they will undertake repetitive checks of items applicable to their responsibility from the SSSCL, and that could impact on the safety of the operation if not monitored. This interval should be noted in the declaration, after which the two representatives may agree to start operations and add their details.  The tanker and terminal should retain a copy of all checklist parts and declarations for their files in accordance with the operators document retention period.

Summary of repetitive checks during and after transfer

The tanker personnel should complete the repetitive checks in part 8 at the agreed intervals. The record should be available for terminal personnel to review

The terminal personnel should complete the repetitive checks in part 9 at the agreed intervals. The record should be available for tanker personnel to review

Tanker and terminal personnel should provide a final copy of their parts 8 and 9 to the other when operations are completed. This will provide a basis for review of the operation and verification of checks undertaken.