Loading and discharging rates

| Maximum Loading Rate (Each Line) |  | Cubic Meter/ hour |
| --- | --- | --- |
| Homogenous Loading Rate (Each Line) |  | Cubic Meter/ hour |
| Maximum Discharge Rate (Each line) |  | Cubic Meter/ hour |
| Maximum simultaneous Discharge Rate |  | Cubic Meter/ hour |

Loading rates for static & non static accumulator cargoes

Ref.Max.Loading Rate = Cross sectional area of pipe (m2) X Instant flow rate (1m/s or 7m/s or 12 m/s) X 3600 seconds.

| Diameter and identification of Cargo line | (I)* Initial Loading Rate @1 m/s | (A) Max bulk Loading Rate (Non Inerted with Static Accumulators) | (B) Max bulk Loading Rate (Inerted with Static Accumulators) | (C) Max bulk Loading Rate (Non Inerted with Non Static Accumulators) |
| --- | --- | --- | --- | --- |

*Min rate to be maintained till the time the tank inlet is well covered and all splashing and surface turbulence in the tank has ceased.

- Refer ISGOTT 7.3.3.2 and 11.1.7.3 for more information.

Example: For a 6 inches pipeline (150 mm), cross sectional area will be: 3.14 X 0.075 X 0.075 = 0.0177 m2 (I)* Loading rate @ 1m/sec = 0.0177 (m2) X 1 (m/sec) X 3600 (Sec) = 63.72 m3/hr (Max initial flow rate for loading static accumulator cargoes in a non Inerted atmosphere NOT TO EXCEED 1 m/s) (A) Loading rate @ 7m/sec = 0.0177 (m2) X 7 (m/sec) X 3600 (Sec) = 446 m3/hr (Max Bulk Loading for Static Accumulator into Non Inerted Tanks, NOT to exceed 7 m/sec) (B),(C) Loading rate @ 12m/sec = 0.0177 (m2) X 12 (m/sec) X 3600 (Sec) = 764 m3/hr (Max Bulk Loading for Static Accumulator into Inerted Tanks & Non Static Accumulator Cargoes, NOT to exceed 12 m/sec)