1	APPLICATION

Fully Managed Vessels

2	PURPOSE

To ensure that hazardous substances are dealt with in a safe way and to make a suitable and sufficient assessment of the risks posed by any hazardous substance in use, to the health of employees and others on board.

3	RESPONSIBILITY

Marine Superintendent

The Shipboard Management Team Crew

# INTRODUCTION

This procedure provides guidance on:

  - The use and storage of chemical substances hazardous (chemical substances) to health;

  - The various types of chemical substances found on board and risks associated with them;

  - The use of SDS and PPE while working/ using such hazardous substances.

# GENERAL

When handling materials that might be harmful to health, employees must be alert at all times to prevent accidents and injuries.

Chemical substances shall only be used if the respective Material Safety Data Sheet is available on board and the user is familiar with its content.

Records of all chemical substances shall be available on board and the respective Material Safety Data Sheets shall be in English.

# ASSESSMENTS

When chemical substances are received on board the vessel, the chief engineer or safety officer shall verify if the substances are accompanied with the correct Safety Data Sheet (SDS). This sheet has to be provided by the manufacturer / downloaded from manufacturer official website. In case the sheet is missing, respective Vessel Manager should be contacted who will inform Company purchasing department. Should the SDS for the chemical substance not be available on board, the product shall be segregated and put into quarantine until the availability of SDS.

The SDS shall be kept in the storage area where the chemical substances are being stored together with a fully and routinely updated Chemical & Hazardous Substances Register, including standard Risk Assessment applicable to those substances. A second complete set of SDS shall be filed on the bridge together with all the various fully and routinely updated Chemical and Hazardous Substances Registers, including Risk Assessment, if applicable, on board.

CE or Safety Officer to ensure the chemical substances are stored in accordance with to the SDS guideline.

If the chemical substance has the potential to cause harm if inhaled, ingested, come into contact with or is absorbed through the eyes or skin, a COSHH Risk Assessment must be carried out prior usage.

Risk Assessment

The person who is using a chemical substance with the potential to cause harm must carry out a COSHH Risk Assessment based on the contents of the SDS in conjunction with the knowledge of the work to be carried out and working environment.

This Risk Assessment will be recorded on the Risk Assessment (TOM form 089 /Bassnet) The person carrying out the Risk Assessment has to ensure that risk levels are reduced to low or allowable risk. The completed COSHH Risk Assessment must be filed accordingly.

Implement Controls

Once the Risk Assessment process has been completed, it is essential that all identified controls are implemented before use of the chemical substance for the first time.

On all subsequent usage of the chemical substance, a check should be carried out to ensure the controls are still in place.

If any of the controls cannot be successfully implemented, then the substance must not be used. It is vital that all personnel who may use the chemical substance are provided with adequate information, training and supervision to allow them to carry out the work in a safe manner.

# USING HAZARDOUS SUBSTANCES

Prior using any hazardous substances the SDS and COSHH Assessment must be fully understood. The area must be properly ventilated. Check if the necessary fire extinguishers and spill containment kit are available close to the worksite. Check that the appropriate first aid materials are available.

Usage

The proper PPE as stated on the SDS sheet shall be worn during the handling of the hazardous substances. Make sure the PPE is in good condition. Do not eat, drink or smoke while using chemicals. Upon finishing using the chemicals, always clean your hands and the PPE used.

Use of Chemical Substances

All instructions could be found at both the packaging and SDS, with regard to the handling of the substance. Both the risk and safety information should be studied carefully and under no circumstances shall the products be taken into use if the necessary PPE or treatment facilities and equipment is not available.

Spill

Before tackling spills, make sure correct PPE is worn. Clean the spill straight away and dispose the substance in appropriate method. Always report any spill as per your SOPEP / SMPEP Manual.

# RADIOACTIVE SUBSTANCES

Radioactive sources are seldom carried on board Company managed vessels except by specific request of a Client. Should the vessel be required to carry radioactive substances, the following procedure shall apply during operations involving the storage, handling, use and transportation of radioactive substances.

Handling, Storage and Transportation Radioactive Substances

The contractor representative supplying the service involving the use of radioactive sources shall;

  - provide a safety handbook/ instruction guide conforming to the National/International Regulations in force for the area or operation;

  - provide an emergency procedure handbook; detailing what measures shall be taken if a radiation hazard exists;

  - provide written advice for the Master with respect to personnel evacuation, bearing in mind that radiation is emitted three dimensional and can permeate decks;

  - be qualified to act as the Radiation Protection Advisor and shall make himself known to the Master upon arrival of the vessel;

  - be responsible for ensuring that all safety precautions are observed and that no personnel other than Qualified Persons are exposed to radiation levels in excess of the maximum permitted limit as defined in the government regulations in force;

  - prior engaging in any work involving the use of radioactive source, obtain permission via the Permit to Work System. He shall inform the Master of the scope of work and when he would wish the work to start;

  - monitor all operations involving the use of radioactive substances to ensure the safety requirements are being adhered to;

  - be responsible for supervising the movement of radioactive sources from the transport container to the permanent storage area;

  - maintain records of all sources held on the vessel and copies of relevant transport documentation;

  - record the radiation levels in storage areas daily (or as instructed in safety handbook/ instruction guide provided by contractor).

Storage and Handling

Radioactive substances must be stored in suitable protective containers, clearly marked and should be stored as far away from the accommodation as practicable and in designated IMDG cargo area.

No radioactive substances shall be handled/ stored onboard unless all required precautions and protection have been made and equipment provided (eg. Radioactive protective clothing, radiation measuring devices). When radioactive substances are stored on the vessel, the relevant location should be shown on the hazardous area plan.

# STANDARD SYMBOLS INDICATING THE HAZARD OF THE SUBSTANCES

Below is the symbols and indication of danger which was adopted by several countries including EU, USA, Australia and UK.

The classification of dangerous substances is based largely on the characteristic properties of the substances and their effects on man or the environment as follows:

Carcinogenic

A number of substances cause cancer. They may be in solid or liquid form and of a chemical or physical nature. For instance, Beta-napthylamine is known to cause cancer of the bladder, and blue asbestos, and indeed other forms of asbestos, can cause lung cancer and mesothelioma. It should be appreciated that such cancers are only one aspect of the much wider problem of toxicity. Cancer is fundamentally a disease process which may affect tissues in the body and is a disorder o cell growth. The new growth may remain simple and localised. i.e. it is being or it may invade and destroy neighbouring tissues and distant body organs, i.e. it is malignant.

Corrosive

A substance which may, on contact with  living tissues, destroy them.  Corrosive substances  are those which,  by chemical action, can cause severe damage when in contact with living tissues or, in the case of leakage, will substantially damage or destroy other materials. Thus in relatively mild cases of contact with corrosives, dermatitis may occur, where in more severe cases the result may be chemical burns. Strong acids, including anhydrides and acid chlorides, strong bases, including hydroxide plus alkaline earth materials, and their oxides, halogens and oxidizing agents are classic examples of corrosive materials. Some of these possess additional hazards, e.g. corrosion of heir containers, or violent reaction with water and other substances.

Cryogenic

Cryogenic substances are those which, because of their low temperature, present a special danger. They are generally liquefied gases at temperatures below 0 degree.

Common examples are liquid nitrogen (bp – 196 Degree Celsius), liquid oxygen (bp – 183 Degree Celsius) and liquid helium (bp – 269 Degree Celsius). The principal hazards associated with cryogenic substances are:

  - Chemical – violent reactions can occur when certain cryogenic substances react with other chemical substances, such as liquid oxygen with oil or charcoal, or liquid hydrogen with oxygen;

  - Physical – the normal properties of structural and other materials change at low temperatures and may are rendered brittle, e.g. rubber, plastic, metals;

  - Physiological – conditions such as frostbite and burns the skin can result from contact with cryogenic materials. Furthermore, since such substances vaporize rapidly at room temperature, there is always the danger of anoxia (oxygen deficiency).

Explosive

An explosive is a substance that may explode under the effect of flame, shock or friction. Broadly, an explosion can be considered as a rapid exothermic reaction (or decomposition) in which gases are produced.

This causes a sudden  vast increase in pressure within a confined space. Conditions which  can ultimately result in materials exploding include:

  - Mechanical shock sudden movement, dropping or knocking of the containers;

  - Friction stirring or transferring material;

  - Temperature accumulation of heat can cause detonation;

  - Violent chemical reactions between reagents. Substances capable of undergoing explosions include:

  - Highly flammable material if ignited in confined spaces, e.g. solvent vapours, gases;

  - Authorised explosives;

  - Other thermodynamically unstable materials; classes of such materials include acetylides, acetylene compounds, polynitro compounds, azides, diazo compounds, fulminates, certain nitrates, peroxy compounds, vinyl monomers, epoxides. In addition, explosion hazards can result from the explosive oxidation of reducing agents by strong oxidizing agents such as persulphuric acid, concentrated nitric acid and nitrates of sodium and silver.

Extremely Flammable

Such substances are classified as liquids having a flash point of less than 0 Degree Celsius and boiling point of less than or equal to 35 Degree Celsius, and when such a liquid forms more than 45 per cent by weight or more than 250g of the contents of an aerosol dispenser, the contents of that dispenser.

Flammable

A flammable substance:

  - Is a liquid having a flash point equal to or greater than 210 C and less than or equal to 55 C, and which when tested in the manner specified in the Second Schedule to the Highly Flammable Liquids and Liquefied Petroleum Gases Regulations 1972 does support combustion;

  - Forms the contents of an aerosol dispenser when they include more than 45 per cent by weight or 250g of a liquid which has a flash point equal to or greater than 210 C and less than or equal to 1000 C.

Harmful

A substance which, if it is inhaled  or ingested, or if it penetrates the skin, may involve   limited health risks.

Highly Flammable

This is a substance which:

- May become hot and finally catch fire on contact with air at ambient temperature without any application of energy;

- May readily catch fire after brief contact with source of ignition and which continues to burn or to be consumed after removal of the source of ignition, e.g. methanol;

- Is gaseous and flammable in air at normal pressure, e.g. butane;

- In contact with water or damp air evolves highly flammable gases in dangerous quantities,

e.g. sodium produces hydrogen gas;

- Is a liquid having a flash point below 210 Degree Celsius, and when such a liquid forms more than 45 percent or more than 250 gram of the contents of an aerosol dispenser, the content of that dispenser is considered flammable.

Irritant

An irritant is a non-corrosive substance that, through immediate, prolonged or repeated contact with the skin or mucous membrane, can cause dermatitis or a localized inflammation.

Mutagenic

A number of substances are capable of causing damage to the individual cell in such a way that subsequent cell division may become uncontrolled. In normal body cells, this may give rise to cancer.

Narcotic

Some chemical substances, e.g. alcohol, affect the brain and central nervous system to cause a depressant effect on their functions.

Oxidizing

An oxidizing agent is a substance which gives rise to a highly exothermic reaction when in contact with other substances, particularly flammable substances. Strong oxidizing agents include those substances which, though not spontaneously flammable themselves, are sources of oxygen. They present a fire hazard, in that they can;

- Support combustion in an otherwise oxygen deficient atmosphere; and

- React so vigorously with reducing agents – e.g. organic solvents, paper, and cotton – as to bring about ignition.

Examples of this hazard category include concentrated per choric acid, per sulphuric acid and organic proxy compounds, many of which also represent an explosion hazard, together with concentrated nitric acid, potassium chromate, dichromate and permanganate.

Radioactive

This means that the substance emits ionizing radiations. Many such substances are isotopes of chemical elements such as cobalt-60 or strontium-90, and emissions may take the form of alpha particles, beta rays, gamma rays or neutrons.

Toxic

A substance, if inhaled/ingested, or if it penetrates the skin, may involve serious/acute or chronic health risks and even death.

Very Toxic

Very toxic means a substance which, if haled or ingested, or if it penetrates the skin, may involve extremely serious, acute or chronic health risks and even death.

# TRAINING

To ensure personnel are able to handle chemical/ hazardous substances in safe manner, training is essential and necessary. When determining the level of training required for existing and new employees, the following should be considered;

  - The hazard associated with the hazardous substance to be used;

  - The complexity of the work processes;

  - Any controls, work practices and PPE which may be required to minimise the risks.

Existing and new crew / personnel need to be able to recognise and interpret information on any hazardous substances in the workplace; therefore, training programs shall incorporate the following:

  - Labelling of containers of hazardous substances, an explanation of the information that each part of the label provides and why the information is being provided;

  - How to access SDS and the information that each part of the SDS provides;

  - How to perform Basic and Full COSHH Assessment;

  - Information about hazardous substances to which crew / personnel are or may be exposed in the course of their work;

  - Work practices and procedures dealing with handling, use, storage, cleaning up and disposal;

  - Measures used to control exposure:

    - Proper use of PPE;

    - Procedures to be followed in case of an emergency involving hazardous substances;

    - Incident reporting procedures.