## APPLICATION

This document applies to all shipboard personnel.

## PURPOSE

The purpose of this document is to specify requirements for safe bunker management and safe bunkering operations.

## RESPONSIBILITIES

  - Chief Engineer

    - Being on board and in charge whenever bunkers are received.

    - In charge of bunkering operations, including the:

      - Loading

      - Storage

      - Consumption of all bunkers

    - Developing a plan to:

      - Transfer

      - Receive

      - Consume from fuel tanks

    - Witnessing:

      - Sounding on the bunker barge / road tanker BEFORE and AFTER delivery.

      - Gauging of shore tanks.

    - Calculating quantities delivered to the vessel.

    - Recording the flowmeter of the bunker barge / road tanker or shore tanks before and after delivery.

    - Obtaining a representative bunker sample at the vessel’s manifold using the continuous

drip method.

    - Performing onboard tests of fuel received to check density, water & compatibility, wherever applicable

    - Establishing un-pumpable quantities

    - Conducting bunker surveys

    - Ensuring bunker quality specifications stated in BDN meet:

      - The Company specifications

      - Charter Party requirements

    - Bunker operations checklist and other records are completed as required by this procedure.

## Master

    - Complying with all:

      - International, National and Local Regulations,

      - Rules, codes including industry guidelines where applicable to this procedure. This includes ECAs.

    - Ensuring that sufficient bunkers, with adequate safety margin are onboard.

    - Ship to ship transfer (Bunkering operations) checklist E22A is completed prior mooring and unmooring with the bunker tanker.

## Chief Officer

    - Calculations of trim, stress and stability

    - Assisting the Chief Engineer in ensuring compliance with all safety, environmental and security requirements as described within this procedure.

## ENGINEERING REQUIREMENTS

  - Bunker manifold

Chief Engineer shall:

    - Assign ship’s engineer to supervise the connection of joint of the flange between the supplier's hose and the ship’s manifold.

    - Inspect the hoses and gaskets. Reject any hose and/or gasket that is suspect or appears to be defective.

    - Confirm that bunker flanges on both port and starboard side have bolts in all holes.

## Bunker tanks

    - Stencil area near the sounding pipes of each bunker tanks with 90% & 95% filling level soundings.

    - Stencil the 0.1% S fuel bunker tank filling valves “0.1% S Tk”.

    - Post temporary placards near the sounding pipes prior start of every bunker operation to indicate the final sounding.

## BUNKER REQUIREMENTS

  - Bunker stems

    - Master shall calculate bunker stems in close co-operation with the Chief Engineer for the coming voyage.

    - Always state the fuel or diesel oil specifications as per company minimum requirements.

    - Stem the bunkers as per:

      - The voyage instructions from the commercial operators of the vessel. (I.e. one-way or round-trip bunkers, etc.).

      - Bunker restrictions, e.g. vessel arriving at shipyard, going to emission controlled areas/EU requirements/CARB requirements etc.

## Bunkers quality specification

Order

    - Fuel as per specification ISO 8217:2017 (latest standard) or ISO 8217:2015as applicable.

    - Maximum Sulphur content of the fuel should be specified. Refer to FOM 4.19

‘Emission Control Areas (ECA) – Sulphur regulations’.

    - Bunker requisition terms (specifications to the operators arranging the bunker supply for the vessel) should specifically state that the fuel must meet with requirements of Clause 5 of ISO 8217:2017, which offers guidance on ‘General requirements’ of marine fuel to “be free from any material at a concentration that causes the fuel to be unacceptable for use”, together with other considerations. Accordingly, the fuel should exclude any anomalous components.

    - Information regarding density and temperature which have been used to calculate bunker quantity ordered must be provided to Charterer/owner so that they know the volume to be supplied in case the density of arranged bunker is different from the one used for calculation.

    - Charterer and owners must be requested for density and temperature at which bunker is intended to be supplied

    - Designated grades of fuels, in accordance with recommendations from engine builders (equipment makers) should be ordered.

Note:

The Chief Engineer shall not accept other grade or substitute unless approved by Head of Business Unit.

Test results for the parameters TAN (Total Acid Number) and RSN (Reserve Stability Number) should be obtained from the bunker supplier, if available. Problems can be expected with fuel having TAN value above 0.4 and RSN value greater than 9.

# Fuel sample analyses:

Representative drip samples collected during bunkering should be tested by a competent shore laboratory for following:

    - All parameters under latest ISO 8217 standard.

    - All parameters in accordance with the standard for which the fuel is ordered.

    - For residual grade fuels, additional tests should be carried out to identify presence of any contaminants and concerns with instability – example, basic GSMS Screening, Reserve Stability, Xylene Equivalence, Total Acid Number etc. The tests can be decided in consultation with the contracted bunker analyses laboratory.

    - Distillate Fuels should be tested for Cold Filter Plugging Point and Cloud Point (also a part of ISO 8217:2017 standards).

    - Extended analyses in consultation with the laboratory should be carried out, in case unwarranted contaminants (example, chlorinated hydrocarbons, styrenes, alkenes, indenes, phenols, acidic contaminants etc.) are detected in the fuel.

    - Testing of hybrid fuels such as ULSFO / VLSFO as per the applicable specifications.

# Actions in case the fuels are detected with abnormalities

| Nature of off-spec condition | Measures to be taken in case of off-spec fuel | Measures to be taken in case it is inevitable to use the off-spec fuel |
| --- | --- | --- |
| ISO 8217 parameters outside the allowable range of the standard | - Do not use the fuel - Inform Vessel Manager - Seek advice from fuel laboratory - Thoroughly assess the risk of using the fuel - Intimate owners and commercial operators and advise on the identified risks - Put supplier on notice and push supplier to accept the fuel back, and to provide compliant bunkers of acceptable quality - In case the usage of fuel detected to be off-spec is inevitable, approval should be obtained from the Head of Business Unit. | - Exercise control measures after seeking advice from fuel laboratory |
| Sulphur content outside the specification |  | - Advise Flag authorities & PSC - Raise FONAR |
| Contaminants that indicate non- compliance to Clause 5 of ISO 8217 standard |  | - Exercise control measures after seeking advice from fuel laboratory - Based on laboratory advice, carry out additional tests to determine the concentration of the contaminant, and the presence of any other contaminants |

Using bunkers without laboratory analysis reports:

    - The company does not recommend using bunkers without lab analysis report. However, in case it is inadvertent due to non-availability of bunkers with lab analysis report, consult the VM prior using untested fuel and seek permission.

    - Following precautions are recommended in case untested fuel has to be used by vessel:

Table-A Recommended Fuel Cleaning temperature in the separator

| Fuel type |  | Fuel temperature in separator |
| --- | --- | --- |
| Distillate |  | 40~500C |
| ULSFO | Viscosity @ 50cst |  |
|  | Up to 20 cSt | 500C |
|  | 20~40cSt | 600C |
|  | 40~50cSt | 700C |
|  | 50~80cSt | 800C |
|  | >80cSt | 980C |
| HFO |  | 980C or higher |

      - All Engineer Officers are apprised of the problems that can be encountered due to inferior quality fuel and a risk assessment should be carried out.

      - Chief Engineer must ensure that mixing with existing fuel is kept to minimum possible and only kept limited to Service Tank.

      - Keep the fuel in storage/settling tanks for maximum possible time to allow for separation by gravity.

      - Storage and Settling and Service Tank temperatures should be maintained sufficiently to accelerate the abrasives/water separation by settling efficiently.

      - Maintain a stable temperature at the settling tank, this will ensure temperature to inlet of purifier will not fluctuate.

      - Drain the settling and service little longer after first transfer from storage tanks to settling tank. Subsequently settling tank should be drained off for water/sediments

at regular intervals, observing any change in the degree of water/sediment being drained.

      - Two purifiers to be operated continuously at minimum feed rate and at a temperature as specified in table A for effective purification. Desludging interval should be set at 30 minutes.

      - Gravity disc of correct size should be used, as suitable for the fuel density (depending on the type of purifier).

      - Ensure that purifiers are operating normally, and have no deficiencies. In case any abnormality is observed (excessive sludge etc.), vessel to report to office immediately.

      - Filters (in Fuel Oil Transfer pump, FO Purifier and ME FO Systems) should be inspected and cleaned regularly; and confirm not damaged. Filter condition must be advise to office basis observations on checking the filters after starting usage of this fuel.

      - Filtration plant and auto backwash filters should be closely monitored, particularly observing the auto backflush frequency. (a record must be maintained)

      - Maintain Fuel Oil viscosity at Engine inlet at 12 ~14 cSt.

      - Closely monitoring Main Engine and Auxiliary Engine parameters, take engine performance and inform vessel manager in case any abnormality is observed.

      - As far as possible use only one Auxiliary Engine till the new fuel is ascertained to be of acceptable quality through test reports or based on satisfactory performance over a significant period

    - Take following actions upon bunkering awaiting the analysis reports:

      - Consult the VM prior to the vessel sailing out

      - Allow for a minimum of 5 working days for fuel analysis report to be received

    - Take following actions when newly bunkered fuel specifications has failed:

      - Inform the fuel analysis lab. to seek any interim measures which may allow the safe use of bunkers such as:

        - Increase in separation and temperature

        - Purification with minimum feed rate

        - Draining of settling and service tanks

        - Increasing cylinder oil feed rate

        - Inform all parties regarding the failure of vessel’s bunkered fuel in meeting the specifications; and the deviation.

        - Proceed at safe speed while maintaining normal operating parameters of engines to the nearest port of call if bunker is considered unsafe.

## Bunker compatibility and stability

Chief Engineer shall closely evaluate performance of the plant on the bunkered fuel. Any abnormalities, such as those listed below, should be carefully noticed, and reported to Vessel Manager:

    - A drop in fuel burning efficiency or reduction in engine horsepower by careful analysis of

the vessel’s engine abstracts

    - Irregular exhaust gas temperatures, under otherwise "normal" circumstances

    - Frequent blockages of the fuel injectors

    - Blocking of the fuel filters

    - Overloading of the fuel purifiers such that they require frequent overhauling and cleaning

    - Hidden signs, such as:

      - Reduced compression pressure and peak pressure

      - Dirty scavenge spaces and under piston areas

      - Broken piston rings

# Guidelines: Refer FOM 4.11 for Company Comingling policy

## Using Bunkers with High catfines (Below limits specified in FOM 4.13)

Chief engineer must ensure following while consuming bunkers with high catfines levels which are below limits specified in FOM 4.13 but in higher ranges:

    - Fuel should be efficiently settled and decanted - Water drained at Settling and Service Tanks every watch, with careful observation of the contents emerging from the drains. Effective water draining is instrumental in catfine separation.

    - Settling and Service Tank temperatures should be maintained sufficiently to facilitate efficient water removal.

    - Two purifiers must run in parallel.

    - Fuel should be fed to purifiers at minimum feed rate and at a temperature as specified in table A.

    - Desludging interval of fuel oil purifiers must not be more than 60 minutes.

    - Purifiers are run continuously, even when there is no fuel consumption separators should be kept running (Settling Tank to Service Tank, with oil from Service Tank overflowing to Settling Tank). As far as possible try to circulate the fuel through purifiers multiple times to bring the catfine content down to acceptable levels.

    - Filters (in Fuel Oil Transfer, FO Purifier and ME FO Systems) should be inspected and cleaned regularly; and confirm they are not damaged.

    - Ensure purification efficiency analysis been carried out recently. If not, vessel to land samples before and after purifier for analyses to ascertain that purifiers are operating effectively.

## Problems with new bunker use

There may be chances that vessel encounters problems with purification, filtration or usage of bunker, in spite of fuel specifications tested to be conforming. The problems are most likely to surface immediately or within short span of time after starting the consumption.

Following problems may be encountered by vessel in relation to bunker:

- Fuel Filter Clogging

- Purifier generating excess sludge

- Problem with engine

  - Excessive wear of running surfaces (liners, piston rings, piston ring grooves etc.).

  - High Exhaust temperatures

  - Excessive fouling in combustion spaces

  - Piston Ring breakage

  - Fuel Pump seizure

  - Fuel Pump / Nozzle wear

In such a case, vessel must immediately inform Vessel Manager and must take bunker samples from fuel purification / supply system for further additional analysis.

One or more of following additional tests may be required based on problem encountered.

| S.No. | Machinery problem | Test to identify problem |
| --- | --- | --- |
| 1 | Filter Choking | Toulene Insoluble |
| 2 | Purifier/Filter Choking due to unstable fuel | Stability |
| 3 | Contaminants in Fuel causing Filter Choking/fuel pump seizure | GC-MC Screening |
| 4 | Piston Ring Breakage & Purifier/Filter Choking | Asphaltene Analysis |
| 5 | Very high wear in rubbing surfaces due to catfines | Patented Particle Count Test |
| 6 | Fuel Pump/Nozzle Wear | Acidity-pH |

## Purifier efficiency check – Every 6 months

Vessel must take samples of the fuel oil before and after each HFO purifier, and land these for shore analysis by the contracted Lab / service provider in order to evaluate the effectiveness of purifiers in separating out water, cat fines and other impurities.

Following precautions should be exercised:

    - Samples should be drawn at standard collection points every time, and as close as possible to the purifier.

    - Sample collection should be done 15 mins after the de-sludge operation to ensure that the purifier operation has settled down.

    - In order to make the assessment of the analysis results to be as meaningful as possible, the samples should be drawn within a short period of time and when the fuel from the last bunkering (which is intended to be tested) has displaced the fuel oil from the previous delivery.

    - Ensure the lines are thoroughly flushed before drawing the sample.

    - The samples may be filled directly into the sample bottles. Where samples are collected at points with elevated temperature, it is recommended to use a closed container.

    - The bottle cap should be firmly fastened after the sample has cooled to avoid leakage.

    - It should be endeavoured that the samples are collected (and landed for sampling) when fuels with higher cat fine and water content are bunkered and being treated. This will enable more effective evaluation of purification efficiency.

## Fuel additives

Additives can be employed during 3 stages:

- Storage and pre-treatment

- Combustion of fuel

- Passage of the exhaust

Under normal circumstances, if a well-blended fuel is stored properly at the correct temperature, there should be no need for an additive. If the fuel has any of the following characteristics, the use of additives may be required:

  - If the fuel is unstable

  - If it has excess sludge

  - If it contains excess Vanadium

  - If it suffers from Bacterial contamination

  - If it contains dissolved Hydrogen sulphide gas

  - If it contains excess water in an emulsified condition or

  - If it has high paraffinic content and a high pour point.

Different additives listed in the table below can mitigate the problems listed above.

| No. | Purpose | Why needed | Additive | How it works | Effectiveness |
| --- | --- | --- | --- | --- | --- |
| 1 | Sludge Dispersant | Clogs purifier, forms deposits in storage | Surface active agents | Separates nucleus of sludge (metal band) from Asphaltenes | Reduces sludge; helps combustion; |
| 2 | De- emulsifiers | Oil & water mix intimately this is difficult to separate out. If the water has salts, more problem potential | Surface additive agents | Lowers surface tension of the liquid & facilitates separation | Works in many cases. Fails in some cases |
| 3 | Combustion Improvers | Reduced ignition & combustion properties | Metallic & oil-soluble compounds | Increases surface area of fuel droplets | Not reliable-difficult to prove |
| 4 | High temp. Corrosion inhibitors | Ash from high V sticks to combustion surface & causes corrosion at that spot | Oxides of Mg or ca | Raises the temperature so the deposits don’t form & stick to surface | Good effectiveness |
| 5 | Pour Point Depressants | To prevent fuel from solidifying, freezing in tanks, prevent clogging & improving fluidity |  |  | Good |
| 6 | Anti-Bacterial | Bacterial growth; clogs filter | Biocides | Kills bacteria | Yes, but biocides used must be water & oil soluble |
| 7 | Dissolved Hydrogen Sulphide | Highly toxic gas | Amines | Absorbs H2S | Good |

## BUNKER QUANTITIES

  - Bunker quantity - Fuel oil & lubricating oils

    - The Master and Chief Engineer shall calculate the bunker requirements for the coming voyage including any additional quantities taking into account:

      - Fuel amount currently on-board

      - Fuel required to perform the next voyage

      - Safety margin

      - Other consumptions:

        - Tank cleaning

        - Ballast changing

        - Heating the cargo

        - Manoeuvring (in / out of port)

        - Inerting

        - Cargo operations

        - Deviations expected due to weather and / or traffic conditions

    - The Master shall inform commercial operators in copy to operations department and technical department.

    - Contact VM for consultation if there is doubt in any respect, such as, geographical location of the next bunkering port, or the longest leg of the voyage, or safety margin/minimum bunker reserve etc.

## Minimum bunker reserves

    - Minimum reserves for fuel oil should be calculated for operations:

      - at full power

      - for all passages

      - excluding un-pumpable quantities

    - Use following table guiding on additional bunker reserve:

| Sea passage duration (days) | Additional bunker reserves |
| --- | --- |
| 1 to 5 | 50% |
| 6 to 10 | 40% |
| 11 to 20 | 30% |
| over 20 | 20% |

    - Apply a safety margin of at least 30% where:

- The vessel will pass through a seasonal winter zone in winter.

- Adverse weather may be anticipated at certain times of the year.

    - Minimum reserves of diesel oil (distillate fuel):

      - Diesel oil min. quantity = ((voyage days+5 days reserve) x (diesel gen. cons/day))

+ consumption for manoeuvring (in/out of port) + 5% of total D.O. capacity as an un-pumpable quantity.

      - Minimum Bunker Reserve for LSFO or LSMDO/LSMGO

Note:

Offshore vessels do not need to comply with this requirement.

Vessel trading in ECA area to follow above guidelines for minimum bunker reserve for low sulphur fuel oil.

    - Report to the technical department immediately if the quantity of bunkers remaining onboard varies significantly from that last reported or expected.

    - Marine gas oil required for emergency diesel generator:

      - Should be stemmed through the technical dept. through the requisition process.

      - Minimum quantity to be maintained onboard is 1.5 times the capacity of the fuel storage tank for the emergency diesel generator.

## Minimum lubrication oil reserves

| Lubricating Oils Minimum ROB Requirement |  |
| --- | --- |
| Lubricating Oil Grade | Minimum ROB |
| Main Engine Cylinder Oil | Greater of the following quantities should be maintained on board: - Consumption over the duration of voyage + 10 days OR - Consumption over a period of one month |

| Lubricating Oils Minimum ROB Requirement |  |
| --- | --- |
| Lubricating Oil Grade | Minimum ROB |
| All other lubricating oils: - Main Engine System Oil - Aux Engine Crank Case Oil - Steering Gear Hydraulic Oil - Valve Remote Control System Hydraulic Oil - Framo System Hydraulic Oil - Main Air Compressor - Refrigeration / Air- conditioning compressors - Any other loose oil grades | For each grade, quantity maintained on board should be sufficient for following: - Full charge (capacity of the equipment and system) for the application of maximum capacity (out of all applications that use the oil grade). Example - If framo system and steering gear are using the same oil grade, and framo system has greater oil capacity, then the quantity to be retained will be determined by framo system. PLUS - 3 months’ oil consumption on the system |

## BUNKERING PROCEDURES

## Bunkering plan

    - A bunkering plan shall be:

      - Completed prior to commencement of bunkering operations.

      - Reviewed by all personnel assigned to assist in the bunkering operation.

      - Posted in a conspicuous position and a copy retained in a designated file together with the Declaration of Inspection (U.S.A. ports) if applicable.

    - The bunker plan shall include:

      - Bunker tanks to be loaded:

        - Identification

        - Location

        - Capacity

        - Density & temperature at which the calculation has been made for bunker quantity.

      - The grade of fuel oil to be bunkered and % of sulphur content.

      - The ullage or sounding of all the tanks (including drain and overflow tanks):

        - Prior to bunkering

        - Upon completion of bunkering

      - Sequence in which tanks are to be filled

      - Pipeline plan showing valve line-up

      - Watch arrangements for monitoring tank levels

      - Loading rates

      - Valve operation arrangements

      - Specific instructions for monitoring the ullage or sounding of the non-bunkering tank levels at the:

        - Start

        - During change-over

        - Prior to the completion of bunkering

      - The arrangements required for the relief of personnel for meals, going to rest room etc.

      - Arrangement of all sampling equipment to collect a representative sample at vessel’s

manifold.

    - A risk assessment (RA) must be carried out to identify all the hazards associated with bunkering.

## Safety and pollution prevention

In charge: Chief Engineer Assistant: Chief Officer

Prior commencing bunker operations, the Chief Engineer shall implement:

    - The safety and anti-pollution requirements contained in E22A Bunkering safety checklist.

      - FOM 4.21 Bunkering HSE guidelines

    - Throughout the bunkering operations:

      - Oil spill equipment and salvage pump readily available on deck

      - Scuppers plugged

      - Bunker tank lids closed

      - Portable fire extinguishers deployed at bunker manifold

      - Hoses connected to fire main

      - All ‘save-alls’ empty and plugged prior to commencing bunker operations

      - Safe access to vessel provided

      - No smoking and visitor notices displayed

      - Bending Moments (BM) and Shear Force (SF) checked by Chief Officer. on loading computer

      - ‘B’ Flag, red light or other signal displayed as required by local regulations

      - Vessel securely moored, if applicable

      - Check communications

        - Inspect all equipment to be used, including hoses, pipelines, fittings and air vents on tanks

        - Test high level and overfill alarms, where fitted to:

          - Fuel, diesel and other secondary tanks part of fuel system designated to receive bunkers

          - Tanks forming part of contingency (such as in the event of overflow during transfer)

## Simultaneous cargo and bunkering operations

–

Note:

If vessel has to undertake bunkering along with cargo operation, office approval must be taken prior operation and no other (non-cargo) operations should be conducted simultaneously during such critical cargo operations.

A comprehensive RA is to be carried out for bunkering and cargo operation simultaneously, addressing the below key points:

| Supervision | Each operation is adequately supervised (requires separate supervisors) |
| --- | --- |
| Distraction | Common control location (CCR, ECR etc.). Multi-tasking of personnel and/or resources (e.g. topping of bunker tanks and stripping/topping off cargo tanks simultaneously). |
| Resources | Adequate number of personnel for each operation. Personnel involved in bunkering and cargo operation are not assigned any other tasks. |
| Communication | Sufficient number of walkie talkies & VHF sets & allocation of separate communication channels for each operation. |
| Knowledge | Personnel allocated are experienced in the concerned operation |
| Fatigue | Rest hours are complied with. |

    - A meeting must be carried out by the senior management on board to review the upcoming simultaneous operation, discuss the RA, company procedures, and contingency measures.

    - Carry out toolbox meeting prior execution of simultaneous operation with the following to be discussed:

      - Prevailing and forecasted weather condition

      - Shore limitations if any

      - Review of number and understanding of the allocated duties of the personnel involved

      - Availability of personal protective equipment / spill response equipment

      - Availability of communication tools (Walkie talkies sets, VHF sets)

      - Experience sharing – If an operation was carried out under similar conditions

## Bunker plan review

    - The Chief Engineer, together with all personnel (including deck rowing watch keepers and bridge duty officer) assigned to assist in the bunkering operation, shall carry out a pre-bunker meeting and carry out review of the procedures. Minutes of meeting must be recorded in ER Log Book.

    - The review shall include but not limited to the following items:

      - Bunkering Plan

      - Oil spill prevention, including consequence and penalties in event of infringement

Note:

Refer to Appendix 1; 33CFR 155.750 Contents of oil transfer procedures.

Oil transfer Procedures including individual watch keepers’ responsibilities

    - Communications :

      - On-board

      - Between supplier and receiver

    - Duties of Engineer at the point of transfer and deck watch

    - Emergency shut down procedures

    - Record the review process in engine room logbook and underlined in red.

      - In addition, the Master shall make a deck logbook entry noting that the person in charge of the bunkering operation conducted the review.

## Pre-bunker conference

    - The Chief Engineer shall arrange a pre-bunker conference between the ‘Persons in-charge of bunkering’ of both the Receiving ship and bunker facility before commencing bunkering operations.

    - The pre-transfer conference shall cover but not limited to the following agenda:

      - Name of delivery vessel or terminal / Estimated date and time of bunker transfer

      - Identification of product to be transferred

      - Designate the fuel tanks to be loaded.

      - Sequence of transfer

      - Connection of loading arms or hoses

      - Watch arrangements including the names of the personnel involved with emphasis

of the necessity for ‘Teamwork’ in order to avoid ‘One Man Errors’

      - Communications including predetermined frequencies for radio communications during transfer

      - Transfer plan posted in accordance with vessel transfer procedures

      - Estimated date and time of completion

      - Confirm there is enough space for the nominated loading volume.

      - Establish the maximum loading volume for each of the designated tanks.

      - Check that bunker system valves are correctly lined up (two-man independent check required)

      - Determine the rates for the start of loading, bulk loading and topping-off.

      - Set out the tank atmosphere control arrangements.

      - Establish the internal tank transfer arrangements.

      - Verify the operation and accuracy of the gauging system.

      - Confirm the alarm settings on overfill alarm units.

      - Agree with the bunker supplier, before starting, to establish and record:

        - The loading procedure.

        - How and where to carry out quantity checks.

        - How and where to carry out quality checks, including installing a sampler if used.

      - Establish safe access to and from the ship.

      - Decide how to monitor the temperature of the bunkers during loading.

      - Set out the communication procedure for the operation, including the emergency stop signal and the steps for emergency disconnection and unberthing.

      - Decide the personnel requirements to carry out the operation safely.

      - Monitor the bunkering operation and that it conforms to the agreed procedure.

      - Verify the procedure for changing bunker tanks during loading.

      - Confirm the spill response and containment arrangements.

      - Define weather operating limitations.

    - In a USA port the pre-transfer conference shall cover each item on the Declaration of Inspection, which is contained in Appendix 2; 33CFR 156.150 Declaration of Inspection (U.S.A. Ports)

    - Details of the pre-transfer conference shall be entered in both the deck and engine room logbooks noting that the person in charge of the bunkering operation conducted it

    - Bunker density and temperature must be confirmed with supplier and ensure that ships tanks can accommodate the actual volume basis density and temperature at which bunker is supplied.

## Bunker operations watch-keepers and training requirements

Note:

Do not assign any other tasks to crewmembers designated as watch-keepers for the bunkering operation

Do not include the duty UMS engineer in the bunkering team.

Do not handover duties or leave designated stations unless suitably relieved by other personnel & convey same to personnel involved in the operation.

Do not carry out relief during commencing, completion, topping off or tank change over operations

Chief Engineer shall:

    - Decide the number of personnel assigned to each bunkering operation depending upon the circumstances of each case

    - Plan Bunkering to allow for the availability of sufficient rested personnel

    - Delay bunkering operations until sufficient resources can be dedicated to the operation

    - Inform Operations Department and VM immediately if such circumstances may cause delay to the ship

# Training

Chief Engineer shall:

    - Train all ship’s personnel involved in bunkering

    - Record Date, time and names of attending personnel

    - Perform the training on bunkering to include any new signed on personnel at least 48 hours before bunkering.

    - Include the following in the training session, as a minimum:

      - Company’s environmental policy with specific reference to prevention of pollution.

      - The ship’s bunker tanks, piping, valves and pumps. The bunker piping diagram is

also to be consulted.

      - The specific duties of each person.

      - An explanation of methods of communication to be used between ship and supplier.

      - A demonstration of the hand signals as described in FOM 4.12A Bunkering HSE guidelines.

      - Immediately.

      - Gas checks on open deck areas should be made downwind of vents to ensure any hazardous gas is sensed. Adequate PPE to be used as necessary.

## Communications

The Chief Engineer shall ensure that:

    - Communications between the bunkering barge / road tanker or shore facility and the ship are accomplished visually and by voice, two-way radios or other effective means.

    - Personnel standing the watch during bunkering must be able to communicate amongst each other and with the Chief Engineer and the Master, usually via two-way intrinsically safe radios.

    - Proper English phrases and hand signals are used by all persons involved in bunkering to communicate with the bunkering supplier.

## Watch keeping duties

Chief Engineer shall ensure:

    - An Engineer is on duty throughout the bunkering operation to carry out his orders in:

      - Conducting checks

      - Connecting hoses

      - Measuring quantities etc

      - Stationed on deck for monitoring operations

      - Implementing the bunker pre-loading plan

    - An additional responsible person is to be on watch to monitor gauges and perform other watch duties as required.

    - One person from the Engineering staff to stand watch at the transfer point (i.e., the bunker manifold).

    - An additional deck watch to monitor:

      - The anti-pollution measures in place

      - Oil spills on deck

Note:

The deck watch will visually inspect water near or opposite all bunker tanks, sounding pipes and tank vents and in particular at commencement of operations, during tank changeover or ‘topping off’ operations.

Over the ship’s side

## Work hours

    - Crewmembers involved in bunkering operations shall not work excessive hours except in the case of emergency.

    - –Emergency means an unforeseen situation that poses an imminent threat to:

      - Human safety

      - Environment

      - Loss of property

    - Crewmembers shall ensure that hours worked in bunkering operations are recorded in their record of hours worked log.

## Filling tanks

Before topping off tanks or changing tanks:

    - Inform the delivering vessel or terminal

    - Order to adjust the flow rate and stops or ‘shut downs’ as required.

Caution:

It is the Company policy that subject to local requirements, NO fuel oil, diesel oil or lubricating oil tank shall be filled to greater than 90% of its capacity.

Inform the deck watch so that spill-monitoring procedures are observed.

## Deviation from filling policy

    - If requested by owners/charterers & subject to approval from the Vessel Manager, filling restriction can be increased to 95 % from 90 %.

    - Vessels shall follow the following guidelines to bunker in the range of 90% - 95% of tank capacity:

      - Submit details of the fuel oil / diesel oil tanks from tank calibration tables clearly indicating:

        - Soundings/ullage in meters

        - Corresponding volume in cubic meters for range from 90% to full capacity

      - Advise if any limitations are mentioned in tank calibration table by approving authority.

Note:

Higher chances for vapour locking in DB Tanks, and faster rate of ullage reduction in smaller tanks, can introduce greater risks of overflowing, which needs to be well evaluated and aptly addressed in risk assessment.

Conduct risk assessment for filling up the Bunker tanks to the range of 95 % to ascertain the Hazards & the Risk control measures.

    - Send risk assessment to the Vessel Manager/Marine- HSSEQ Superintendent for review and approval.

Note:

Air locks in 95% filling can be detrimental to tank structure.

Ensure tank venting system is checked and verified for operation.

    - Ensure that level gauges and high level alarms for all tanks in fuel system are tested for satisfactory operations.

    - Check manual soundings of receiving tank at interval not exceeding 5 minutes and compare with remote level gauges while topping off to 95% from 90 %.

    - –Check any trim and/or list with C/O

    - Keep overflow tanks empty as a contingency wherever fitted.

    - Final topping off rate for HFO bunker tanks & MDO / LSDO bunker tanks should be established.

      - Recommended rate 1/5th (20%) of the maximum loading rate,

      - Consider the construction of the tank and the location of the sounding / air pipes to determine maximum loading rate

      - Agree topping off rates with bunker supplier during pre-bunker transfer meeting and record in bunker safety checklist.

      - Follow above mentioned topping off rates regardless of remaining quantity to be received.

    - Carry out final topping off under direct supervision of the Chief Engineer, with a responsible person monitoring the loaded tanks soundings.

      - Seek additional guidance on case to case basis from Vessel Manager/Marine HSSEQ Supt. as office response to risk assessment.

## During bunkering

# Monitoring

Throughout the whole operation:

    - Continuously monitor the rate of bunkers being loaded and onboard quantities.

    - The engineers assigned to this task shall not release or hand over duty to their assigned relieves before:

      - Transmitting all the required handover information and understanding verified.

      - Informing the Chief Engineer.

    - Enforce two man sounding system for ships that don’t have high level alarms in their bunker tanks.

    - The Deck Watch shall pay attention to the:

      - Bunker barge moorings

      - Pilot ladders rigged (no Jacobs ladder) with adequate lighting

    - Keep all main deck scuppers plugged.

    - Check tightness of scupper plugs by pulling up the scupper plugs by hand.

# Ullages

    - Do not exceed 30 minutes for tank soundings/ullaging during bunkering operation.

    - Maintain record of the tank sounding/ullaging from both manual sounding and remote gauges.

    - Regularly compare sounding tape readings and remote gauging system readings.

    - Report any unexpected ullage or any uncertainty regarding ullages to the Chief Engineer immediately.

    - Gas checks on open deck areas should be made downwind of vents to ensure any hazardous gas is sensed. Adequate PPE to be used as necessary.

–

Note:

Sounding pipes located in the engine room, steering flat and other enclosed spaces are usually fitted with self-closing devices.

Self-closing sounding pipes

    - Never lock or tie self-closing sounding cocks in the open position.

# Temperatures, density and flow meter

    - Measure temperature and density during the bunkering operation.

Note:

If hydrometer is not available on board, the Chief Engineer must requisition hydrometers for measurement of densities in the range of densities - 0.8 to 1.1.

Compare the results with figures given by the supplier.

    - Master must issue a letter of protest if any difference between supplier’s and ship’s figures

is noticed.

    - When a flow meter is used, the Chief Engineer shall:

      - Verify the meter readings before commencement and on completion of bunkering.

      - Compare the figures against ship’s figures.

    - When loading bunkers into ship’s tanks which are in service, Fuel oil temperature:

      - Shall not exceed 140°F (60°C)

      - Or a maximum temperature of 10°C below flash point

Caution:

U.S.C.G. and SOLAS regulations including class rules do not permit a flash point of less than 60°C.

Or fall below a temperature at which viscosity increases above the easy pumping limit

# Refer to Appendix 3 – 46CFR 35.25 – 10 Requirements for fuel oil

## Topping off and completion

    - When the bunkering nears completion, responsible engineer shall:

      - Verify tank ullages continuously

      - Comply with the agreed stop procedure

      - Slow down the rate of transfer as agreed with the supplier when the tanks reach 85% of their capacity

      - Close all valves used after consulting with the Chief Engineer

## Shut downs

Note:

A standard shut-down is to be considered to be identical to an “emergency shut-down” in terms of the process involved and is to be carried out with an appropriate degree of urgency

Stop oil transfer if the risk cannot be managed.

## –Shut down:

    - Means that:

      - The liquid flow is stopped

      - Valves are progressively shut to normal pre-operation isolation status

      - Flange and hoses disconnected and blanked, if necessary

    - Will be initiated:

      - If there is any change in the situation which alters the status of any item on the bunkering safety checklist resulting in an unacceptably elevated risk situation.

      - If there is any change in the parameters used in compiling the bunkering plan, or which could lead to change in expected outcomes, or exceed the calculated values used in the bunkering plan.

Caution:

Do not re-commence the bunkering operation after a SHUT DOWN until the stated SHUT DOWN criteria have been cleared and the Chief Engineer has given permission to resume the bunkering operation.

In case bunkering operations have to be stopped mid-way for some reasons, in cold weather conditions, the line should be cleared by blowing with air and / or draining at relevant locations (depending on requirement). In any case at the end of the bunkering operations, it should be ensured that the line is cleared. (For vessels taking bunkers with Mass flow meter, readings must be taken before & after blowing through with air).

If there is any doubt or reason to consider that a spill has or is likely to occur.

## Quantities loaded

    - The Responsible Engineer will record the tank ullages and temperatures

    - The Chief Engineer will calculate the quantity received on board and compare with information given by the supplier.

    - The Chief Engineer or responsible engineer nominated by the Chief Engineer shall

Caution:

This must be done even if an independent bunker surveyor has been appointed.

measure both the supplier’s and ship’s tanks before and after the bunkering operation.

    - The Master and/or the Chief Engineer shall sign bunker receipts for quantities received at average temperature.

    - Never sign any paper or document presented by the supplier and which purports or implies indemnity to a bunker supplier or his representatives.

    - If the supplier’s quantity exceeds the ship’s quantity by 0.5% or if the bunkers contain

an excessive amount of water (maximum 1% by volume):

      - Present a letter of protest to the supplier’s representative

      - Vessel Manager must be notified immediately

## Post-operation review and debriefing

    - Carry out a post-operation review and debriefing of the bunkering team to identify:

      - Any inappropriate acts that may have occurred

      - Any near misses, hazardous occurrences or any non-conforming incidents

      - –Any areas of potential improvement to the operation

    - The Master and Chief Engineer shall sign off on this review to indicate that:

      - The operation is complete

      - The requirements of this procedure have been complied with

Note:

This section is meant for guidance to ship staff receiving bunkers from tankers equipped with approved mass flow metering systems.

OPERATION WITH MASS FLOW METER

## Prior bunkering

    - Bunker Tanker shall provide copy of following documents for verification and signing by Chief Engineer and Bunker Surveyor (if engaged):

      - Bunker requisition form (mass flow metering)

      - Mass flow metering system seals checklist

      - Meter reading record form (delivery)

      - To check prior bunkering that approved piping diagram is available on bunker barge

/ road tanker, and that there are no discrepancies between the actual piping and the approved piping diagram.

      - Bunker operation must not commence if there is an error displayed on the bunker metering computer.

    - Chief Engineer shall confirm:

      - Quantity

      - Grade of bunkers

      - Sequence of bunkering

      - Bunkering rate within the calibrated flow range for the mass flow meter (as stated on the stamping plate of the meter)

      - Meter calibration certificate

      - The Metering system diagram and all sealing points intact

Note:

Meter should be set to zero, wherever resettable totalizer meters are fitted.

Opening meter reading

    - Report the matter to the implementing authority if any seal is missing or broken or seal number not matching

Caution:

The meter shall not be used for the custody measurement of bunker transfers until the missing or broken seal is replaced and approved for use by the implementing authority.

Note:

During bunkering

–

Bunker tank gauging is still important for verification, close control of bunker tank filling levels and timely action to prevent oil spill, and as a contingency/back up in case MFM System breaks down.

Maintain flow rate within the calibrated flow range of the mass flow meter.

    - Record all stoppages with details.

    - Conduct tank stripping and line cleaning only the end of the bunkering operations.

    - Carry out line clearing by following steps:

      - Close the discharge valve after the pump

      - Build up the pressure in the pipeline using pump

      - Open the discharge valve for the remaining bunker in the bunker hose to be cleared

into the vessel’s tank.

      - Actions in case of MFM failure:

        - Pumping should be stopped immediately, meter totalizer reading recorded, and stoppage / failure reported to MPA (if Bunkering in Singapore).

        - Tank Gauging, in accordance with SS 600 shall be used to determine the remaining quantity delivered and a separate BDN shall be issued.

Caution:

No air compressors or air bottles shall be used by the bunker tanker for the line clearing process.

The line clearing process shall not be repeated more than twice after the completion of the pumping operation.

Note:

After pumping operation is completed, Cargo Officer of the bunker vessel shall notify the Chief Engineer and Bunker Surveyor (if engaged) prior to the commencement of line clearing operation.

## After bunkering

    - Bunker Tanker shall provide copy of following documents for verification and signing by Chief Engineer and Bunker Surveyor (if engaged):

      - Mass flow metering system seals checklist

      - Meter Reading Record Form (Delivery)

      - Bunker metering ticket shall be printed at the end of delivery and filed for reference

      - Bunker Delivery Note (BDN)

    - The Chief engineer shall verify:

      - All sealing points intact (not tampered) in the course of bunkering

      - Closing meter reading and quantity of the bunker delivered

      - Bunker metering ticket contains information on:

        - Name of bunker tanker

        - Craft license number (SB no.)

        - Meter Unique Identification Number

        - Bunker start date & time

        - –Bunker end date & time

        - Print time

        - Mass in Air

      - The BDN:

        - Comply with the IMO regulation 18(3) of Annex VI MARPOL 73/78

        - All relevant and applicable columns filled in

        - “NA” (Not Applicable) inserted in blank columns

        - Any cancellation or amendment on the BDN endorsed and stamped by the Cargo Officer and the Chief Engineer

        - No negotiations should be done – BDN figures should just be based on the calculations obtained from bunkering meter record form.

## Quantity dispute

In the event of any dispute at the end of the delivery with respect to the quantity of bunkers delivered, Chief Engineer and the Bunker Surveyor (if engaged) should act as follows:

    - Re-witness the meter totalizer readings.

    - Re-check all sealing points based on the sealing plan and confirm all seals listed in the seal verification report are intact.

    - Confirm that all relevant lines have not been modified from that as stated in the piping diagram.

    - Obtain and examine photocopies of the relevant pages of the bunker tanker’s Meter

Totalizer Log showing all the totalizer movements related to the bunker delivery.

    - Examine and obtain copies of certificates / documents listed below:

      - Meter calibration certificate

      - Metering system diagram and sealing points

      - Meter seal verification report

      - Meter zero verification report

      - Cargo system piping diagram

      - Mass flow metering procedure for bunkering endorsed by implementing authority

      - Mass flow metering system approval letter from implementing authority for custody transfer

      - Copy of enhanced bunker cargo officer course certificates accompanied with photo identity of the Cargo Officer

      - Last certificate of quality or equivalent document by the cargo provider (terminal or ship)

    - If any of the preceding steps are disallowed, record the reasons in a note of protest by the Chief Engineer and statement of facts by the Bunker Surveyor (if engaged).

    - The Bunker Surveyor (if engaged) shall record all the relevant details and findings of the dispute in a statement of fact. This document should be completed and acknowledged by the Chief Engineer and/or Cargo Officer of bunker vessel.

    - –If the dispute remains unresolved, the Chief Engineer shall raise a note of protest and get it signed by the Cargo Officer of the bunker vessel.

Note:

During bunkering, be alert and observe any signs of the barge / road tanker pumping in air which will result in froth in the oil.

SPECIAL ATTENTION REQUIRED/CAPPUCCINO EFFECT

- Tell-tale signs to be noted:-

  - Bunker hose jerking or whipping around

  - Gurgling sound when standing in vicinity of bunker manifold

  - Fluctuations of pressure indication on manifold pressure gauge

  - Fluctuations of level indication gauge of bunker tank (if fitted)

- For confirmation of froth:

  - Open the bunker tank manhole

  - Lower a 20 litre bucket to take sample from the top

Note:

In a deep tank the froth level can be up to 1 meter.

Typically the cappuccino effect may take up to 48 hours to settle. For a high viscosity grade at lower temperatures, the effect will take longer to clear.

Visually confirm the difference in liquid and froth by pouring into a transparent glass bottle

- Action if frothing is suspected in the bunkers received in ship’s tank:

  - Stop bunker

  - Call Vessel Manager and charterer/owner

  - Allow about 2 ~ 3 hours, between initial and 2nd ullaging whenever there is a doubt of air entrapment

  - Pour a bucket of MDO into the sounding pipe before taking sounding

  - Request for and make a photocopy of the barge’s / road tanker sounding table relevant page - Once at start/once at stop

  - Do not sign the BDN unless a clear understanding and confirmation of the received quantity is reached

  - In case of severe and brazen case of threatening the CE to force his signature on BDN, inform the agent and Vessel Manager

Caution:

Under NO CIRCUMSTANCES should there be any extra or “UNOFFICIAL ROB”. These figures will support our case at the time of a bunker dispute resolution – otherwise the quantities measured by a 3rd party surveyor will not match the ship’s log book figures and undermine our case.

Issue a letter of protest and immediately send to the Vessel Manager & to the charterer/owner who has nominated the bunker supply

## –RECORDS

- The Chief Engineer shall note all information regarding bunkering operations in the engine room log book (chronology of events and quantity of bunkers loaded).

- OOW shall record above information plus movements of the supply barge / road tanker in the cargo or Port Log Book.

- Record bunkering operations as per MARPOL instructions in the oil record book part I.

- Other documents, all relevant documents are to be filed:

  - Bunker delivery notes

  - Bunker sample record or log book

  - Bunker operations checklist

  - Bunkering plan

  - Record of training

  - Minutes of pre – transfer conference

  - Shore analysis documentation

  - Letters of protest

  - Sounding books

  - Request to witness sampling

  - Minutes of post – operation review

## BDN (Bunker Delivery Note)

As required under MARPOL Annex VI, following information to be included in the bunker delivery note (BDN), w.e.f. 01st January 2019.

    - Name and IMO Number of receiving ship

    - Port

    - Date of commencement of delivery

    - Name, address and telephone number of marine fuel oil supplier

    - Product name(s)

    - Quantity in metric tonnes

    - Density at 15°C (kg/m3)

    - Sulphur content (% m/m)

BDN should include a declaration signed and certified by the fuel oil supplier’s representative that the fuel oil supplied is in conformity with regulation 18.3 of Annex VI and that the sulphur content of the fuel oil supplied does not exceed:

    - the limit value given by regulation 14.1 of Annex VI; or

    - the limit value given by regulation 14.4 of Annex VI; or

    - the purchaser’s specified limit value of  (% m/m), as completed by the fuel oil  supplier’s representative and on the basis of purchaser’s notification that the fuel oil is intended to be used:

      - in combination with an equivalent means of compliance in accordance with regulation 4 of Annex VI; or

      - –is subject to a relevant exemption for a ship to conduct trials for sulphur oxides emission reduction and control technology research in accordance with regulation

3.2 of Annex VI.

Note:

Sample Seal Numbers on BDN: Seal numbers entered in BDN should be for the fuel samples collected at vessel’s bunker manifold through drip sampling (i.e., representative samples), witnessed and signed by the barge representatives.

This declaration shall be completed by the fuel oil supplier’s representative by marking the applicable box (es) with a cross (x).”

## SAMPLE COLLECTION AND PROCESSING

## Prearrangements for sampling

    - Use fuel analysis vendor nominated for your vessel for bunker sample analysis.

    - Use sampling Kit supplied by nominated fuel analysis vendor

    - Raise requisition well before the sampling bottles are out of stock,

    - Refer to FOM 4.20 Bunker sampling guidelines for comprehensive sampling requirements.

    - Collect the custody transfer sample at the manifold of the vessel.

Note:

The sample collected shall constitute the representative sample of the bunker operation.

Starting the sampling

The Chief Engineer shall:

    - Ensure that the sampling container is clean and with transparent wall so that continuous drip is visible.

    - Ensure that the line sampler is ready for use with a new collection container.

    - Complete a ‘request to witness sampling’ form, and give top copy to the supplier or

supplier’s representative.

    - Retain copy for the ship’s file.

    - –If the supplier or supplier’s representative declines to attend the witnessing of sampling,

record this fact in the ship’s logbook.

    - Take sample by adjusting the line sampler flow valve to give a slow continuous drip throughout the bunkering period.

    - Use the security seal facility to prevent tampering.

    - Ensure that approximately 5 liters is collected from each barge / road tanker or over the bunkering period if from one source.

## Once the sample has been collected

Chief Engineer shall:

    - Seal the container and shake vigorously to mix the sample thoroughly.

Note:

Security seals must have a unique identification number.

Fill 5 sample bottles to indicator line by making several passes.

    - Cap and seal the sample bottles recording unique security seal numbers in the Chief

Engineers report form and in the ship’s logbook.

    - Complete sample bottle labels and sign together with the supplier or supplier’s

representative.

Caution:

If offered a sample by the barge / road tanker crew, and you have not witnessed correct sampling procedures, ensure that it is stamped ‘For Receipt Only – Source Unknown’. In addition, sample labels presented before bunkering operations commencement, must not be signed.

Fix label to bottle.

## Distribution of samples:

    - The Chief Engineer shall distribute these samples as follows:

      - 1 set to be sent to nominated laboratory for analysis

      - 1 set to be kept on board for a period of at least 12 months

      - 1 set to be kept on board as MARPOL 73/78 Annex VI fuel oil sample for a period of at least 12 months

      - 1 set for fuel supplier

      - 1 set for surveyor (when applicable)

    - If the supplier or supplier’s representative discards the sample this fact shall be recorded

in the ships’ logbook.

    - Inform Vessel Manager each time samples have been taken, mentioning how many samples have been sent for analysis.

Note:

The aforementioned action is of the utmost importance in that the terms and conditions under which bunker fuels are supplied normally contain strict time limits for the notification of claims.

Inform Vessel Manager immediately where fuel delivered is found to be unsuitable for use.

## –SOUNDING OF FUEL OIL TANKS

- Keep factual records .Never report over or under quantities of the bunkers.

- Survey/take soundings manually on daily basis (weather permitting) to ascertain the quantity of fuel oil, regardless of whether or not vessels are equipped with fuel oil flow meters.

- Chief Engineer shall prepare the report and notify Master in his daily noon report.

- In addition to daily checks, survey/take soundings of all fuel oil tanks manually on following occasions.

  - Weekly report to VM

  - At least once in three days, a water cut on sounding tape must be taken for fuel oil tanks (HFO / MDO / MGO storage tanks) for checking any ingress of water.

  - On arrival in port immediately after F.W.E

  - Within three to four hours prior departure from port

  - Prior to and upon completion of bunkering operations

Caution:

In order to curb malicious practices on-board vessels, unannounced bunker surveys shall be initiated. Such surveys shall be conducted by visiting vessel manager, marine superintendent or internal auditor on instructions from the Environment Compliance Manager or FGM.

During handing over /taking over of Chief Engineers as per CRV-02 – Incoming C/E shall personally verify/witness soundings and prepare bunker survey report for all fuel/diesel tanks.

- Report of such bunker survey shall be sent to: FGM, ECM and Head of Business Unit. For offshore vessels, below instructions shall be followed:

Follow the below procedure for verification and monitoring purposes:

- Tank soundings MUST be taken, recorded and signed off by two independent staff (e.g. 1 from engine room and one from bridge) before start of bunkering / fuel oil transfer operations and after completion of bunkering / fuel oil transfer operations.

- Contents of all individual tanks including day tanks (service + settling) MUST be recorded in the engine room logbook.

- Details of ALL internal fuel oil transfer operations MUST be recorded in the engine room logbook.

Note:

Samples are to be kept in a safe storage location, outside the ship’s accommodation and where personnel would not be exposed to concentrated vapours which may be released from the sample.

SAMPLE STORAGE AND INVENTORY

- Store the retained samples in a dedicated sample locker with:

  - Adequate storage space

  - With ventilation ensuring adequate air flow

  - A fixed firefighting system installation or portable fire extinguisher

  - –Locking arrangement to prevent tampering

  - Constructed of oil resistant material with suitable cells or divisions to prevent shifting of the sample bottles at sea

  - Sheltered location where they will not be subject to elevated temperatures and not exposed to direct sunlight

- Develop an inventory system (e.g. log book) to keep track of the retained samples.

Note:

The below mentioned internal transfer operations are applicable to storage tanks and exclude the daily transfer to settling and service tanks.

Avoid the storage tank bunker transfers during all commercial operations when the vessel is in port or at anchor.

Automatic shut off valves, remote gauging systems and high level alarms must not solely be relied on.

INTERNAL TRANSFER OPERATIONS

- An engineering officer shall:

  - Be in charge of internal transfers and this responsibility cannot be delegated to a rating.

  - Be experienced and ship specifically trained in all aspects of the transfer operation,

including knowledge of the limitations of the ship’s monitoring and control systems.

  - Not perform other duties interfering with this task.

  - Take manual soundings of both tanks throughout the whole operation.

  - Never leave internal transfer operations unattended

  - Utilize E22- Bunker safety checklist.

- The operation to be carried out during day light hours, where practical. If required to carry out Internal transfer during hours of darkness, Vessel manager to be notified and approval sought.

- Carry out a risk assessment of the transfer operation prior to commencement.

- The risk assessment shall include, at a minimum, but not be limited to:

  - Tank transfer discussed amongst the senior officers, to ensure:

    - Cross departmental planning.

    - Other aspects such as vessels list, stress and stability are in order.

  - A bunker transfer plan with tank details and the quantities.

  - Sufficient and dedicated personnel allotted for this work.

  - A pre-transfer meeting wherein the actual job details of the operation discussed in detail and any doubts clarified.

  - Test overfill alarms and other safety devices

  - During the actual bunker transfer:

    - Bridge informed to establish the watch on deck.

    - Ensure that all precautions are taken.

    - SOPEP equipment in readiness.

- –In case of any temporary repairs or soft patches on the transfer system pipelines:

  - Inform VM immediately

  - Do not commence any transfer without prior approval of the Head of Business Unit.

## DISCHARGE OF WASTE OIL OR SLUDGE

- Apply all company safety and environmental protection procedures relating to bunkering operations as an oil transfer procedure to the discharging of waste oil or sludge to another vessel or to shore reception facilities.

- Dispose oily waste by settling out the mixture and then passing the bottom liquid overboard through the oily water separator.

- Heat the contents of oily waste tanks to assist the process of settling

- Make every effort to reduce the water content of waste oil by evaporation.

- In ships where incinerators or dual fuel boilers are fitted, incinerate sludge and the oil

content of oily waste in accordance with the equipment maker’s instructions.

- Where local shore contractors are prepared to pay for oily waste products or shore disposal is free, take the opportunity to dispose off these residues.

- Attach a signed receipt for waste oil disposal to the relevant page in the Oil Record Book and forward a copy to the VM.

## FOR TANKERS ONLY: Pumping of engine room waste oil or sludge into cargo slop tanks

The pumping of engine room waste oil or sludge into cargo slop tanks is to be avoided if possible and other means of disposal (incineration) are to be used to the maximum extent possible.

Prior to pumping engine room sludge into cargo slop tanks, the following requirements shall apply:

## General requirement

- The "pumping into a cargo slop tank is legally allowed" ONLY IF:

Note:

In case the form B does not have this provision, for a one-off contingency, the VM will approach the classification society for permission to vessel to transfer sludge to slop tank, which may be granted at the discretion of classification society, subject to certain conditions being met.

This fact is mentioned on the document "Records of construction and equipment for oil tankers (form B)" checkbox 3.2.4, clearly mentioning the tanks in which the pumping is allowed.

- Make an appropriate entry in BOTH oil record books on for every transfer to slop tanks.

- Additional commercial requirements

  - Vessel loaded: Pumping of engine sludge into the cargo slop tank is NOT allowed without the prior written agreement from the VM and from the commercial department of your vessel.

  - Vessel in ballast: Pumping of engine sludge into the cargo slop tank is allowed if a written agreement from the VM has been received.

- –LOW SULPHUR MARINE GAS OIL (LSMGO) STRENGTHENING

Warning:

All vessels are required to comply with the following LSMGO strengthening requirements.

## Change over procedure

    - Vessel’s fuel oil change over procedure (for switching between residual fuel to distillate fuel) for main engine, auxiliary engine and auxiliary boiler should be prepared using the template E71 – Fuel oil change over procedure.

    - A sample procedure (prepared using the same template) is provided as “FOM 4.17 Sample me changeover procedure’.

    - Include maximum information with pictures (as applicable).

    - Send the procedure to Vessel Manager for approval.

    - Time required for complete change over from HFO to LSMGO should be calculated using appropriate tools (FOBAS / DNV calculators). Add two hours to the calculated time, for safe margin. Accordingly, time between 3-way valve change over and vessel touching ECA boundary should be at least two hours more than the time calculated by the calculator.

## Recommended spares onboard

    - List of spares recommended to be maintained on board for smooth long term operations on LSMGO, is provided in “E19- Recommended Spares onboard” for efficient continuous operations on LSMGO’.

## Recommended long term measures

    - “E-20 Recommended Fuel Oil System Features (for safe LSMGO usage)” provides information on recommended features for FO lines (Bunkering/Transfer/Service lines) to enable efficient and safe handling and usage of LSMGO.

## Performance tests

Note:

These performance checks will enable any deficiencies to be picked up early and timely action taken to ensure that equipment stays reliably available for usage on LSMGO.

“FOM 4.18 Performance tests’” provides guidance on the checks and measures for routine performance evaluation of engines, boilers, and fuel oil pumps.

## Pre-ECA zone checks

    - “E23- Pre-ECA zone checks’” should be carried out by vessels that are likely to trade in ECA.

    - Inform any requirements or concerns as identified from “pre-ECA zone checks” to the Vessel Manager, well before trading in the ECA region.

    - Take measures for rectification of identified deficiencies.

- FONAR

- FONAR to be sent to the flag Administration and to the competent authorities in the relevant port(s) of destination in accordance with regulation 18.2.4 of MARPOL Annex VI. The report shall be sent as soon as it is determined that the ship/operator will be unable

–to procure compliant fuel oil and preferably before the ship leaves the port/terminal where compliant fuel cannot be obtained. A copy of the FONAR should be kept on board for inspection for at least 36 months.

- FONAR should be used to provide evidence if a ship is unable to obtain fuel oil compliant with the provisions stipulated in regulations 14.1 or 14.4 of MARPOL Annex VI.

- Before filing a FONAR, the following should be observed by the ship/operator:

  - A Fuel Oil Non-Availability Report is not an exemption. According to regulation 18.2 of MARPOL Annex VI, it is the responsibility of the Party of the destination port, through its competent authority, to scrutinize the information provided and take action, as appropriate.

  - In the case of insufficiently supported and/or repeated claims of non-availability, the Party may require additional documentation and substantiation of fuel oil non- availability claims. The ship/operator may also be subject to more extensive inspections or examinations while in port.

  - Ships/operators are expected to take into account logistical conditions and/or terminal/port policies when planning bunkering, including but not limited to having to change berth or anchor within a port or terminal in order to obtain compliant fuel.

  - Ships/operators are expected to prepare as far as reasonably practicable to be able to operate on compliant fuel oils. This could include, but is not limited to, fuel oils with different viscosity and different sulphur content not exceeding regulatory requirements

- Kindly check with respective Port state if it has any specific form format for FONAR.

- Master should inform Vessel Manager and seek office approval prior submission of FONAR.

## REFERENCED FORMS AND CHECKLISTS

E71 – Fuel oil change over procedure

E22A - Fuel oil non-availability report (FONAR)