# SAFETY HEALTH AND ENVIRONMENT

  - Planning for safety

    - Fire fighting

      - Keep firefighting equipment ready for immediate use on the bunker tanker and the vessel at all times.

# Emergency preparedness

      - Conduct appropriate drills in accordance with the vessels’ approved contingency plans.

      - Document such drills when carried out.

# Condition of crew

      - The masters of the bunker tankers and the vessel shall ensure that they, as well as all senior personnel and crew members involved in the bunkering operation, are:

        - Properly rested

        - Not under the influence of alcohol or any controlled drug

# Bunkering pre-delivery safety checklist

      - The safety checklist for bunkering operations as set out in E22 shall be:

        - Presented by the representative of the bunker tanker to the representative of the vessel at the pre-delivery conference.

        - Used to adhere to the relevant safety procedures of a bunkering operation.

        - Should be supplemented by continuous vigilance throughout the whole bunkering operation.

# Observing Safe Work Practices

    - Smoking and naked lights

      - Strictly enforce regulations regarding smoking and the use of naked lights

      - Display warning notices clearly and appropriately

# Safe access

      - The vessel shall provide safe ladder(s) in compliance with SOLAS regulations

      - The access shall be safely rigged and properly illuminated during darkness

      - Personnel shall use only the designated means of access between the bunker tanker and vessel

# Outboard deck way

      - Walk on the outboard side of the main deck to avoid exposure to falling objects from vessels such as lashing materials

# Unauthorized craft

      - No unauthorized craft shall be allowed alongside either vessel throughout the bunkering operation unless permitted by the respective duty officers

# General communications

    - Before bunkering operations commence, establish:

      - A common language for communication

      - Hand signals for communication between the parties

      - Emergency signal in the form of an air horn and/or hand signals

    - During bunkering operations:

      - Arrange a reliable, common means of communication at all times, including a backup system and spare radios/ batteries

    - In the event of a breakdown of communications:

      - Sound the emergency signal

      - Suspend all operations in progress

Note:

Bunkering operations should not be resumed until satisfactory communications have been re- established.

| 1. Wait/hold | 2. Start | 3. Reduce pumping rate |
| --- | --- | --- |
| 4. Increase pumping rate | 5. Stop | 6. Finish |

# Figure 1:  Hand Signals

  - Emergency shut down

    - Every crew member of the bunker tanker and vessel is empowered to initiate an emergency shut down of the bunkering operation in an emergency or suspicious circumstance

    - The bunkering operations must remain suspended until it is agreed between the relevant persons/authorities that it is safe to resume

# Preventing Exposure to Health Hazards Hydrogen sulphide (H2S) & Benzene

| Gas | TLV-TWA (exposure limit for an 8 hour workday) |
| --- | --- |
| Benzene (C6H6) | 0.5 ppm |
| Hydrogen Sulphide (H2S) | 1 ppm |

H2S is a colorless, transparent gas with a characteristic rotten-egg odour at low concentrations. At high concentrations, it has a sweetish odour, which may not be able to be detected at very high concentrations.

Benzene is colourless, and it has sweet, aromatic, gasoline-like odour, and can be absorbed directly from skin and by breathing.

For details on the associated hazards and PPE, refer to OTM 7.1 Hazards of oil cargoes & FOM 10.5 Ship cargo specific PPE requirements.

# Measures during bunkering operation for prevention of exposure to Hydrogen Sulphide (H2S) & Benzene

Immediately after commencement of bunkering operation, the engineer in charge shall measure the H2S & Benzene gas concentration through the air vent of Fuel Tank being bunkered, using shipboard Gas detection equipment under supervision of the Chief Engineer.

In case the concentration is found higher than 50% of TLV-TWA of H2S or Benzene, following measures should be taken:

Note:

Gas check on open deck areas should be made downwind of vents to ensure any hazardous gas is sensed. Adequate PPE to be used as necessary.

    - The personnel involved in bunker operation at site shall put proper PPE (a breathing apparatus of approved standard)

    - The openings to the accommodation area near fuel tank air vent shall be closed completely

    - All personnel should be kept away from areas where bunker vapours are suspected (e.g. FO Air Vent, manifold area and sounding area), other than the persons directly involved with bunkering

    - Until completion of bunkering, the gas concentration measuring shall be carried out at regular interval (at least once in an hour)