1	BUNKERING

- Supplier’s quality certificate should be closely checked, and compliance with ISO 8217

standards confirmed.

- As far as possible dedicated tanks should be used for bunkering ULSFO / VLSFO due to very low compatibility (tolerance to mixing).

- If dedicated tanks are unable to be used, fuel should be bunkered only in clean tanks (stripped to maximum extent). Mixing should be as minimum as possible, with ratio kept well within the range recommended by the fuel supplier.

- Compatibility test should be carried out before deciding to mix hybrid fuel with any other hybrid or conventional fuel, if it is unavoidable to mix. If available time does not allow getting compatibility tested from laboratory, on board test using a test kit  may be carried out.

- Fuel should only be bunkered in tanks with heating coils, and temperature maintained well above the cloud point, and at least 10 deg. C above the pour point.

- After bunkering operations, the lines should be well drained and blown to ensure that no residual fuel stays in the line, in order to avoid mixing of this with the next product.

- Enhanced fuel sample tests (example Asphaltene analyses, particle count test, GC-MC screening, stability, acidity – pH, Toulene Insoluble) are recommended to be carried out.

2	TRANSFERRING / HANDLING

- Settling and Service tanks used for ULSFO / VLSFO should either be dedicated tanks, or stripped thoroughly to ensure mixing is minimum, with ratio kept well within the range recommended by the fuel supplier.

- Settling and Service Tanks should have heating coils, in order that sufficient temperature can be maintained for purification and preheating process for combustion.

- Since overflow tank contents are usually transferred back to settling tank, to avoid mixing of hybrid fuel with other fuels in Overflow Tanks, the tank should be emptied before being lined up for a different fuel (new fuel grade or a different hybrid product).

3	PURIFICATION

- Purification should be diligently carried out to separate sludge and cat fines (that hybrid fuels

/ blended fuels are prone to contain):

  - Desludging interval should be set at 30 minutes.

  - Gravity disc of correct size should be used, as suitable for the fuel density (depending on the type of purifier). Page 2 of 2 FOM 4.14 Guidelines on usage of hybrid

/ blended residual fuels – ULSFO & VLSFO

  - Purifier should be used with minimum throughput

  - Purification temperature as recommended in FOM 4.12 Table A Recommended Fuel Cleaning temperature in the separator should be maintained

- Settling Tank should be maintained at adequate temperatures, and drained at regular intervals

4	USAGE / SERVICE LINE

- Viscosity controller should be confirmed to be working in order that fuel heating is maintained to achieve appropriate viscosity. Else, fuel temperature should be maintained at correct level (for injection viscosity), as recommended in the lab test report (based on measured viscosity).

- Cylinder lube oil of appropriate TBN should be used, as per recommendations of the engine maker. Generally cylinder oil TBN lower than 40 is recommended to be used in conjunction with fuels of sulphur content below 0.5%, and TBN below 20 in conjunction with fuel of sulphur content below 0.1%

- Due to risks from incompatibility and cleaning effects of the fuel, filter choking can be expected. Therefore:

  - Fuel Filters should be ensured to be clean prior change over to a different hybrid product or a distillate fuel.

  - During the process of change over, and immediately after, fuel pressure and differential pressure across the filters should be carefully monitored, and filters changed over / cleaned, if required.

  - All FO filters in use, should be cleaned after 6 to 8 hours of change over to a different hybrid product or to a distillate fuel (Example, FO Suction Filter, Flow Meter FO Filter, Fuel Oil Hot Filter / Auto Backwash Filters in both ME and AE service line).

- All fuel parameters and Main Engine performance during and after change over should be closely monitored, in order that timely action can be taken in case of any abnormalities.

- Sufficient reserve stock of fuel additives to be maintained in case any problem with fuel is observed due to stability and compatibility.