# BENZENE FUELS HANDLING PROCEDURES

# Benzene in fuels

        - Benzene is a highly flammable and volatile liquid, which occurs naturally in crude oil, gasoline & other fuels, natural gas, etc. It was formerly used as a solvent, but in most cases, safer substances have now replaced this use.

        - Petrol contains below 1% benzene. Small amounts of benzene are produced when some organic substances burn incompletely, e.g. it is found in cigarette smoke and vehicle exhausts. Benzene evaporates easily, and most people can just detect its distinctive smell at concentrations between 2.5 and 5 parts per million (ppm) in air.

        - Benzene can occur in Fuels commonly used on board the vessel and therefore sufficient precautions should be taken to ensure that personnel involved with Bunkering or handling the fuel are not exposed to high levels of Benzene.

        - When fuel with benzene is present on board, special care must be taken for routine work on fuel system components, such as filters, purifiers, sludge handling, rags, etc or when opening sounding pipes and in vicinity of vents to fuel tanks.

          - Any maintenance work should be carried out in a well ventilated area, where vapors are not allowed to collect. When working on fuel system components in confined spaces, forced ventilation or extraction fans should be used.

          - When opening sounding pipes for ullaging, it is recommended to stand to one side of the pipe, neither upwind nor downwind to prevent contact with vapors coming from the pipe. This also applies to fuel tank vents.

          - Since benzene can be absorbed by the skin in small amounts, personnel should wear suitable protective gloves / eye protection / face shield when handling fuels containing benzene if there is a possibility of the fuel coming in contact with the skin.

        - The below is information regarding Benzene in pure form.

Benzene is a clear, colorless liquid with a pleasant, sweet odor. The odor of benzene does not provide adequate warning of its hazard. Benzene liquid is highly flammable. It should be stored in tightly closed containers in a cool, well ventilated area.

Benzene vapor may form explosive mixtures in air. All sources of ignition must be controlled. Benzene vapors are heavier than air; thus the vapors may travel along the ground and be ignited by open flames or sparks at locations remote from the site at which benzene is handled.

# Permissible Exposure

        - Airborne: The maximum time-weighted average (TWA) exposure limit is 0.5 part of benzene vapor per million parts of air (0.5 ppm) for an 8-hour workday and the maximum short-term exposure limit (STEL) is 5 ppm for any 15-minute period.

        - Dermal: Eye contact shall be prevented and skin contact with liquid benzene shall be limited.

# Health Hazard Data

        - Benzene can affect your health if you inhale it, or if it comes in contact with your skin or eyes. Benzene is also harmful if you happen to swallow it.

        - Effects of overexposure.

          - Short-term (acute) overexposure: If you are overexposed to high concentrations of benzene, well above the levels where its odor is first recognizable, you may feel breathless, irritable, euphoric, or giddy; you may experience irritation in eyes, nose, and respiratory tract. You may develop a headache, feel dizzy, nauseated, or intoxicated. Severe exposures may lead to convulsions and loss of consciousness.

          - Long-term (chronic) exposure. Repeated or prolonged exposure to benzene, even at relatively low concentrations, may result in various blood disorders, ranging from anemia to leukemia, an irreversible, fatal disease. Many blood disorders associated with benzene exposure may occur without symptoms.

# Protective Clothing and Equipment

        - Respirators. Respirators are required for those operations in which engineering controls or work practice controls are not feasible to reduce exposure to the permissible level.

        - Protective Clothing. You must wear appropriate protective clothing (such as boots, gloves, sleeves, aprons, etc.) over any parts of your body that could be exposed to liquid benzene.

        - Eye and Face Protection. You must wear splash-proof safety goggles if it is possible that benzene may get into your eyes. In addition, you must wear a face shield if your face could be splashed with benzene liquid.

# Emergency and First Aid Procedures

        - Eye and face exposure. If benzene is splashed in your eyes, wash it out immediately with large amounts of water. If irritation persists or vision appears to be affected, see a doctor as soon as possible.

        - Skin exposure. If benzene is spilled on your clothing or skin, remove the contaminated clothing and wash the exposed skin with large amounts of water and soap immediately. Wash contaminated clothing before you wear it again.

        - Breathing. If you or any other person breathes in large amounts of benzene, get the exposed person to fresh air at once. Apply artificial respiration if breathing has stopped.  Never enter any vessel or confined space where the benzene concentration might be high without proper safety equipment.

        - Swallowing. If benzene has been swallowed and the patient is conscious, do not induce vomiting. Call for medical assistance or a doctor immediately.

-x-