# OIL TRANSFER PROCEDURE

Internal oil transfers are a potential source of pollution and require extra attention similar to bunkering. All internal transfers are to be recorded in the Oil Record Book.

# BUNKER OIL TRANSFER PROCEDURES IN USA

Bunkering procedures in US are as per the USCG rules & regulations. Salient points of these regulations are as follows:

        - Master & Chief Engineer shall ensure total compliance with local regulations for each state of USA. It is to note that in US many states have their own laws with respect to reporting and recording prior, during and after bunkering. In some states different forms may also be required. Master shall find out all such requirements from local agent of the port well before arrival and bunkering operation. In any doubt master shall contact company.

        - The Chief Engineer is responsible for the bunkering operations and shall have total control at all times.

        - The Deck Officer of the watch is responsible for moorings to the pier, barges, etc., and for ensuring that bunker hoses and other equipment does not become snagged or fouled in any way.

        - When bunkering operations are in progress there must always be one (1) person stationed by the bunker connection.  Likewise, one (1) person must be dedicated to continuously take soundings in addition to the Engineer officer assisting in Bunkering. In addition to it a deck rover watch to be carried out by One (1) person whose primary duty will be to monitor for oil spill on deck & over the side during bunkering. Additionally, deck rover should visually inspect the water near or opposite all bunker tanks, sounding pipes & vents if accessible. Deck Rover should be in position to view spillage on deck or in water during tank change over & topping up.

Note I: Names of these persons shall be posted on the Bunkering Instructions during bunker operations.

Note II: When changing the watch keeper names on the Bunker Instructions should also be changed.

        - The bunkering procedure and a list of the persons involved in the bunkering shall be posted by the bunker connection. Tank plans and oil transfer diagrams shall be available at all times.

        - All valves on the bunker line connected to the tank, which is being filled, shall be open and labelled to that effect.  The valves to all other tanks shall be kept closed.

        - During the bunkering all other manifold connections shall be blanked off.

        - Pipelines connected to the bunker systems shall be blanked off when not in use.

        - When the tank is 80% full the bunkering rate shall be reduced. In Case of Double bottom tank bunkering rate to be reduced at 75% of the tank capacity by volume.

        - Starting and stopping of bunkering operations shall be reported to the Deck Officer of the Watch.

        - This bunkering procedure shall be available in the languages used by the ship's complement.

        - In case of any accidents or spills the Master or the Chief Officer shall be informed without delay in order that they can inform the USCG. The Master or the Chief Officer shall report the incident to the USCG immediately, and at the same time inform the Company, QI, ship's agent and the local representative of the P&I 	Club.

        - It is advisable not to start bunkering operations until the permission to commence bunkering has been granted by USCG.  In any case the ship's agent should be consulted on this matter as several of the States have this rule.  Fines may be imposed in case of violation.

        - An emergency shutdown procedure shall be agreed with the bunkers supplier in the event of something happening during bunkering operations.

Reference: "CODE OF FEDERAL REG - NAVIGATION AND NAVIGABLE WATERS", §155.700 Subpart C - Transfer Personnel, Procedures, Equipment, & Records - for Bunker Transfer in US.

# Internal oil transfers at sea.

        - Internal oil transfer from bunker tank to bunker tank should be avoided. It is not allowed to comingle any fuel which is not tested. Comingling, if required for making space in bunker tank should be not be more than 5%.

        - For this purpose the chief engineer and chief officer should plan bunkering, bunker consumption, ballast condition and cargo condition, such that bunker transfers are not necessary for stability purpose.

        - Internal oil transfers from bunker tank to settling or day tanks, if manual, are to be monitored by the duty engineer from start to finish. The duty engineer is not to be allotted any other maintenance work during such time.

        - Internal oil transfer from bunker tank to settling tank, if automatic, is monitored daily by noting settling tank level in the logbook. In addition, the auto start/stop of the fuel oil transfer pump is to be tested every month and entered in the PMS. Automatic transfers are to be recorded in the Oil Record Book once daily.

# Internal oil transfer in port

        - Internal oil transfer from bunker tank to bunker tank is prohibited in port.

        - If it is extremely necessary to transfer oil internally then all procedures and precautions as in bunkering operation, to be followed. In addition, the port and terminal regulations are to be ascertained and followed.

        - Internal oil transfer from bunker tank to settling or day tank is to be avoided in port by manually filling settling tank before arrival port.

        - If due to long port stay or high consumption, a transfer from bunker tank to settling tank is necessary, and then the automatic transfer mode should be selected and frequent checks made of the transfer pump and the settling tank level during anticipated start/stop time of the pump.

-x-