| S/N | Equipment | Frequency | Items to check |
| --- | --- | --- | --- |
| A1 | Main engines | Monthly | Main Engine to be tested at 75% to 80% load. |
| A2 |  |  | Performance parameters to be noted and compared with corresponding parameters at corresponding load during Sea Trials / Shop Trials. |
| A3 |  |  | Leakages at Fuel Injection Pumps to be checked. |
| A4 |  |  | At least quarterly (every 3 months), main engine to be tested on LSMGO. Opportunities of sailing in ECA should be used for main engine performance measurements on LSMGO. |
| A5 |  |  | Fuel racks during performance test to be compared with rack positions at similar load during sea trials / shop trials, and condition of Fuel pumps verified. When measurements are taken with the engine running on LSMGO, change in the rack positions in comparison to the corresponding positions in HFO operation (for similar load) should also be analysed as an indication of the deterioration in fuel pump plunger and barrel condition. |
| A6 |  |  | On Wartsila RT FLEX Engines, check following and compare with maker recommended levels, to ensure integrity of fuel system: |

| S/N | Equipment | Frequency | Items to check |
| --- | --- | --- | --- |
|  |  |  | - Rate of pressure drop of Common Rail System on stopping the engine - Rate of fuel leakage at various leakage monitoring points (ICU etc.) in Fuel Leakage System |
| A7 |  |  | M/ Engine to be tested ahead & astern. Astern tested when the ship is moving at 4~5 kts speed ahead (with propeller not turning). |
| A8 |  |  | Funnel smoke checked and confirmed to be satisfactory. |
| A9 |  |  | Any limitations in loading the main Engine up to 80% rated load should be identified, and appropriate measures taken for rectification of the condition, in line with maker’s guidelines. |
| A10 |  |  | For MAN main engines of the type ‘ME’, check following and compare with makers recommended levels, to assess the extent of internal leakages in the system (FIVA Pilot valves, hydraulic pumps, etc.), and record the values for assessing the trend. Maximum hydraulic pressure build-up by the pumps (usually 220 ~ 225 Bar). Time taken to build up the pressure. |
| B1 | Auxiliary engines | Monthly & before calling ECA. | Auxiliary engines to be tested at 80% load on LSMGO. For effective analyses of the performance trend, the measurements should be recorded at a consistent load every month, as far as practical. |
| B2 |  |  | Performance parameters to be noted and compared with corresponding parameters at corresponding load during sea Trials / shop Trials. In case of significant deviations, remedial measures to be taken. |
| B3 |  |  | Leakages at fuel injection pumps to be checked, and rectified. |
| B4 |  |  | Fuel racks during performance test on LSMGO compared with rack position corresponding to HFO operation (and rack positions at similar load during sea trials / shop trials) and condition of Fuel pumps verified. In case fuel rack is noted increased by 10%, problem to be rectified, and plunger and barrel renewal to be considered. |
| B5 |  |  | Funnel smoke checked and confirmed to be satisfactory. |
| B6 |  |  | Any limitations in loading the auxiliary engines up to 80% rated load should be identified, and appropriate measures taken for rectification of the condition, in line with maker’s guidelines. |

| S/N | Equipment | Frequency | Items to check |
| --- | --- | --- | --- |
| C1 | Boiler | Quarterly (every 3 months) & before calling ECA. | Boiler to be run at minimum 75% load on LSMGO. |
| C2 |  |  | Air Fuel ratio to be checked and confirm the IG O2 can be maintained at higher loads. |
| C3 |  |  | Funnel Smoke and Flame colour checked and confirmed to be satisfactory. If required adjustments to be carried out. |
| C4 |  |  | Any limitations in loading the Boiler up to 80% rated load should be identified, and appropriate measures taken for rectification of the condition, in line with maker’s guidelines. |
| D1 | Fuel supply, circulating & booster pumps (for AE/ ME/ boiler) | Quarterly (every 3 months) & before calling ECA. | Pump performance to be checked in conjunction with running of equipment at minimum 75% load on LSMGO. |
| D2 |  |  | Pressure developed to be noted and confirmed to be satisfactory when the Equipment is used at minimum 75% load. If needed, Fuel Oil Line pressure / back pressure to be adjusted. |
| D3 |  |  | Pump to be assessed for any leakages (internal & external) |
| D4 |  |  | Any limitations / leakages to be identified, and appropriate measures taken for rectification of the condition, in line with maker’s guidelines. |