# BUNKER HANDLING ECA AREA

Do not Take Bunker.

Inform Office /Supplier/Agent/Charterer

NO

As per BDN is the Sulphur Content Within Specified Limits as per MARPOL Annex VI Reg.14(4)

Do Not Use Bunker till analysis report is received. In case need to use untested Bunker, Inform Office

YES

As per DNV/FOBAS fuel analysis report is the Sulphur Content Within Specified Limits as per MARPOL Annex VI Reg.14(4)

Inform to Owner/Charterer/Office and discuss.

If the fuel is to be used in Emission control area is Time Available for Re-testing retained Sample?

NO

YES

YES

Use Bunker After taking precautions as per bunker analysis report & PIC advice

NO

ERP-05-05A Initial Bunker Report to be sent to Office

Master to send Notification to

- Flag Administration

- Port State Control ( Next  Port Of Call)

- Office, Owners & Charterer’s

- Local Agent /Supplier

Send retained sample to Re-test after mutual agreement with supplier.

In Case Next port of call comes under Emission control area inform Port State control (Next Port OF Call) about the facts and take permission to arrive with bunkers available onboard.

As per DNV/FOBAS fuel analysis report after Re-Testing Fuel Sample. The Sulphur Content Within Specified Limits as per MARPOL Annex VI Reg.14 (4)

NO

PSC Approves to use Bunker

NO

YES

YES

Use Bunker only after taking approval from Vessel PIC

- Inform Company and charterer.

- Take New Bunker.

After this Port if Next Port in emission control Area?

Note: - In above Situation vessel should pay special emphasize to keep close communication with Office, Owner & Charterer’s.

- MARPOL ANNEX VI (SOX EMISSION CONTROL) – REGULATION 14

SOx emission controls apply to all fuel oil for usage on combustion equipment and devices onboard - main engines, auxiliary engines, boilers inert gas generators etc.

Maximum sulphur content (expressed in terms of % m/m – that is by mass) of the fuel oils as loaded, bunkered, and subsequently used onboard are limited by values stated in following table.

| Outside an ECA | Inside an ECA |
| --- | --- |
| 0.50% m/m on and after 1 January 2020 | 0.10% m/m on and after 1 January 2015 |

The SOx ECAs established are:

  - Baltic Sea area – as defined in Annex I of MARPOL

  - North Sea area – as defined in Annex V of MARPOL

  - North American area – as defined in Appendix VII of Annex VI of MARPOL

  - United States Caribbean Sea area – as defined in Appendix VII of Annex VI of MARPOL

Alternately, vessels may be installed with scrubbers (Exhaust Gas Cleaning Systems) to achieve equivalent levels of SOx emission control, both outside and inside ECA.

# NON-COMPLIANT FUEL CARRIAGE BAN

In accordance with MEPC.1/Circ.881, from 1 March 2020 onward, there is a ban on carriage of non-compliant fuel oil (Sulphur content >0.5%) onboard, unless the vessel is installed with abatement technology such as scrubbers (Exhaust Gas Cleaning Systems).

# EMISSIONS CONTROL AREAS – LOCAL PORT REGULATIONS

China Emission Control Areas

International ships entering China waters (designated ‘Inland Waterways ECA’, except Hainan

waters), must comply with fuel oil Sulphur content (m/m) limit of 0.1%.

With effect from 01st January 2022, limit of 0.1% Sulphur content (m/m) will also be applicable to Hainan Waters within ECA.

# European Emission Control Areas

Vessels must use fuels with Sulphur content below 0.10% (m/m) in the territorial waters, exclusive economic zones and pollution control zones (SOx Emission Control Areas) of the member states of European Union.

Ships at berth in European Union ports should not use marine fuels with a sulphur content exceeding 0.10 % by mass. Necessary fuel-changeover operation should be completed as soon as possible after arrival at berth, and as late as possible before departure.

# Panama Canal Emission Control Area

Vessels arriving at Panama Canal (ACP) waters (other than vessels making only a local port call and not transiting) shall switch residual fuels to marine distillate fuels being used for their main propulsion engines, boilers, auxiliary generator engines, and other ancillary equipment. Panama Canal waters consist of the Pacific and Atlantic Anchorage areas and the fresh water reservoirs, as outlined in Nautical Charts DMA No. 21603 and DMA No. 26068, respectively.

Vessels may supplement or replace marine distillate fuels with any of the following:

  - LNG fuel (including boil-off gas)

  - Biofuels compliant with MARPOL Annex VI

  - ULSFO or VLSFO not exceeding a viscosity of 70 centistokes @ 50°C. These vessels shall submit to the ACP for evaluation, at least 96 hours prior to ETA at Panama Canal waters, a copy of the corresponding bunker delivery note.

  - A closed loop scrubber in zero discharge mode or a hybrid scrubber in closed loop and zero discharge mode

Vessels opting to use closed loop scrubbers or hybrid scrubbers in closed loop and zero discharge mode, shall submit the following to the ACP for evaluation, at least 96 hours prior to ETA at Panama Canal waters:

  - Copy of the Supplement to the IAPP Certificate

  - Total volume of effluent holding tanks

  - Total time (in hours) required to reach the capacity of the effluent holding tanks when operating in zero discharge mode. Effluent holding tanks shall be empty upon arrival at Panama Canal waters.

Vessels anchoring prior to transit will be permitted to use marine residual fuel with a maximum sulphur content of 0.5% to operate their auxiliary generator engines, boilers, and other ancillary equipment while at the Pacific or Atlantic Anchorages, only if they are capable of maintaining their main propulsion engines simultaneously on marine distillate fuel. Such vessels shall switch from the fuel being used for their auxiliary generators, boilers, and other ancillary equipment to marine distillate fuel at least two hours prior to the scheduled pilot time for their Canal transit.

Note:

In Emission Control Areas, vessels can employ alternative measures (equivalent to usage of fuel with Sulphur content not exceeding 0.1% m/m), such as using shore power or scrubbers (Exhaust Gas Cleaning Systems) or other clean energy technologies.

Vessels proceeding to port terminals or anchorages within Canal waters immediately after completing their transit will be permitted to switch back to residual marine fuel with a maximum sulphur content of 0.5%, once the vessel is moored alongside the dock or is anchored.

# LOCAL PORT REGULATIONS REGARDING RESTRICTIONS ON SCRUBBERS

Wash water from open loop scrubbers must be treated and monitored at the inlet and outlet to ensure that it conforms with the MEPC 259(68) discharge criteria, with no risk of harm to the environment. However, some coastal states and ports have implemented local regulations with more stringent requirements for the quality of wash water being discharged overboard. Details of some such ports are given below:

| China | Discharge prohibited in inland & coastal ECAs and Bohai Bay waters. **Ban in the entire coastal domestic ECA may be announced soon. |
| --- | --- |
| Hong Kong | Discharge may be allowed but vessels are to request permission to use non-compliant fuel 14 days before arrival. |
| Singapore | Use of open loop scrubbers prohibited within Singapore port limit. |
| Malaysia | Use of open loop scrubbers prohibited throughout Malaysian territorial waters & ports. |
| India | Generally permitted but some ports may have more stringent requirements in place. |
| UAE | Abu Dhabi may allow discharge overboard if free from pollutants, whilst scrubber sludge should be discharged to a licensed waste disposal contractor. Fujairah has banned the use of open loop scrubbers in territorial waters. |
| Belgium | Discharge prohibited in ports and inland waterways. |
| Germany | Discharge prohibited in inland waterways and on the Rhine. |
| Lithuania | Discharge prohibited when in port. |
| Latvia | Discharge prohibited in territorial and port waters. |
| Ireland | Discharge prohibited in ports of Dublin, Waterford and Cork. |
| Norway | Discharge prohibited in Norwegian fjords (visible emissions reduction also required). |
| Panama | Use of open loop scrubbers prohibited in Panama Canal waters. |
| Hawaii | Currently permitted subject to special conditions. |
| Connecticut | Discharge prohibited. |
| California | Use of scrubbers is banned. |
| Note: Vessels Master to communicate with the local agents, as soon as next port is known to check for local port regulations for following, and to comply accordingly: - Allowed fuel sulphur content - The exact position for fuel changeover / scrubber mode change over - Allowance for usage of open loop scrubbers / Any wash water discharge restrictions |  |

# GUIDANCE REGARDING CHANGE OVER IN ECAS

Vessels without scrubbers (or with scrubbers out of operation) entering ECA

  - Vessels calling these areas should maintain enough stock of compliant bunker fuels. Required quantities of compliant fuels should be determined (in accordance with FOM

4.12 ‘Bunker management and operations’, chapter 6 ‘Bunker quantities’), and bunkering carried out in advance, in co-ordination with the charterer, operator and vessel manager.

  - Prior to entry into the ECA, vessel is required to have fully changed-over to using the ECA compliant fuel oil. Similarly change-over from using the ECA compliant fuel oil is not to commence until after exiting the ECA.

  - Time between 3-way valve change over and vessel touching ECA boundary should be at least two hours more than the time calculated by the calculator (FOBAS / DNV Calculator).

  - Fuel change overs should be carried out timely to ensure that the vessel is using the compliant fuel, in line with the local requirements, closely following the vessel specific procedures approved by Vessel Manager

  - At each change-over, quantities of the ECA compliant fuel oils onboard should be recorded, together with the date, time and position of the ship when either completing the change-over prior to entry or commencing change-over after exit from such areas.

  - These records are to be made in a logbook as prescribed by the ship's flag State, or in the ship's Annex I Oil Record Book.

  - Records should be duly maintained to demonstrate vessel’s compliance, by regular entries

in MARPOL Annex VI Record Book (Green Book):

- Section 1 (regarding bunkers received)

- Section 4 (details of fuel change over events)

- Bunker Delivery Notes (BDNs), and representative samples of the low Sulphur fuel oil are retained on board to maintain records of the quality of fuel bunkered.

- For situations where using low Sulphur fuel oil is unsafe for the vessel or impossible due to failure to obtain qualified low Sulphur fuel, vessel should, in advance, apply for exemption through prior communication with local port authorities, through agents.

# Vessels with scrubbers in operation entering ECA

- Change over Scrubber to ECA Mode i.e 0.1% sulphur mode, well before entering ECA and make appropriate log entries. Changing back of scrubber to non-ECA mode, i.e. 0.5% sulphur mode, is not to commence until after exiting the ECA.

- Vessels fitted with open-loop scrubber must check with ports state or local agents regarding scrubber wash water discharge restrictions.

- Entry must be made in EGCS log book and in a logbook as prescribed by the ship's flag State for change of scrubber mode.

- NITROGEN OXIDES (NOX) – REGULATION 13 (MARPOL ANNEX VI)

A marine diesel engine of over 130 kW output power (other than those used solely for emergency purposes) that is installed on a ship constructed on or after the following dates and operating in the following Emission Control Areas shall comply with the Tier III NOx standard:

  - 1 January 2016 and operating in the North American ECA and the United States Caribbean Sea ECA

  - 1 January 2021 and operating in the Baltic Sea ECA or the North Sea ECA.

Tier II NOx standards apply outside the conditions stated above. In accordance with regulation 13.5.2, certain small ships would not be required to install Tier III engines.

Vessels should maintain following documentation to demonstrate compliance to the applicable NOx emission standards, in accordance with the requirements of the mandatory, regulations

13.8 and 5.3.2 respectively, NOx Technical Code 2008:

  - Engine International Air Pollution Prevention (EIAPP) Certificate

  - Updated NOx Technical Files for in service compliance

Records to demonstrate that engines have been throughout operated in the NOx Tier III mode, wherever applicable, as per the conditions stated above (in ECAs, based on the