# HIGH H2S BUNKER Handling PROCEDURE

Marine bunkers may contain H2S, but in varying concentrations depending on how it is manufactured. Exposure to high levels of H2S gas can be fatal and inhalation has resulted in seafarers’ lives being lost.

# Table - Typical effects of exposure to hydrogen sulphide (H2S)

| H2S Concentration | Effects |
| --- | --- |
| 0.0047 ppm | The recognition threshold of human smell and the concentration at which 50% Of humans can detect the odor of rotten eggs. |
| 2 ppm | Long term exposure may result in headaches, dizziness, fatigue, loss of appetite and irritability |
| 5 ppm | The maximum exposure limit for an 8 hour period. |
| 10 ppm | The maximum exposure limit for a 15 minute period |
| 10-20 ppm | The borderline concentration for eye irritation |
| 50-100 ppm | Can lead to eye damage |
| 100-150 ppm | The olfactory nerve in the noise is paralysed after a few inhalations and sense of smell disappears, often together with an awareness of danger. |
| 320-530 ppm | Leads to pulmonary oedema (fluid in the lungs) with the associated symptons of shortness to breath or difficulty breathing, anxiety, apprehension, sweating with the possibility of death |
| 530-1000 ppm | Causes strong stimulation of the central nervous system and rapid breathing, leading to loss of breathing. |
| 800 ppm | Lethal to 50% of humans after 5 minutes exposure |
| >1000 ppm | Can cause immediate collapse with loss of breathing, even after a single breath. |

Concentrations and Effects of Hydrogen Sulphide Gas on Humans

# Characteristics of H2S

        - The gas is pungent, colorless, highly toxic and flammable.

        - H2S gas is highly toxic and has a distinctive odor of rotten eggs.

        - The smell intensifies as the gas concentration increases, and above 30ppm the odor becomes sickeningly sweet.

        - Concentrations exceeding 100 ppm paralyze nerves in the nose resulting in loss of smell giving false impression of no H2S.

        - Breathing difficulties may be experienced above 320 ppm, and exposure to concentrations beyond this level may be lethal.

        - A single breath of H2S gas at 1,000 ppm may result in immediate collapse and suffocation.

          - Prior to taking bunkers, SOLAS Regulation VI/5-1 requires the vessel to be provided with a Material Safety Data Sheet (MSDS). It will contain information regarding the composition of the fuel including the concentration of H2S, the effects of exposure to the gas and the first aid measures to be taken. In addition, the MSDS includes guidance on the handling & storage of the product and exposure limits. Please note that MSDS may be generic & not contain reference to H2S.

          - It is important to recognize that there is no direct correlation between the concentration of H2S in the bunker oil on board in a bunker tank and the amount of gas that may be generated or the speed at which it may develop. These issues cannot be predicted with accuracy as they depend on many factors including the chemical properties of the oil, temperature, viscosity, heating, storage time, agitation, tank shape and ventilation.

          - International Standards Organization (ISO) Fuel Standard ISO 8217:2010 limits the H2S content of residual and distillate marine fuels to 2 ppm (mg/kg) by weight in the liquid and not as vapour or gas concentration. However, it should be borne in mind that tanks containing such bunkers may still produce H2S in vapour form concentrations which will be hazardous. The fuel supplied may not adhere to ISO8217:2010 standard.

# Procedure to Measure H2S

        - H2S is Heavier than air hence it should be tested for at the bunker tank vent & area around vents on deck.

        - The testing frequency should be increased if the initial reading is high. Under normal concentration the testing frequency should be every hour for first 2 hours & if the ppm remains stable then the frequency to be increased to every 4 ~6hours. H2S May be measured by means of multi-gas meters.

        - If the concentration is high during the first test, then the frequency should be reduced to 30 min or even more frequent depending on the concentration.

# Safety Precautions- For High H2S

Bunkering

        - When the concentration of H2S exceeds the TLV (Threshold limit value)-TWA (Time weight Average) (5 ppm by volume in air) and the vapour does not disperse easily, considerations should be given for suspending bunkering operations in still wind conditions.

        - Personnel should always carry personal/portable gas monitors when engaged in bunkering operations and working in enclosed spaces.

        - If H2S >5ppm minimum ship crew should be put into the area of high H2S with SCBA set and other crew to be on standby to assist them in case of any emergency. They should be used in conjunction with a personal H2S gas monitoring/alarm instrument.

        - Measurements of H2S should also be done within Engine room and accommodation.

        - Personnel should remain on windward side of the vents, opening from bunker tanks as H2S may be released. Watch should be kept for change of wind direction/speed & relevant person should be informed of any change.

Handling

        - Within the engine room fuel oils will be treated (purified), handled and heated to very much higher temperatures, which in turn will cause greater evolution of this gas. Notwithstanding the forced ventilation in this area, H2S being heavier than air will cause it to accumulate in the machinery space bilges and below the lower engine room plates in case of spillage.

        - In case if H2S is accumulated in high concentration due to FO spill, engine room should be treated in the same way as an entry into an enclosed space.

        - Wherever possible, bunker storage tanks in machinery spaces that has sounding pipes that terminate within the space, side level gauges to be used so that the tank contents can be determined without opening the sounding pipe cap. As an additional safety measure, such sounding pipes caps or gravity dead-weight closing device to be kept closed at all time.

        - Suitable notices should be posted to warn crewmembers that marine bunker tanks and residual dirty oil tanks may contain H2S gas, and of the associated symptoms and hazards.

SCBA should always be worn if it is considered necessary to breach the integrity of the Bunker system and a vapour free atmosphere cannot be guaranteed. This would include the following activities:

          - Open gauging and sampling;

          - Removing blanks for connecting the bunkering hose or loading arm;

          - Cleaning filters;

          - Draining lines to open containment;

          - Sample collection and mixing

# Additional Procedures Very High Concentrations of H2S (>5ppm)

To prevent exposure to high concentrations of hydrogen sulphide, crew members on deck should frequently check H2S concentration, If the concentration is higher than maximum permissible level (>5ppm):

        - Stop bunker operations.

        - Inform other crew members.

        - Inform jetty personnel/bunker barge/Agent + charterer (If Bunker supplied by Charterer).

        - Inform other adjacent tankers (especially those at leeward side).

        - Inform Company office.

        - Ask terminal/bunker barge to perform a measurement.

        - Discuss, in close cooperation with terminal/barge and operator, how to proceed with the transfer operation.

        - As Company Policy Bunkers having more than 5ppm H2S should not be taken, and immediately to inform Company office & stop bunkering if H2S is more than 5ppm. Company will discuss this matter with charterer and advise the vessel.

# Precautions for Visitors

        - Visitors should be warned of possible hazards of presence of H2S.

        - Information about precautions to be taken during their stay onboard.

        - Information for safe areas to be provided.

--x--