# ENGINE ROOM EMERGENCY PROCEDURES

# 4.22.1   Emergency Operation Procedures of Important Machinery / Equipment

# The following emergency procedures for operation of important machinery (ship specific) shall be posted at stated locations in the engine room.  These emergency procedures should be followed in case of emergencies or malfunction of critical equipment.

# All engineering officers and crew are responsible to read these instructions and to be familiar with the operation thereof.

| Item No. | OPERATION OF | LOCATION OF POSTED PROCEDURES |
| --- | --- | --- |
| 1. | Start-up Procedure after Black-out | Engine Control Room |
| 2. | Local Operation of Main Engines | Engine Control Room & Local Starting Console |
| 3. | Emergency Operation of Steering Gear | Steering Gear Command Post, Steering Gear Flat |
| 4. | Emergency operation of Aux. Boiler | Engine Control Room & Boiler Panel |
| 5. | Operation of Emergency Generator | Emergency Generator Room |
| 6. | Operation of Emergency Fire Pump | Emergency Fire Pump Room & Fire Station |
| 7 | Operation of Fixed Fire Fighting System | All Operating Stations |
| 8 | Operation of Aux. Fixed F/F System | All Operating Stations |
| 9. | Operation of Quick Closing Valves | At Operating Station |
| 10. | Remote Shut of E/R Vent Dampers | At Operating Station |
| 11. | Emergency Operation of Elevator | Elevator Machinery Room |
| 12. | Manual Clutch / Declutch of Propeller Clutches | Control Cabinet for each Clutch, E/R Floor |
| 13. | Operation of Bow Thrusters | Bow Thrusters Room / Bridge |

# Fire in Engine Room

# All fires always start small. It is an explosion that is sudden, and can be big; on the other hand, a fire may increase rapidly but always starts small. Hence that is the best time to fight the fire. You can find statistics or reasons for what caused a fire, but it is difficult to get statistics for what did not cause a fire. It is universally accepted that cleanliness and good housekeeping is one of the most important factors that prevents a fire.

# In an engine room some areas are more prone to fire than others. Localized fires inside engine room cannot be generalized, as circumstances will vary with location and machinery affected.  Hence, the approach to fight the fire will be governed by this fact, and for this reason, different procedures for this type of emergency, should be worked out by each vessel, and discussed during fire drills.

# Given below are some of the areas that are considered more fire prone and a short guide to fire fighting in these areas. General precautions such as the use of PPE, Fireman’s suit, SCBA and evacuation have not been mentioned here but should not be overlooked.

# Ship specific procedures should be made on board for such fires in engine room:

# Fire in Purifier Room – This is considered the most fire prone area in the engine room as fuel and heat are both present. Most new ships are fitted with an auxiliary fixed firefighting system for this area. Common systems are ‘Sprinkler, Foam, Micro Fog, or Hyper Mist’. These may be manually operated or automatic. Firefighting procedure for the purifier room would be to shut off the purifier room exhaust fan and close the damper. Switch off power to the distributor panel feeding the purifiers and purifier feed pumps. Close the fuel & lube oil valves leading to the purifiers, from outside the purifier room. First attempt firefighting with portable extinguishers of the foam or dry powder type. Next the Aux. Fixed F/F system. If the fire continues then the fire hose, and finally if nothing helps then the main fixed F/F system.

# Fire in Fuel Pump Space – This is next fire prone area and in many ships the fuel pumps are in the purifier room or purifier space. Procedure for firefighting is similar to purifier room, with the addition that the main engine will have to be stopped as fuel pump will be stopped. As far as possible the generator should be kept running on MDO.

# Fire in Boiler Flat – The boiler burner space is considered a high fire risk area and you may find that this area has a high concentration of portable fire extinguishers, larger semi-portable extinguishers, sand box & scoop, portable foam applicator etc. Procedure for firefighting should be to stop the boiler and FD fan, if necessary, trip it. Stop the fuel oil burning pump and shut valves. First attack with the large number of extinguishers foam and dry powder type, which should contain the fire. Next the aux. F/F system if fitted, followed by fire hose and finally the main fixed F/F system.

# Fire in Auxiliary Engine Area – This is another area where fire is likely. Where possible the generator most far from the fire area should be started, load transferred, and the generator near the fire stopped. If not possible or no time, then trip the main engine and all generators. Start the emergency fire pump after the emergency generator starts. Then as in all above cases fight the fire first with portable extinguishers, then the aux. Fixed F/F system if fitted, next fire hose and finally the main fixed F/F system.

# Fire near Main Engine – The most common cause for fire near main engine is the spraying of high-pressure fuel oil on to the hot exhaust manifold. However, as all high-pressure fuel pipes now have double cover this likelihood has reduced. In case of a fire the main engine and the fuel pump should be stopped, and fuel valves shut. Firefighting as in the above cases should follow the same 4 stages.

# Fire in Exhaust Boiler & Stack – In case of fire in the exhaust boiler, also called an uptake fire the main engine should be stopped, the auxiliary blowers stopped, and turbocharger air filters covered to avoid air intake. The boiler water-circulating pump should be kept running and external boundary cooling of exhaust boiler and uptake done by fire hose. DO NOT carry out soot blowing as under very high temperature the steam can breakdown into oxygen and hydrogen and result in an explosion.

# Fire in M/E Scavenge – Almost all main engines are fitted with a firefighting system for the scavenge space. Generally, this is steam smothering. In case of a fire stop main engine and auxiliary blowers, cover turbocharger air filters to avoid air intake. Release the fire-fighting medium fitted on the engine. Engage turning gear and keep turning the main engine, keep jacket cooling water pump running.

# Fire in Engine Room Workshop – The main cause for fire in the engine room workshop is poor housekeeping and not enough precautions during hot work. Especially hot work not involving arc or flame is often underestimated. Another major cause is electric heating elements or incandescent lamps left on with flammable material above it. Fire-fighting procedure would be to switch off power to equipment and use foam or dry powder portable extinguishers. If this is not successful, then fire hoses should be sufficient.

# Fire near Incinerator – This is another area where fires are likely. Fire-fighting procedure would be shutting off power and fuel to the incinerator and using foam or dry powder portable extinguishers. Some ships have aux. Fixed F/F system here and this is to be used next, followed by fire hoses.

# Fire in the Bilges – Poor housekeeping and falling hot particles from some hot work being carried out above mainly cause this fire. For firefighting, if the burning material is clearly visible then portable foam or dry powder extinguishers can be used. If not, then fire hoses are most effective.

# Fire in Steering Flat – This is generally a very low fire risk area if proper housekeeping is done. In the unlikely event of a fire, switch off ventilation and power to equipment. First use foam or dry powder portable extinguishers followed by fire hoses.

# Fire in Emergency Generator Room – This is also a low-risk area, and in the event of a fire on the generator engine, the engine should be stopped, the fuel tank outlet valve closed, and the fire fought with foam or dry powder portable extinguishers. In case the fire is on the emergency switchboard then portable CO2 extinguishers should be used.

# Fire on Main Switchboard – Fire on main switchboard is unlikely, but in case of a fire the part on fire should be isolated if possible and portable CO2 extinguishers used. NEVER USE FIRE HOSES WITH THE POWER ON. In case portable CO2 extinguishers do not control the fire, then trip and lock shut all running and non-running generators before using any other means or the fixed F/F system. Please be aware that non-running generators in stand-by position may suddenly start without warning.

# As per SOLAS amendment, external high-pressure fuel delivery lines between the high-pressure fuel pumps and fuel injectors shall be protected with a jacketed piping system capable of containing fuel from a high-pressure line failure. This shall be verified by Chief Engineer and be complied with.

# During drills, training sessions and during informal discussions, all staff should be encouraged to see the exact location of the electric breakers, valves, and extinguishers to be used in all the various cases above.

# Implementation of Blackout Recovery Test

General: The Blackout recovery test is to be conducted only after approval from the office and once a thorough RA is conducted and approved by the office.

- Purpose

  - To become familiar with the operation needed to recover from a blackout

  - To detect machinery malfunction

  - To detect deterioration of electronic components

- How to Cause a Blackout

The running DG is tripped by LO low pressure. (The process for initiating the blackout shall be confirmed and discussed with the PIC/GM during the initial approval stage.

- Confirmation

  - Whether the test proved satisfactory or not

  - Time from blackout until full recovery

  - Time from blackout until the main engine becomes operative

  - In the event of the discovery of defects, their contents and countermeasures, Any trouble however small it may be  or any defects experienced during the test must be recorded with the corrective action taken.

  - Any other special comments and observations about the blackout test

- Preparation

  - One day before the blackout test

    - Carry out the trip test for the D/G, which will be tripped during the blackout recovery test, and confirm the good working condition of the trip.

    - Hold pre-meeting for the blackout recovery test.

    - Check general and radio equipment batteries and battery lights.

  - On the day of test

- Carry out test running of ST-BY D/G.

- Carry out test running of emergency generator.

- Stop all purifiers.

- Stop FWG.

- Change fuel oil of running D/G from FO to DO.

- Stop elevator at second deck and inform crew by announcement on PA system. (If applicable)

- Shut down all the computers on board.

- Shut down No. 2 ECDIS and No. 2 radar. Shut down one of the ECDIS and S-band radar.

- Stop accommodation air conditioner.

- Schedule

  - Charge air to both air reservoirs.

  - Reduce M/E revolution and set to “Dead slow ahead.”

  - Ensure that all standby generators are on Auto.

  - Running D/G is tripped by LO low pressure.

  - Blackout is carried out by tripping the running D/G on LO low pressure (start measuring time).

  - Confirm that emergency lights have been switched on.

  - Confirm that first and second standby D/G and emergency generator starts automatically.

  - Confirm that ACB of first ST-BY D/G closes automatically and vessel recovers from blackout.

  - Confirm that auxiliary machines start automatically on sequential system.

  - Reset and restart aux. boiler manually.

  - Reset and restart M/E. (Set the M/E handle to stop position in the telegraph.)

  - Stop emergency generator.

  - Restart machines/equipment that do not start automatically.

  - Refer to the sequential start list and manual start list prepared for vessel specific.

- Guidance on Stations and Roles

  - C/E - ECR

  - 1/E	 - ECR

  - 2/E	 - E/R: 	No.1 D/G to be tripped, roving in E/R

  - 3/E	 - E/G room	After recovery from blackout, proceed to boiler side

  - FTR - 2nd Deck	Roam about E/R

  - O/A - 3rd Deck	Assist 2/E, roam about E/R

  - O/C - Lower floor	Roam about E/R

  - O/B - Boiler side	Assist 3/E, roam about E/R

  - Officer on watch (OOW)	W/H- Check navigation equipment

- Battery light

  - There are battery lights in the ship as follows:

- DC24V battery light for GMDSS by radio use battery (GMDSS Communication Space and VHF radiotelephone)

- DC24V battery light by general use battery (Daylight signalling light and Behind of the main switchboard in ECR)

-x-