| Alarm description | Testing |  |  |  | Measures in case of alarm / trip activation |
| --- | --- | --- | --- | --- | --- |
|  | Testing interval | General guidance for testing | Specific guidance for testing | Test record guidance |  |
| Lube Oil Low Pressure Alarm | 3 months | Alarm set point to be confirmed to be in accordance with the engine maker’s manual / requirements. Procedure recommended in maker’s manual should be followed for testing. | Should be tested using a calibrator (pressure / temperature calibrator), and the actual tripping point (pressure/ temperature) noted. | Alarm test record should include: - Set point - Activation point - Remarks / condition In case any abnormalities or defects are noted with the alarms: - Record in the technical defect list for soonest rectification. | In case of any alarm / trip: Cause of the alarm activation should be investigated into, and rectified, before Auxiliary Engine is attempted to be restarted. Office / Vessel Manager should be informed in case of any trips, or in case the alarm condition is unable to be rectified. Specific measures for troubleshooting, as advised in the maker’s manual should be followed, upon alarm activation. In case of Overspeed trip or OMD alarm: |
| Lube Oil Low Low- Pressure Trip |  |  |  |  |  |
| Lube Oil Temperature High |  |  |  |  |  |
| JCW Outlet Temperature High |  |  |  |  |  |
| JCW Pressure Low |  |  |  |  |  |

| Alarm description | Testing |  |  |  | Measures in case of alarm / trip activation |
| --- | --- | --- | --- | --- | --- |
|  | Testing interval | General guidance for testing | Specific guidance for testing | Test record guidance |  |
| Overspeed Trip |  |  | Should be tested by carefully increasing the speed, and the actual tripping speed noted. | - Carry out a risk assessment if operation is essential with the alarm / trip defective / not working. - Seek approval from office / Vessel Manager, before operation. | Crankcase should be inspected after taking all safety precautions. In case of Overspeed trip: Check if any dislocation / damage of cylinder head valve rocker arms / push rods has occurred. It may be required to reduce the speed setting before starting the engine again for checking, and gradually increasing the speed setting (Refer maker’s manual before restarting). |
| Oil Mist Detector (OMD) High Mist |  |  | Should be tested / simulated in accordance with the procedure in maker’s manual. |  |  |

Caution:

Following actions should NOT be carried out. If inevitable, Office / Vessel Manager(TSI) should be consulted, and a risk assessment carried out for enforcement of appropriate control measures:

- Reposing / inhibition / bypassing of alarms and / or trips

- Changing the set points of the alarms and / or trips