## APPLICATION

This document applies to all engineers on board.

## PURPOSE

To specify requirements for operation of engine room systems in a safe and efficient manner.

## RESPONSIBILITIES

  - Chief engineer

    - Shall ensure that Engineer on Watch (EOW) is familiar with the requirements for operations of Engine Room Systems.

## EOW

    - Shall ensure that operations in Engine Room are carried out in accordance.

## MAIN ENGINE

  - Indicator diagrams

Take Indicator diagrams or peak pressure cards on all motor ships:

    - Monthly (Preferably on loaded passage with Main Engine running at 85% load)

    - Immediately after leaving dry-dock

    - Whenever an engine defect is suspected

Note:

The most frequent causes of fractured cylinder heads are sudden temperature changes and extreme temperature differences between combustion and cooling water spaces.

Cylinder heads, liners and pistons

Take following steps on main engine to prevent cracking,

    - Keep engine warm during normal port calls

    - Warm up engine to maker’s recommended temperature, before starting

    - Inspect cooling water passages periodically at frequency specified by engine maker

    - Circulate the piston cooling oil for at least an hour

    - Keep Jacket water chemical protection at the recommended levels. Record periodic checks in accordance with manufacturers recommendations

    - Turn the engine by turning gear or slow turning with indicator cocks open after:

      - a prolonged port stay

      - maintenance

    - Turn the engine on air, with indicator cocks open, prior to starting after a port stay, anchor stay or a long idle period of over 30 minutes

Caution:

Through periodic inspections and cleaning, ensure that cooling water passages are clear and not obstructed by scale or by graphite accumulation

Stagnant oil in contact with hot surfaces will break down to form carbon deposits.

If water should enter a cylinder while the main engine is running, the exhaust temperature for that cylinder will show a reduction.

Monitor the exhaust temperatures carefully. Investigate the cause of any sudden reduction in temperature immediately

## Valves and valve gear

    - Ensure that exhaust valve spring air pressure and condition is maintained in accordance

with maker’s recommendations

    - Check the exhaust valves in operation by engaging indicator mechanism. Disengage once check is complete

    - Check following for leaks during operation:

      - Fuel valves

      - Starting air valves

      - Relief valves

      - Indicator valves

## Fuel oil and injection system

    - Before feeding to the engines, follow vessel’s specific fuel oil treatment procedures thoroughly for:

      - Dosing of chemical additives

      - Settling of fuel

      - Purification

      - Filtration

    - Both Fuel Oil Service Tanks should not be used simultaneously. One tank should be used at a time (lined up with the service system), while the other one is filled up and oil allowed to settle for water separation and draining, before the tank is taken in use.

    - Only one Fuel Oil Settling Tank should be lined up for use (for purification) at a time, while the other is filled up and oil allowed to settle for water separation and draining, before the tank is taken in use.

Vessels equipped with viscometer must always use viscosity controller for heated fuel and ensure that at engine inlet, fuel viscosity is maintained as 12~14cSt, or as recommended by the engine makers. The vessels not equipped with viscometer (or when this is not functional) must control viscosity by controlling the fuel injection temperature appropriately. The temperature for fuel injection should be maintained corresponding to

Note:

The Engine maker's recommended fuel injection viscosity range is shown in the Engine Operation Manual.

The recommended Fuel oil temperature for achieving the desired Viscosity can be derived from the temperature-viscosity chart.

Caution:

Operating with too low a fuel temperature and a correspondingly higher viscosity will cause late ignition with consequent after burning and carbo deposits on combustion spaces, exhaust valves and turbocharger nozzle rings.

that for viscosity of 12~14 cSt, taking reference from the fuel oil analysis report for the fuel in use. Considering that there will be certain temperature drop along the pipeline, care should be taken to maintain this temperature at the engine inlet, rather than at the fuel heater. Temperature setting must be reviewed whenever a new bunker fuel is taken in use.

## Lubrication system

    - Maintain the pressure and temperature of the lube oil supply within the builders' specification

    - Operate the purifier on a routine basis to maintain Lubricating oil and system in clean condition

    - For optimal Purification of Lubricating oil, always ensure:

      - LO feed temperature at maker’s recommended level

      - Throughput is around 25% of purifier rated capacity

      - Gravity disc size is as recommended in Nomogram

      - Desludging interval is adjusted as per experience and observation of purifier condition

## Charge air systems (Turbochargers, air coolers and scavenge spaces):

Closely monitor and record below:

    - Exhaust gas inlet and outlet temperatures to turbochargers

    - Scavenge air inlet and outlet temperature to air cooler

    - Scavenge air inlet and outlet differential pressure across air intake filter

    - Scavenge air inlet and outlet differential pressure across air cooler

    - Turbocharger RPM

    - Turbocharger lubricating oil flow

Caution:

Cooling water supply to the air cooler must be regulated to maintain the air temperature just above dew point.

Under Piston Space temperatures

## Crankcase

    - Monitor crankcase during operation by feeling over the doors or by using infra-red thermometers

    - Observe crankcase for any abnormal sound during operation

    - Monitor the stuffing box drain oil daily collection rate on monthly basis and compare with previous month collection rate

Caution:

EOW shall enforce use of intrinsically safe lights and the adherence to no smoking regulations, in the vicinity of an open crankcase.

Inspect the crankcase and running gear at monthly intervals

## Precautions for overheating of bearings

    - Monitor lube oil pressure and flow through pressure gauges and sight glasses

    - Monitor bearing temperatures during operation in accordance with maker’s instructions

    - Monitor oil mist detector during operation

Caution:

Overheated bearings can lead to an oil mist explosion in the crankcase.

Early detection of overheating and the prompt slowing or stopping of the main engine help prevent the occurrence of conditions conducive to fire or explosion.

Warning:

Do not open the crankcase or inspection doors until the main engine has cooled down in case of detection of oil mist. Do not restart the main engine until the cause of overheating has been established and rectified.

Test oil mist detector as per planned maintenance system

## Main engine alarm management

    - In case alarms are encountered on main engine crank case oil mist detector, main engine bearing monitoring system, or main engine lube oil system, or in case of alarms leading to slow down or shut down of main engine, following measures should be taken:

      - Wheelhouse should be informed immediately, and engine should be stopped at earliest safe opportunity.

      - Appropriate measures should be taken to identify the cause.

      - Crank case inspection and lube oil filter inspection should be carried out to ensure no abnormalities exist.

      - Office should be informed about the situation.

Caution:

Engine should not be started until the reason for the oil mist detector or bearing monitor alarm is clarified and abnormal condition rectified.

The actions as specified in the maker’s instruction manual, should be followed.

## Starting Air System

    - All drain points in the starting air system (main air compressor, starting air reservoir, starting airline, control air dryers etc.) to be identified and marked.

    - All drain points in the starting air system should be regularly drained every watch with careful observation, to ensure that all water content is removed.

## MAIN PROPULSION – STEAM TURBINE

  - Warming and shutdown procedures

Carry out the warming of steam turbines in accordance with the maker's instructions.

Operate the gland steam system in accordance with the manufacturers' recommendations to minimize gear case humidity.

Note:

The propeller rotation required is negligible, and one or two rev/min will ensure satisfactory rotation of the turbine.

Caution:

Appreciable distortion can occur in both the rotors and casings during this process, and also during cooling down periods, if the rotor is allowed to remain stationary for any length of time.

Warning:

The turbine rotors shall never be allowed to remain stationary for longer than five minutes, while steam is supplied to the glands, to prevent local heating stresses occurring in the area of the gland pockets of the stationary rotor.

The propeller shaft must not be turned until it is established by the bridge that the propeller is clear.

## Maximum power underway

    - Make full power available as soon as steady conditions permit, without overloading

    - Adjust the plant to provide the maximum efficiency under prevailing conditions

Note:

A comparison of the temperature and pressures obtained with those specified in the Heat Balance Diagram will give an indication of efficiency.

Keep temperatures and pressures as close as possible to the system specifications

## Reduction gearing

Regularly inspect the gears and flexible couplings for wear or deterioration.

Caution:

Exercise care to ensure that nothing falls into the gear case when inspection covers are removed.

Warning:

EOW shall enforce the use of intrinsically safe lights and no smoking regulations in the vicinity of an open gear case.

## Lubrication system

    - Arrange adequate supply of clean lubricating oil. Record Lubricating oil usage daily

    - Run the purifier on a regular basis for at least 7 days in every 10, to remove any water or sediment

    - Positively use lube oil filters

    - Check gravity tank alarms and emergency main turbine trip valves at monthly intervals

Caution:

If oil starvation has occurred after a lube oil failure, an examination of the bearings and gearing shall be carried out before returning the turbine to service.

Where a lube oil gravity tank is fitted, Keep the run-down valve locked open at all times when the main turbines are turning

## Safety devices

Warning:

This is extremely important and must not under any circumstances be neglected.

Note:

The over-speed governor cannot normally be tested whilst the ship is in service and an item covering the examination and testing shall be submitted at repair periods / dry-dock.

Check the operation of all governors and trip devices at intervals not exceeding one month.

## WATER TUBE BOILERS

  - Feed water system and chemical treatment

EOW must:

    - Use distilled water only

    - Carry out chemical tests daily and record the results

    - Dose the required chemicals on basis of test results and in accordance with

manufacturers’ recommendations and suppliers’ instructions

    - Confirm that boiler water gauge glasses are blown down daily, and fully operational

Note:

If de-mineralization columns are also used, the outlet conductivity must not exceed normal limits or regeneration of the de-mineralisation column may be required.

Test Low water level alarms and trips monthly and report in the PMS

    - Maintain full heating steam to the de-aerator under all conditions of plant operation

    - Remove residual traces of oxygen by hydrazine or sodium sulphite treatment

    - Monitor condensate salinity by means of the salinometer

    - Test the operation of salinometer every week

    - Blow down Boilers to remove sludge accumulations:

      - Weekly

      - When the total dissolved solids concentration limit is exceeded

    - The Chief Engineer must:

      - Ensure that Engine department personnel are chemical testing the boiler water and treating the feed water as per maker’s recommendations

      - Personally test the boiler water once a month and compare with readings recorded by responsible engineer

      - Carry a stock of chemicals sufficient to degrease each boiler twice in case of oil contamination of boilers

      - Tabulate results of chemical testing on the record sheet for monthly reporting to chemicals supplier and head office

    - It is preferable to blow down under light steam load conditions

    - A single blow, using the drum blow down valves are normally sufficient

    - Where water-wall header drain valves are fitted, the system must only be blown down on light load

    - If blowing down in port, exercise care if the ship's side valve is above water and craft are alongside. Open the valves for a very short period only

    - Remote level indicators must be considered to be indicators only and their readings must be frequently checked against the directly mounted boiler gauge glasses

    - In the event of water falling or rising below the visible range of the water gauge, the following course of action must be taken:

      - Extinguish the burners immediately

      - Close the feed check valve

      - Close the steam stop valves

      - If boiler has super heater, maintain steam flow through the super heater by opening the vents

      - Stop the forced draught supply after a full purge

      - Add water only when the boiler has cooled sufficiently to prevent water coming in contact with overheated pressure parts

## Combustion system and operation

Ensure complete burning of fuel with the minimum amount of air by:

Note:

Check the temperature at the fuel delivery rail to the boilers and not at the heaters, as considerable heat loss can take place between these two points.

Injecting the fuel into the furnace in a finely atomised state. Obtain the required viscosity for correct atomizing from a temperature viscosity chart

    - Regulating air quantity and mixture by correct adjustment of the fan dampers

Caution:

For ships which are provided with continuous excess oxygen analysers, the O2 readings will be affected if there is any forced draught air leakage through casings.

Note:

The Engineer of the Watch (EOW) must be present before the boilers are lit up and when raising steam,

No engineer of a rank more junior than that of fourth engineer may be left in charge.

Carry out following checks before lighting up:

    - That the gauge glass cocks are in the correct position

    - That the water level is correct i.e. there is a level showing at about 50mm from the bottom of the glass

    - That the super heater vents are fully open to allow steam to circulate through the super heater elements and that they remain open until the boiler is in service (If boiler has super heater)

    - That the super heater drains are open to remove any water that may have lodged there and which would evaporate leaving solids (If boiler has super heater)

    - That the economiser tubes are full

    - That there is no accumulation of un-burnt fuel on the furnace floor

    - Complete purging sequence if the vessel is fitted with automatic purging

    - On other vessels, blow through the furnace for at least two minutes before any attempt is made to flash the boiler

Caution:

The use of limiting devices such as gags or wedges which restrict the complete closing of fuel valves is strictly prohibited.

Where possible, raise steam using the air heater in order to even out warming-through

    - Observe the safe practice of adjusting the master controller set point to reduce the boiler working pressure under port and manoeuvring conditions

Warning:

Care must be exercised when lighting a boiler fire by hand. The furnace must be purged through with air beforehand for at least 3 minutes on maximum draught. Should fuel be admitted before the insertion of the igniters or torch, it may vaporise in contact with hot surfaces and when ignited, result in a blow-back and serious injury.

Note:

Under exceptional circumstances, boiler fires and naked lights in the engine room may be prohibited.

Master must always inform the Chief Engineer of the precautions necessary when these circumstances prevail.

Maintain the uptake temperature at least 173°C where gas/air pre-heaters or economiser are fitted

## Pressure vessel relief devices

The EOW shall:

    - Test and confirm that all relief devices fitted to any pressure vessel operate correctly

    - Check and confirm that relief valve waste pipes are clear and unrestricted

The Chief Engineer shall:

Prepare a routine inspection programme to ensure that waste pipes and drains of all pressure relief devices are correctly fitted and unobstructed.

## Testing

    - EOW shall test all low level trips, both level switch and transmitter actuated, every month

    - The operation of the fuel valves must be checked at the same time

    - The test detail must be entered in the Engine Log

    - When navigating in close waters, Fire the auxiliary boiler for one-minute intervals to test the fuel oil and combustion system once per day

## Boiler lay up

Note:

Adopt the wet lay-up method for very short lay ups only.

Head office shall advise the method of lay up to be adopted.

Instructions for wet lay up:

    - Completely fill water drum with distilled water which has been chemically treated to bring the alkalinity up to a maximum of 300 ppm

    - Allow all air to escape through the super heater vent

    - Raise pressure of 3.5 bar hydraulically. Maintain until the boiler is at engine room temperature

    - Test the super heater vents to ensure that no air remains

    - The EOW shall arrange for testing to check that there are no leaks

## Soot blowing at sea

Boilers soot blow should be carried out at following frequency and entries made into engine log book:

    - Every day

    - Depending on actual condition of boiler when firing on fuel oil

    - Every third day when firing on gas only

Note:

The danger arising from particles of incandescent soot being discharged from the funnel can be minimised by operation of the soot blowers on a routine basis at sea and also by operating the soot blowers as close before entering port as permissible.

Before entering port, regardless if fired on fuel or gas

Caution:

Prior doing soot blowing Duty Engineer must inform Duty officer on bridge and check with bridge if it is safe to carry out soot blow.

Duty officer must ensure:

    - No combustible material present on deck which may catch fire due to sparks from funnel.

    - Wind direction must be athwart the ship and should not be in direction such that it blows the soot or sparks to hazardous area on ship.

    - Lookout must be kept while soot blowing to ensure sparks do not lead to a situation which can cause fire.

## Soot blowing into port

Do not carry out soot blowing within port limits.

## Boilers operations in maneuvering

Operate the vessel main propulsion as follows:

    - Master boiler

      - To be operated in fuel only mode

    - Slave boiler

      - To be operated in dual fuel mode

## ELECTRICAL SYSTEMS

## Safety precautions

For maintenance:

    - Consider all electrical conductors installed in electrical equipment on board to be live until they are isolated from all power sources

    - Use a test meter, suitable for the normal rated voltage to prove that the circuit is dead after isolation

    - Test the meters on a live circuit to prove the correct operation immediately before and after being used to prove an isolated circuit

    - Do not apply any tests to live main switchboard bus bars

    - Use an electrical insulating non-slip mat suitable for the rated voltage as floor covering where it is necessary to work in the vicinity of live conductors

    - Use protective gear including:

      - Rubber soled footwear

      - Electrician's rubber gloves

      - Insulated tools

      - Dry clothing

    - Isolate the load before reinserting fuse links into a circuit For operation:

    - Maintain electrical apparatus well in order that it offers maximum protection against shock to the operator.

    - Use control knobs or handles of insulated design for 440 volt supplies to ensure safety to personnel

    - Ensure that portable electrical equipment with a metallic outer case has a secure earth connection

    - Do not use equipment with worn or damaged flexible cables

Caution:

Insulating non-slip mat as floor covering is mandatory on switchboards.

Any voltage shall be regarded as capable of producing major injury or loss of life by electric shock.

The Ship Captain's Medical Guide, Chapter 1 shall be referred to for treatment of victims of electric shock.

Feed portable equipment from outlets supplied by isolating transformers when such supplies are available on board

## Operation

    - Follow Instructions for the particular equipment on board any vessel

    - Test Emergency diesel generators to confirm:

      - It can be started by two independent starting methods.

      - The generator can provide power simultaneously, for the following services (besides any other safety and emergency equipment meant to be run by emergency power sources):

        - Emergency lighting

        - Navigation lights

        - Daylight signalling lamp

        - Emergency Fire Pump

        - Emergency Steering

        - Emergency Air Compressor

      - It can start automatically and connect to the emergency switchboard within 45 seconds of power failure.

Note:

Following instruction should be posted on board at relevant locations (including the Emergency Generator Room).

The emergency generator should not be left unattended when it is running. A continuous watch should be maintained, on the performance parameters and the condition. Any deficiencies should be rectified by stopping the generator, at the earliest safest opportunity.

While running the emergency generator a continuous watch should be maintained, for closely monitoring the performance parameters and the general condition (vibrations and sound). Any discrepancies identified should be rectified soonest (as soon as it is safely possible) by stopping the generator.

    - Test all diesel-alternator sets monthly on 85% load

    - Keep batteries for starting purposes fully charged

    - Keep air receivers and fuel tanks full

    - Change over alternator sets at intervals not exceeding 1 month in order to maintain the insulation, governors and bearings in good condition

    - On-board steam ships which are fitted with one main turbo-alternator and one diesel- alternator of the same capacity:

      - Switch on the alternator heaters in the diesel-alternator and also the emergency diesel generator. When these machines are not in use

    - Turn these machines weekly

    - In order to ensure that the sets are capable of satisfactory load sharing, leave them in parallel for approximately four hours on each change over. Consider inability to operate satisfactorily in parallel as an operational defect and Head Office must be notified

    - When on standby conditions, keep main alternator running at operational speed, synchronised with the first alternator and supplying a share of the load

    - Keep diesel-alternators equally loaded

    - Do not allow to run unloaded for more than a few minutes

    - Change over all standby equipment including motors, feeder cables, steering supplies, etc. with running equipment at regular intervals

    - Switch on heaters, where fitted in standby electric motors

## Battery systems

    - Adequately ventilate battery rooms to prevent build-up of hydrogen gas

    - Where automatic equipment is installed to supply emergency lighting circuits on failure of power to the switchboards, Test these circuits weekly

## Hazardous area installations

Caution:

Failure to observe these requirements will destroy the effectiveness of the enclosure and allow the external propagation of flame resulting from an internal ignition.

The barrier units are sealed and must not be opened. Testing must be confined to replacing a suspect unit with an identical spare. No other maintenance action may be permitted.

Warning:

Test meters must not be connected to the wiring on the hazardous side of the barrier.

Secure enclosure covers as specified by the manufacturers

    - The electrician shall:

      - Inspect all explosion proof electrical fittings on the cargo deck of each tank ship

      - Remove or clean any silicone sealant applied

      - Replace only the original O-ring gasket so that the fittings are installed as designed

      - Include prohibition to the use of silicone on the cargo deck in the hand-over notes

## Dry-docking precautions

    - Keep Heaters, where fitted to motors, alternators, switchboards, etc. switched on throughout the dry-docking period

    - Check the actual operation of the heaters during the first day of the dry-docking period

Note:

Heating lamps can be fitted to all motors not fitted with heaters, at the superintendent's discretion.

Carry out regular insulation tests on motors to ensure that a safe level is maintained, before any motors are started after the refit period

## Earth faults and insulation testing

Note:

If an earth fault is traced to a motor circuit the possibility of the fault existing in the motor control gear is likely.

The Electrician shall locate and clear an earth fault as soon as indication has been given by the lamps

    - The more modern ships are provided with earth fault monitoring relays. Test the relays as recommended by maker

    - Test all motors not fitted with operational heaters prior to starting after a prolonged shut- down period

Caution:

High voltage insulation testers must not be used on alternator rotor or exciter windings which are rated for low voltage operation.

Rectifiers and other semiconductor devices are also incorporated in this equipment and the application of a high reverse voltage would cause serious damage to these devices.

High voltage insulation testers must not be used on any part of electronic control circuits.

Use 500 Volt high voltage insulation testers of the Megger type or similar to test equipment connected to medium and high voltage

## High voltage 6.6KV

    - Draw out breakers

    - Connect earthing strips/wires as close as possible to work space area

    - Isolate work area by tapes or plastic chains

    - Complete Form HSSEQ18 “Hazard Analysis”

## Microwave ovens - Inspection and testing

    - Inspect and test Microwave ovens at monthly intervals

## Alarms

    - No alarms for any equipment should be reposed. In case of a condition that may require such an action, office to be notified and appropriate mitigating / control measures to be implemented, based on thorough risk assessment.

    - No alarm for any equipment or system on board should be ignored.

    - All alarms should be treated and attended with due diligence.

## ROUTINE TESTING – MISCELLANEOUS EQUIPMENT

The following shall be tested at the specified intervals in planned maintenance system and results recorded:

- Boiler(s)

  - Check boiler water daily

  - Prepare for unannounced random testing by supplier of the boiler water treatment chemical

- Cooling water

  - Test cooling Water for main and auxiliary machinery twice a week

- Lube oil

  - Land Lube oil samples for all relevant systems on-board quarterly for analysis by the supplier

Note:

Test kits for on-board testing do not remove the requirement for laboratory analyses.

Inform head office by email when samples are landed, naming recipient and to whom samples were sent

- Fuel oil

  - Collect and forward the prescribed samples immediately upon bunkering as laid out in the program instruction

  - Inform head office by email when samples are landed, naming recipient and to whom samples were sent

- Pressure testing of pipes

- Test and record annually to 100% of their Maximum Allowable Working Pressure (MAWP) of the pumps in use. :

  - Bunkering/fuel Oil Pipes

  - Cargo pipes,

  - cow pipes,

  - VEC pipes

  - Ballast pipes

- Testing date and pressure to be marked on the pipes on deck and in the pump-rooms

Caution:

Testing of Piping Systems is to be carried out with liquid only. The practice of using air or inert gas to pressurize pipelines is to be discontinued as same is not acceptable.

The system is to be tested by a Static liquid pressure test is to be carried out at least 1.5 times their rated working pressure (MAWP) at least twice within any five year period of the pumps in use

- Megger test of electrical equipment

  - Carry out megger testing on a continuous basis and forward results to head office on E58 Form 54 every six months.

- CO2 fixed fire extinguishing system

  - Blow through pipes by air every twelve (12) months

  - Request for general service of the system every 2 years or as required by specific flag state

- Inert gas system

  - Test inert Gas System as per the Maintenance Program and/or in accordance with the Inert Gas operations procedures. See OTM 7.10 Maintenance and Testing.

9	INSPECTION OF ANTI-SPLASHING TAPE OF ALL FUEL/LUBE OIL PRESSURIZED FLANGES/COUPLINGS

## General

Reference:

    - IMO MSC Circ. 647, IMO MSC.1/Circ.1321 Coverage areas:

    - Flange connections and mechanical joints (threaded pipe joints, compression joints etc.) Exemption eligible areas:

    - Piping of FO located below lowest engine room floor

    - Piping with low design pressure (e.g. overflow line, drain line etc.)

    - Piping that contain no oil under normal conditions and are used occasionally and intentionally for short times (e.g. FO filling line)

Instruction:

    - Fit spray shields around:

      - Flanged joints

      - Flanged bonnets

      - Any other flanged or threaded connections of fuel and lubricating oil systems

      - Which have the possibility of being in contact with potential ignition sources by direct spray or by reflection

    - Spray shield types:

      - Anti-splash tapes

      - metal flange cover

      - Effective insulation in way of connections or joints of fuel pipes

      - Any other means approved by classification society

## Examples

    - Thermal insulation having sufficient thickness.

Caution:

should be taken to avoid using the anti-splashing tape in areas of high temperature so as to maintain its adhesive characteristics. In case of rewrapping of the new tape, the surface area of the tape should be clean and dry.

Anti-splashing tape made of approved materials.

Recommended arrangement of anti-spray cover for flange joint

## Threaded connections and pressurized systems

    - Treat following as spray shield on threaded connections:

      - Anti-splashing tape

      - Other equivalent method

    - Apply spray shields to:

      - Heat exchanger

      - Filter or strainer body joints

      - Pressurized equipment and/or fittings on oil fuel systems

## Inspection and maintenance

    - Inspect spray shields quarterly for their integrity

    - Refit any shield which have been removed for maintenance purposes on completion of the task

    - Replace any peeled off ends, loss of adhesion, worn or torn tapes with new anti-splash tapes

    - Renew any oil soaked anti-splash tapes or insulation laggings

## HEAT EXCHANGERS

## Cleaning and testing of heat exchangers

    - Perform cleaning, inspection and testing of heat exchangers

    - Manufacturer’s maintenance recommendations shall be followed

## Methods of marine heat exchanger testing

    - General

    - Test heat exchangers for leakages at regular intervals as defined by maker of the equipment.

Note:

Low level alarms are also useful in detecting major leaks.

Mixing of the two liquids in the heat exchanger shall be prevented. One common method is to keep the sea water at a pressure lower than the jacket water or any other liquid used. This reduces the risk of sea water entering into other mediums.

# Shell and tube type heat exchangers

    - Isolate the heat exchanger from the system

    - Drain the sea water

    - Remove the end covers or headers to expose the tubes or plates

    - If the surface is clean and dry, inspect the liquid flow from around the tube ends and through the perforations

    - In large coolers add fluorescent dye to the shell side of the cooler

    - Show ultraviolet light on the tube, the dye glows, revealing the tube leaks

# Plate type heat exchangers

    - Use visual inspection or fluorescent dye penetrate to find any defeats. (Dye penetrant on one side, followed by ultraviolet rays on the other side)

# Air coolers

    - Detect location of the leaks by allowing low air pressure on the air side and checking the flooded sea water side for air bubbles

Caution:

Leakages in air coolers can be dangerous as they allow sea water to pass through to the engine

cylinder. This can lead to formation of scales on air inlet valves’ spindle.

For better results, use soapy water for sea water side flooding

## Pressure testing of heat exchangers

    - General

    - Carry out pressure testing of heat exchangers:

      - After major repairs

      - After exchange of parts

Caution:

Care needs to exercise so that working pressure is different on both sides and hence correct pressure should be applied in order to avoid damage to the heat exchanger.

Test pressure shall be 1.5 times the working pressure of heat exchanger or as specified by maker

# Shell and tube type/plate type heat exchangers & air coolers

    - Assemble heat exchanger with shell and end plates clamped together

    - Remove and blank off all connections except one opening to connect hydraulic pump

    - Fill up heat exchanger shell with testing medium

    - Connect hydraulic pump with pressure gauge to heat exchanger shell

    - Increase the pressure up to test pressure

    - Hold the pressure for about 8 hours and check for leaks

    - Then depressurise the heat exchanger and empty out the contents

## AUXILIARY ENGINE

## Lubricating oil purification

    - Auxiliary Engine lube oil should be circulated through the purifiers regularly, in the most effective way, depending on the purification pipeline configuration.

      - Where the purification system allows that the lube oil can be safely circulated through the purifier, without running the risk that the leakage in the purification system can hazardously impact the engine sump level, purification should be carried out while the engine is running.

      - In other cases, where the pipeline arrangement doesn’t allow a safe auxiliary engine operation with the lube oil being circulated through the purifier, the purification should be carried out when the engine is in idle / stand by state, for adequate durations.

      - Inline centrifugal filters (example, Glacier filters) are recommended to be installed in order that the lube oil gets purified when the auxiliary engine is running.

    - PMS should cover appropriate maintenance routines for the auxiliary engine lube oil purifiers and the inline centrifugal filters (example, Glacier filters).

## Auxiliary engines critical alarms

    - Auxiliary engine critical alarm testing to be carried out every 3 months.

    - See FOM 4.4 Auxiliary engine critical alarm guidelines.

## Precautions if auxiliary engine has been idle for prolong period

If a generator has not been run for a prolonged period (15 days), following measures should be taken, before taking the engine on load or keeping it on standby:

    - The engine should be first run without load for minimum 10 to 15 minutes and parameters should be confirmed to be normal.

    - The governor speed setting should be first checked using the manual speed setting (frequency adjustment switches) on the main switchboard, and the governor speed setting motor ascertained to be responding well, before taking the engine on load. In case any problems with the remote control are noted, the local governor speed setting knob should be used for actuating the speed setting movement. Subsequently, the motor response should be confirmed to be restored.

## REFERENCED FORMS AND CHECKLISTS

HSSEQ-18 - Hazard analysis E58 - Megger test

## APPENDIX

FOM 4.4 Auxiliary engine critical alarms guidelines