APPLICATION

This document is applicable to Senior Officers on board fully managed vessels.

PURPOSE

The purpose of this document is to specify requirements for readiness of machinery.

# RESPONSIBILITIES

# Chief Engineer shall ensure that:

    - All equipment on board is available for normal operations, unless being maintained or undergoing repairs.

    - Any equipment defects rendering the equipment inoperative are informed to Master and Vessel Manager, and entered in technical defect list.

# Master shall ensure that:

    - Risk assessment is carried out for any unavailable equipment.

# PRECAUTIONS AND PREPARATION FOR MAIN ENGINE MAINTENANCE

# General

    - Never immobilise the main engines without the consent of the Master.

    - Ensure approval from port authorities is obtained prior immobilising.

    - Chief Engineer to advise the Master about expected duration of immobilisation and the time required to make the engines available in case of an emergency.

    - Carry out a risk assessment and obtain concurrence from Vessel Manager.

Caution:

Main engine maintenance must be carried out as per critical equipment maintenance. Use E30  Critical equipment maintenance permit and HSSEQ-18 Risk assessment. See FOM 10.17 Critical equipment for more information.

    - Inhibit the engine from starting

    - Inform Master and Chief Engineer before carrying out any maintenance on main engine control systems.

    - Keep main engine turning gear engaged for the entire period that cargo hoses or loading arms are connected to ships manifold or, when cargo loaders / unloaders are working in the case of bulk carriers.

    - Keep turning gear engaged when the bridge is unmanned.

    - Adopt procedures to prevent inadvertent starting of the main engine when the turning gear is disengaged.

Caution:

Chief Engineer to ensure that all engineers are aware of operation and location of emergency trips. Deck officers are familiar with their position and when they are to be used.

# Underwater Operations

    - Senior officers to brief the diving contractors before any underwater diving operation.

    - Keep the turning gear engaged during diving operation. Do not turn the main engine without diving supervisor’s permission.

    - Do not start any operation or equipment that will cause an underwater disturbance and brief the divers about location of disturbance from sea water inlets and discharges.

Note:

Issuance of permit to work is a must before commencement of diving operation.

# PRE-ARRIVAL & PRE-DEPARTURE TESTS

Test the operation of essential systems within 12 hours before arrival at or departure from a port including following.

    - Primary and secondary steering gear

    - Internal control communications and alarms

    - Standby or emergency generator

    - Emergency lighting and power systems in control and propulsion spaces

    - Main engine - ahead and astern

Note:

Master and Chief Engineer may review and vary the details of the test to suit individual power plants and local conditions.

Warning:

Enter the testing in the official log book and if in the USA, include a reference to "USCG title 33 CFR part 164 equipment tests".

The vessel should follow the checks as per the relevant pre-departure/pre-arrival checklist, i.e. E04 & E05.

Note:

Observe local regulation with regards to fuel specifications and follow changeover procedures and testing of the engines.

# OTHER MACHINERY TESTS

# Main engine testing in port

    - Obtain necessary permits of the port authority

    - Ensure that the ship is moored

    - Ensure all cargo hoses are disconnected

    - Ensure propeller clear

    - Ensure that gangway (portable or fixed) has been removed and secured

Note:

Master, in consultation with the Chief Engineer, will determine the safe operation speeds for the engine.

Steering gear

    - Test the steering gear after every maintenance work on the remote-control system and confirm that the rudder moves in the correct direction. This is in addition to normal pre-arrival and pre-departure tests.

# Main air bottles

Before starting use of the main engine for manoeuvring:

    - Pressurize all main engine starting air bottles to the max working pressure (usually 25 –

30 bar).

    - Drain the air bottles of moisture/ condensed vapour and kept ready for use. During manoeuvring:

      - Keep only one air bottle in service. Keep the second air bottle as stand by.

      - Keep the air compressors in the auto mode and monitor the operation of the units.

Caution:

Switch to the stand by air bottle if frequent starting/stopping of the main engine causes the starting air pressure to drop. Inform navigation bridge about reduced availability of the main engine for manoeuvring.

# Clock synchronization

    - Synchronise the bridge and engine room clocks before commencement of any manoeuvre.

Note:

If cannot synchronise, for any reason, make a note in the engine log on completion of manoeuvres, giving the difference in time between the two clocks.

# Documentation of plant performance trials

    - Carry out main engine performance test every month at maximum load (at least 75% of rated MCR power) and with PTO/Fi-Fi disconnected (if applicable).

    - Send performance test report to Vessel Manager every month, and file a copy on-board.

# Failure or repair follow-up

    - Report all significant defects to Vessel Manager, and update on the status of repairs timely and routinely.

    - Get guidance from head office before approaching port, or other areas, for manoeuvring in close waters.

Note

If the defect cannot be rectified, the Master must carry out a risk assessment, before entering the port.

    - Operate recently repaired / maintained equipment under observation before being put into normal use.

    - Carry out such tests in safe waters where a loss of main engine power for several hours will not endanger the safety.

# Tests of critical and essential items

    - Test all critical or essential equipment regularly to confirm that the equipment is operational.

Note:

The jobs and test frequencies to be entered in the planned maintenance system or recorded on checklists or logs. Use E33 & E60 Regular testing of safety system and critical equipment.