## APPLICATION

This document applies to all shipboard personnel.

## PURPOSE

The purpose of this document is to specify requirements and guidance for maintenance of on board systems.

## RESPONSIBILITIES

  - Chief Engineer

    - The following must be maintained in accordance with the Planned Maintenance System (PMS);

    - and supplemented by instructions from the Vessel Manager:

      - All installations and equipment, mechanical and electrical

      - All machinery and moving parts in the engine room

      - Other parts of the ship

    - Planned Maintenance System reporting and database accuracy

    - Ensure that all tools are in good order and marked, and have fixed storing places on board

    - Cleanliness throughout the ship

    - Arrange for a reasonable stock of relevant parts and material on board

    - Take back-up of stored input in the computer every second day on PMS PC

## Chief Officer

    - Coordinate with Chief Engineer to ensure maintenance and testing of deck side machinery and systems are affected

- PLANNED MAINTENANCE SYSTEM (PMS)

## Implementation

Chief Engineer must:

    - Use the excel spreadsheet for reporting and recording of maintenance jobs, until such time the PMS database has been constructed and installed on board

    - Review and verify the accuracy of the PMS, once installed on board, for the following:

      - All vessel specific components are listed

      - Components have correct details about makers, model, type, spares, job frequency, etc

      - Any new equipment retrofitted on board

      - If changes are required in the database, make a change request. See Section

4.7 below

## Maintenance and reporting Chief Engineer must:

    - Generate work orders for the month after updating the PMS with the jobs completed in the last month, and plan with the responsible officers to carry out in a timely manner.

    - Review and evaluate if any of the major/critical jobs cannot be carried out within the month

    - After drawing a list of the jobs coming due in the month and the major jobs coming due in the next 3 months (from the PMS dashboard), send information to office on the ZMP forms, regarding the following items requiring attention from the Vessel Manager, in the appropriate sections:

- Jobs that require shore assistance for completion (example supply of deficient spares or service arrangement).

- The jobs coming due in the month, and that may require rescheduling.

- The jobs marked on PMS as 'critical', coming due in the month, and requiring office concurrence (after risk assessment)

    - If any jobs cannot be completed within the month, see Section 4.4 Overdue items below

    - Report completed jobs on time

    - When reporting completion of a maintenance job, history should include relevant information regarding observations about condition and measurements, and / or reference to the corresponding report in standard TMS format or maker’s format.

    - Update spares inventory module

Caution:

Contact the Vessel Manager for any rescheduling, postponing or clarifications immediately. Do not wait until the end of the month.

Replication files will be automatically replicate.

Note:

Use ZMP/ E09A for jobs which lie outside of the PMS .e.g. deck chipping, grinding, greasing, etc. Send the form to the Vessel Manager every week.

## Vessel Manager must:

    - Review the PMS jobs coming due within the month and discuss with Chief Engineer and Master the plan for completing the major / critical jobs.

## In case any jobs are at risk of NOT getting completed as scheduled, procedures as outlined in sections 4.3 “Overdue items” and 4.4 “Rescheduling of jobs” should be followed.

    - Review at least once a week (preferably on Monday), the status of PMS work orders and confirm that the maintenance jobs are being carried out on board as per schedule.

    - In case any jobs are found overdue, plan on completing these should be discussed with Chief Engineer and Master, and measures taken to complete these within the week, or reschedule these following the procedure outlined in section 4.4 “Rescheduling of jobs”.

Note:

If it is concluded that the component does not show any deterioration in performance, rescheduling / postponement of maintenance may be considered

## Overdue items Chief Engineer must:

    - Inform the Vessel Manager if any jobs cannot be completed within the due date, with a valid reason

Note:

Jobs which cannot be completed within the due date, will be automatically reflected as overdue in the PMS and KPI report.

    - Communicate with the Vessel Manager if these jobs require postponing or rescheduling

WARNING:

Do not report or update any jobs in the PMS which have not been physically carried out on board.

## Rescheduling of jobs Chief Engineer must:

    - Inform vessel manager (VM) with valid reason for rescheduling the job.

    - Provide below listed details when sending request to VM:

      - Component number

      - Work order number based on running hours

      - Job code

      - Present running hours

Caution:

Additional control measures must be identified and implemented in the RA to reduce the final risk.

Risk Assessment (RA) from Risk Management or HSSEQ-18 Risk Assessment

    - Consider the following factors, together the Vessel Manager, when determining rescheduling:

      - Verify if there has been any deterioration in performance when the PMS indicates that a major overhaul is due

      - Where vibration monitoring records are available these shall be used, in conjunction with inspection while running, to determine if performance remains within satisfactory limits

    - Additional Guidance

Rescheduling of PMS job may be required in only extreme circumstances due to unavoidable conditions such as but not limited to:

    - The length of voyage

    - Duration of port stay

    - Non-availability of spares

Note:

Only the Vessel manager shall reschedule the job on PMS.

In the absence of Vessel Manager, rescheduling job shall be completed by Technical Manager.

Non-availability of any resources

# Postponement Duration & Authority

All maintenance jobs (except for those on critical equipment)

    - For the jobs with calendar-based intervals, the target date for postponed jobs should not exceed the time period specified for the job on PMS. For example, a job assigned monthly time period in PMS should not be postponed by more than a month, at a time.

    - For the jobs with running hours or counter based time interval, the target date for postponed jobs should not exceed the interval between the last maintenance and the due date calculated by the system. For example, if the system projects the next due date to be 40 days from the last done date for a certain job, then the job should not be postponed by more than 40 days.

    - The period of postponement should not exceed 3 months, at a time, irrespective of the job interval on PMS.

    - If a job is to be postponed by a period of greater than 3 months or more than three times, from its original due date, approval should be granted by Technical Manager.

    - If a job is to be postponed by a period of greater than 6 months from its original due date, approval should be granted by MD.

    - For jobs where makers have allowed extended overhauling intervals, based on periodic checks, and control measures, Vessel Manager can reschedule, up to the maximum extended interval allowed by the makers, but each postponement should not exceed a period of 2 months.

    - For such cases, approval need not be sought from higher levels (Tech Director or MD) for postponing more than 3 times. Vessel Manager should maintain appropriate supporting documents (maker’s service letter, records of periodic checks in accordance with maker’s instructions etc.) demonstrating that maker’s guidelines have been well complied.

    - For example, if an engine maker allows the unit overhauling interval to be extended up to 24,000 hours (while the interval is set at 16,000 hours on PMS), Vessel Manager can postpone the job for multiple times by a period not exceeding 2 months each time, as long as total postponement duration doesn’t exceed 8,000 hours from the original due date, and records like scavenge space inspection reports and performance reports show that conditions in accordance with maker’s recommendations have been well maintained.

## Critical equipment maintenance jobs

    - If a critical equipment maintenance job requires postponement, the approval should be granted by Technical Manager.

    - At a time, the period of postponement of a critical equipment maintenance job, should not exceed the time period specified for the job in PMS, or maximum one month, if the PMS interval is greater than one month.

    - If a critical equipment maintenance job is to be postponed by a period of greater than three months from its original due date, approval should be granted by Head of Business Unit.

  - Defect reporting and unplanned maintenance

Note:

Always inform and coordinate with Vessel Manager when reporting and closing defects.

Use Defect module (DSM) for all defect reporting and closing or E-24 form

    - Details of the defect with underlying cause should be entered in the description of each Defect item. Actions identified should cover the corrective and preventive measures

identified for the respective defect. Vessel Manager should review and ensure that the description, causes and action items entered are appropriate and adequate.

    - Severity of the defect to be assessed and appropriate severity level (out of the five risk levels) noted on DSM along with the recording of respective defect.

      - The target date for defect rectification should not exceed durations stated below, for the corresponding severity levels:

| Severity level of the defect | Maximum duration for rectification |
| --- | --- |
| Very Low Risk | 90 days |
| Low Risk |  |
| Medium Risk | 60 days |
| High Risk | 30 days |
| Very High Risk | Immediately |

## For a defect to be allowed a longer period for rectification (than that applicable for the corresponding severity level) or for a defect to be assigned the category “Dry-dock”, approval should be granted by Technical Director. For such cases, a comprehensive risk assessment should be carried out, and identified control measures implemented.

    - Create unplanned maintenance job in PMS

    - Enter PMS work order number for unplanned maintenance job as reference in TDL

    - Update PMS and Defect module when rectification is complete.

    - Technical Defect List to be reviewed and updated by Vessel Manager(TSI) regularly (at least once every month), for progress of action items, timely completion, tracking of overdue items, and shifting of relevant items to dry-dock defect list.

Note:

Any equipment defect noted during any inspection like SIRE, PSC etc. to be recorded in Defect module or E-24( incase some issue in DSM )

## Change management request Chief Engineer must:

    - Send change request to the Vessel Manager and technical Director including all relevant details

    - Use A27 Change Management System Revision Request (PMS) for the change request

    - Verify and inform if vessel is in possession of all documentation required for change processing e.g. manuals / drawings / procedure / spares lists etc

    - Provide above documentation to the office if requested

## Postponement of technical defect list

In case of inability to close a technical defect within the target date, the status of the defect should be discussed with the Technical Manager and extension granted with consent from Technical Director.

## ROUTINES FOR PLANNED VS UNPLANNED MAINTENANCE

Sequences of planned vs unplanned maintenance is summarised in Figure 1 below.

| Planned maintenance |  | Unplanned maintenance (breakdown) |
| --- | --- | --- |
| For non-critical equipment |  | For non-critical equipment |
| ↓ |  | ↓ |
| HSSEQ-18 Risk assessment for non- routine job or Daily Work Safety Notice HSSEQ-26 for routine job |  | Deficiency created in defect module |
| ↓ |  | ↓ |
| PMS to be updated after job completion |  | Risk assessment in Risk management /HSSEQ-18 |
|  |  | ↓ |
|  |  | PMS to be updated after job completion |
|  |  | ↓ |
|  |  | Defect module to be updated after job completion |
|  |  | ↓ |
|  |  | DD specification to be created in case job deferred “For Docking” in defect module |
| For critical equipment |  | For critical equipment |
| ↓ |  | ↓ |
| Risk assessment HSSEQ-18/Risk management |  | Deficiency created in defect module) |
| ↓ |  | ↓ |
| E-30 - Critical Equipment Maintenance Permit, Part A to be filled up by ship staff for jobs identified as critical on PMS and send to office along Risk assessment HSSEQ-18/Risk managementfor comments and concurrence |  | Risk assessment HSSEQ-18/Risk management |
| ↓ |  | ↓ |
| E30- Part B to be filled up by Vessel Manager / Tech Director and Marine Superintendent / Marine Manager for office concurrence* |  | E30-Critical Equipment Maintenance Permit, Part A to be filled up by ship staff for jobs identified as critical on PMS and send to office along with Risk assessment HSSEQ-18/Risk management or comments and concurrence |
| ↓ |  | ↓ |
| PMS to be updated after job completion |  | E30- Part B to be filled up by Vessel Manager / Tech Director and Marine Superintendent / Marine Manager for office concurrence* |
| ↓ |  | ↓ |
| E30 will be attached to job order or can be filed together with correspondence with Office |  | PMS to be updated after job completion |
|  |  | ↓ |
|  |  | Defects to be updated in defect module after job completion |
|  |  | ↓ |
|  |  | E30 will be attached to job order or can be filed together with correspondence with Office |
| *Concurrence to be provided by (E30 signed) Tech Director in case the maintenance is unplanned or is being carried out within port limits. |  |  |

## Figure 2. Planned vs unplanned maintenance

Note:

Main Engine and Auxiliary engines are not categorised as critical equipment, however for main engine and auxiliary engines (on those vessels with only 2 AEs), when carrying out maintenance under following circumstances, critical equipment maintenance procedures outlined in chapter 11 of document ‘Critical Equipment’ should be followed:

- Maintenance at sea more than 2 hours;

- Maintenance in port limits (irrespective of duration), other than at a lay by berth or shipyard.

- TECHNICAL MAINTENANCE

  - Technical documentation

    - Instruction books and yard drawings

Chief Engineer shall:

    - Index and store manufacturers' instruction books and ship construction drawings

    - Report missing drawings and instructions to the Vessel Manager

    - Verify the inventory of drawings and instruction manuals at the time of takeover

# Maintenance and repair records

    - Maintain records of all maintenance and repair activities with relevant measurement and observations data and photographs, in accordance with the PMS and TMS requirements.

# Engine log

      - Log books are the only official means of recording operational data relevant to the ship

      - Always meet flag state requirements for log keeping

      - Keep a record of data relevant to specific operations for the benefit of the officers

## SAFETY PRECAUTIONS

Chief Engineer shall make full use of code of Safe Working Practices for Seamen when planning maintenance activities.

## Second Engineer shall:

- Arrange for a daily work list to be prepared

- Conduct tool box meeting

- Notify any changes in the work plan to duty engineer/EOW

## Engineer of Watch (EOW) shall:

Note:

All maintenance activities must be adequately planned and the implications of one task upon the safety aspects of another must be discussed, documented and fully appreciated.

Carry out a full discussion of work at the beginning of each work day and prior to each watch or duty hand-over

## LUBRICATION SYSTEM

  - Lubrication routines

    - A programme of routine lubrication must be specified either :

      - In the PMS or

      - On a charted schedule specific to the ship

    - Lubrication routines shall include all permanent fittings and equipment including, but not limited to the following:

      - Fittings and equipment in the main machinery spaces

      - Fire flaps, skylights, doors and openings

      - Ventilation equipment

      - Winches, windlasses, cranes and other deck machinery

      - Fittings installed in forced ventilation trunking within the machinery spaces

      - Telegraph and tele-motors

      - Refrigeration machinery/compressors

    - Selection of lube oil shall be according to:

      - Manufacturers’ recommendation or

      - As advised by the Vessel Manager

Note:

Lubricant selection for a refrigeration system is critical not only for the compressor but also to ensure that the oil does no separate from the refrigerant gas in any part of the plant.

Topping off with oil of a different grade shall not take place without the prior written agreement by Vessel Manager

## Lubricating oil testing & analysis

Lubrication oil testing & analysis must be done at regular time intervals, in accordance with

vessel’s PMS.

Recommended frequencies for different lubricating oils, is specified below:

Quarterly (Every 3 months) Main Engine System Oil AUX ENGINE System Oil

Main Engine Hydraulic Oil (HPS / Servo Oil) should be tested to the specific standards recommended by the engine maker (not just following the standards for the system oil).

Stern Tube

Bow Thruster Gear Oil

Cargo Oil Pump (or Ballast Pump) Turbine Lube Oil Main Engine Scavenge Drain Oil analyses

## Half yearly (Every six months)

Steering Gear

Deck Machinery (Windlass and Mooring Winches) Hydraulic Oil Hatch Cover Hydraulic Oil

Deck Cranes Hydraulic Oil Grabs Hydraulic Oil

Emergency Generator Crankcase Oil Framo System Hydraulic Oil

Remote Valve Operation Hydraulic Power Unit Oil CPP Hydraulic Power Unit Oil

## Yearly (Every 12 months)

Deck Cranes Gear Oil

Deck Machinery (Windlass and Mooring Winches) Gear Oil

Chief Engineer and Vessel Manager must ensure that lube oil sample analysis is free from deficiencies. Any deficiency indicated by analysis results must be addressed through adequate measures taken by Chief Engineer, with advice from Vessel Manager. Chief Engineer must update Vessel Manager after taking necessary action. The samples must be landed for verification of effectiveness of the corrective measures taken, for re-checking lube oil quality and to ensure it can be used further safely.

On board testing of following lube oils is recommended at monthly interval (as applicable), where test kits are available:

    - Water content for main engine, stern tube, turbines, hydraulic oil / gear oil for deck machinery, and valve remote hydraulic systems oil.

    - Drop test on filter paper for main engine, emergency generator and any other auxiliary engine (Framo).

    - Base Number of the system oil for main engine, emergency generator

    - Scavenge drain oil analyses / scrape down analyses

## Auxiliary engine lubricating oil analysis

    - Auxiliary engine lube oil analyses using filter paper and onboard test kits should be carried out weekly. Following tests should be covered:

      - Filter paper spot test

      - Viscosity

      - Base Number

      - Water in Oil

    - An elaborate laboratory analyses should be carried out every quarter.

    - Any alarming conditions / alerts revealed by lube oil analyses should be acted upon for timely rectification, through appropriate measures.

      - Crankcase lube oil should be completely renewed, in case the oil is tested unfit for further use.

## Calibration of onboard lube oil test kit

- Onboard L.O. analysis test results should be compared with quarterly shore laboratory tests for calibration of the onboard lube oil test kit. For this onboard test kit should be used for testing a sample drawn from the same sampling point at the same time as the sample landed for shore laboratory analysis, and later the test results should be compared.

- Any significant deviations should be noted for future adjustments to the test results from the onboard lube oil test kit.

## MACHINERY WITH OZONE DEPLETING SUBSTANCES

## Management of ozone depleting substances

    - CFC's

    - Halons

Caution:

Do not release following ozone depleting substances into the atmosphere: Halon 1211, Halon 1301, Halon 2402

HCFC's

R-11, R-12, R-113, R-114 and R-115

Carry out leak test of equipment containing such substances on annual basis

    - Remove contents via vacuum pump and stored in appropriate containers whenever systems are emptied

    - If removed from the ship, deliver the container to proper handling facilities

    - Record all inspections and maintenance of refrigeration and halon systems on EMS-02-Refrigerant / Halon log

## Precautions for charging and recovery of refrigerants

For safe and environmentally compliant work on Refrigeration machinery:

    - Read the Material Safety Data Sheet (MSDS) on all compounds including refrigerant and oil that you are likely to come in contact with

    - Take all precautions while carrying out recovery of refrigerants to prevent emission to atmosphere

    - Maintain a log of all refrigerant handling on board (recovery, charging, etc.) as per EMS-02- Refrigerant / Halon Log.

## Precautions for charging and recovery of halon (where applicable)

For safe and environmentally compliant work on halon fixed firefighting systems for servicing, pressure-testing, overhauling, etc.:

    - Deliver halon bottles back to an approved supplier

    - Maintain a log of all halon being handled on board as per EMS-02 -Refrigerant / Halon Log.

WARNING:

Do not release halon from the system intentionally

## HULL AND DECK

- Carry out hull and deck maintenance at schedules described in the computer based PMS and recorded therein

- Use ZMP/E-09A for vessels without a computer based PMS and the Master shall submit it to Vessel Manager weekly

## ELECTRONIC NAVIGATIONAL AIDS & RADIO EQUIPMENT

## Routine maintenance

    - Follow maintenance directions carefully

    - Do not carry out trimming or adjustment other than as specified in the maintenance manual

Caution:

The persons (s) performing routine maintenance and inspections must never effect any repair or operation that they do not fully understand.

This also applies in respect to alterations in wiring diagrams etc

## Faults in equipment

If a defect is discovered:

    - Observe care when subsequently operating the system

    - Submit a report to Head Office for forwarding to the supplier / maker to clarify the cause

    - Include accurate and detailed information and appropriate readings in the report

    - Repairs servicing

## Chief Engineer shall:

    - Ensure that lists of the appropriate makers' service agents are available on board

    - Inform Head Office of all service visit requirements

    - Coordinate their attendance

    - Assign the most suitable officer to follow repair work

    - Sign detailed report from the service technician, including a time-sheet, and counter- signed by the master

    - File the reports according to the filing index for each component

    - Carry out risk assessment and take appropriate control measures in ports where repair is difficult to arrange

## Moisture

    - Operate electric and electronic equipment daily

    - Operate the radar daily, or maintain in the standby mode when not required

## Cleaning

    - Keep equipment clean and free from dust, dirt, and rust through regular inspection and maintenance

    - Apply Vaseline to parts that may rust

    - Cover the appliances and instruments if grinding, wire brushing or similar work is being carried out nearby

## Antennas

    - Navigating officer must inspect the ship's antennas at monthly intervals

    - Carry out any repairs and renewals as recommended by maker

    - Notify Officer of the watch if work is to be carried out on masts or in the vicinity of antennas (radio, radar, VHF, Decca navigator etc.),

Caution:

Installation of any private antenna that may affect the ship's radio station or position fixing system must not be permitted.

Place warning signs on radar and radio transmitters while work is being done

## Spare parts

    - Keep a reasonable stock of spare parts

    - Replace any parts drawn from stock as soon as possible

    - Check components before use

    - Any defective components must be disposed or returned to maker

    - Keep a log of materials for the radio station and Bridge equipment

## Portable radio equipment

The Master shall:

    - Prevent theft of the portable radio equipment and EPIRB's while the ship is in port

    - If necessary, move the equipment to the radio room at port, and return to its usual locations once the ship is at sea again

## Gyro Compass

    - Annual gyro compass maintenance should be carried out by maker authorised service engineers only.

    - If the vessel is provided with two gyro compasses, and one is out of operation, servicing of the other one should not be carried out till the operational status of the first one is restored due to the high risk of failure in the course of or immediately after servicing.

    - Anticipated spares requirement should be identified prior annual gyro compass servicing, and service engineer advised to carry these during the service.

    - Based on the age of the gyro sphere and maker’s recommendation regarding the estimated service life of the component, it should be considered that the service engineer carry a spare gyro sphere when attending, in order that this can be renewed, in case the requirement is identified in the course of servicing.

## SPECIAL MAINTENANCE REQUIREMENTS

## Boilers

Chief Engineers shall:

    - Implement checks for boiler feed water, boiler treatment, blow down and in particular all safety devices as per company’s guidelines, makers’ recommendations and PMS

    - Provide clear, preferably written, instructions which operators clearly understand

    - Use soot blowers to keep the fire sides of the tubes clean

    - Maintain fuel burning equipment for good combustion

    - Check infraphones, if fitted, to ensure that they are operating correctly

# Inspection and cleaning of fire side

    - Examine the condition of the fireside of the tubes:

      - At intervals defined in PMS

      - After long voyage exceeding 30days

      - When the boiler performance indicates a drop in efficiency

    - Shut down the boiler

    - Use a gentle but plentiful application of fresh water, as hot as possible, and at least 76°C through a suitable lance to loosen and wash away the slag

    - Flash up a fire as soon as possible using a small size burner tip to dry the refractory

    - If washing is unavoidable prior to an extended shut-down, dry out the refractory using the forced draught fan and air heater or fire until dry

    - Inspect the refractory carefully and repair as required

Note:

Salt water must not be used under any circumstances for water washing boilers or steam generators.

Refit the doors correctly

Caution:

Permission must be obtained from the Vessel Manager before any boiler is shut down for routine maintenance, if this affects charter speed for the voyage.

Water washing must be avoided prior to an extended shut-down in excess of 48 hours to prevent major corrosion.

All safety precautions must be observed when firing and when washing.

Junior engineers and crew must be supervised in order to maintain adequate safety standards.

# When tubes have been renewed

    - Remove all traces of the oil which is used when expanding the tubes

    - Boil out the boiler to remove the oil in the case of an extensive renewal of tubes

## Electrical systems

    - General

    - Follow maker’s maintenance instructions for particular electrical equipment

# Cargo clusters and handheld electrical equipment

    - Carry out maintenance of cargo clusters, electric drills, and old types of hand lamps

      - Check earth connections for continuity

      - Inspect the cable entries at plug and equipment for chafing and wear

# Batteries

    - Maintain batteries in accordance with the manufacturer’s instructions

# Installations in hazardous areas

    - Maintain all installations in hazardous areas as per maker instructions

    - Maintain flameproof equipment in a safe condition

    - Replace the unserviceable equipment

# Maintenance of intrinsically safe equipment

Caution:

This is particularly relevant in the case of explosion-proof lights where incorrect closing after changing a light bulb could compromise the integrity of the light.

Maintain the integrity of the protection afforded by the design of intrinsically safe electrical equipment

    - Contact Vessel Manager before starting maintenance in cases:

      - Where the manufacturer’s instructions are not available on board

      - In case of any doubt

# Stand by machinery auto start

    - Carry out testing of auto start of stand by machinery (Pumps, Auxiliary Engines, and Compressors etc.) every 3 months.

      - Test may be carried out by simulation or actual stopping of the running machinery, after consideration of all safety aspects.

      - On starting of the stand by machinery, all operational parameters (pressures, temperatures, amperes etc.) and operating conditions (noise and vibration levels, leakage etc.) should be confirmed to be satisfactory.

      - Record the testing of stand by machinery auto start in ER Log Book.

## Marine heat exchangers

    - Isolate the heat exchanger by shutting off the line valves for both media

    - Drain the remaining liquid using the drain cock

    - The vent must be open to ensure everything is drained from the heat exchanger

    - Remove the deposits on the heat exchanger using a wire brush, if not hard

    - Immerse the parts in chemical solution, if the deposits are stubborn

    - Refer to maker’s instruction regarding cleaning method and correct application of special tools

    - Once cleaning is done, flush the heat exchanger with fresh water to remove any remaining chemical or dirt from the surface, followed by air blowing using clean dry air to dry up the surfaces

    - Check the sacrificial anodes and change if required

    - Always renew the cover gasket if it is damaged during opening of heat exchanger

    - In oil coolers and heaters, clean oil side of tubes by chemical flushing using carbon

remover or similar chemicals in accordance with maker’s instructions.

    - In plate type heat exchangers:

      - Remove the stack of plates to expose the surface.

      - Clean the plate surface with brush or by the methods suggested by the manufacturer.

      - Clean in such a way that it does not damage the plate seals.

      - Replace any damaged plate seals.

Caution:

Usage of sharp tools for cleaning should be avoided

Carry out even tightening of all the exchanger studs and bolts while tightening the plates together

## Lifting equipment

See FOM 10.10 Personal safety and Cargo gear for further details

    - Inspect lifting equipment, including chain blocks, beams and trolleys, wire ropes and slings, every month

    - Carry out a thorough annual inspection

    - Record the results of such inspections in the cargo / lifting gear book

    - Person carrying out the inspection should sign the record On ships fitted with elevators:

    - Carry out regular maintenance as required by the manufacturers

    - Enter any elevator maintenance in the register and get it signed by the inspector

## Wire replacement for gangway/accommodation ladders

    - Wire replacement frequency is 12 months

    - Replace wires with spares of similar specification as original

## Life boat maintenance

Recommendations for lifeboat maintenance and drills:

    - Wire replacement frequency is 30 months

    - Secure the lifeboat during maintenance (e.g. hanging pendants, chain blocks, etc.)

    - Replace corroding parts as necessary

    - Replace spare parts at specified lifetime intervals

    - Verify proper operation of limit switches

    - Keep fuel tanks in a topped up condition

    - Test life boat rigging after any maintenance

    - Perform testing or launching of the lifeboat without personnel

    - Recognize and adhere to the difference between mustering (no embarkation) and launching of lifeboat

    - To prevent parting of lifeboat falls or tackle, Chief Officers:

      - To personally verify the operation of all limit switches

      - To add checking of falls to the weekly routine

Caution:

Painting of moving parts on LSA systems should never be done

To remind crew on each occasion to use care when painting so as not to seize the roller or other moving parts

## Ballasting equipment maintenance and inspection

Note:

Inspect once in 6 months in duct keel and during ballast tank inspections.

Inspect the ballast water pipelines for leakages and corrosion

    - Apply pressure of 3 kg/cm2 on the ballast system pipelines during the above inspection

    - Closely examine all joints, couplings, nuts and bolts, clamps and supports

    - Prevent corrosion on the ballast pipelines by applying suitable paint.

    - Inspect the ballast tanks as per PMS routine

    - Inspect the ballast pumps and overhaul as per the manufacturer’s instruction or more

frequently if required

## Tank gauging systems

    - All local and remote tank gauges (for measurement of level, temperature and pressure) for cargo, ballast, fuel oil, lube oil and cooling water tanks shall be tested at intervals stated below:

      - Weekly – Carry out functional check

      - Annual – Calibration by ship staff using a recently calibrated and certified master gauge

      - 5 yearly – Servicing and calibration by a recognized company

## Auxiliary engine

    - Guidelines for the vital maintenance routines on auxiliary engines should be complied as stated in the table below.

Caution:

In case any deviations are necessitated, a thorough risk assessment should be carried out, recorded on HSSEQ-18, and appropriate control measures should be identified and strictly implemented. Approval shall be taken from Tech Director or the Head of Business Unit.

## Auxiliary engines vital maintenance routines

| Maintenance Routine | Maintenance Strategy |  |
| --- | --- | --- |
| Governor servicing |  | Maker authorised workshop to be engaged for governor overhauling and installation on the engine, along with carrying out of the trials and settings. Genuine spares to be used. |
|  |  | Caution: In case of an eventuality, where a maker authorised servicing company is unavailable and the ship staff is required to carry out replacement of existing |
|  | AE governor (with a governor overhauled by an authorised workshop), following should be strictly followed: - Risk assessment carried out and stringent control measures enforced. - Approval sought from TD or the MD. - A thorough and clear procedure for the job obtained from the maker should be shared with the ship staff. A good understanding of the procedure should be confirmed through a discussion with the Chief Engineer. |  |
| Governor drive gear checking | Gear teeth condition to be closely checked Pictures to be clicked and sent to office |  |
| Auxiliary Engine unit overhaul | Auxiliary Engine Overhaul Preparatory Checklist’- E43A - Overhaul (requiring opening up and reassembling of the piston and connecting rod) should be planned well in advance (minimum three months prior scheduled overhauling date),. - For unscheduled/breakdown maintenance (requiring opening up and reassembling of the piston and connecting rod), preparatory checks should be carried out in accordance with Maker manual. - E43A to be filled up by Chief Engineer and reviewed by Vessel Manager, to ensure due preparations are carried out for AE overhaul and any concerns are addressed. During overhaul/repair - Vital checks concerning conrods, bottom ends and bolts (covered in E43 / E43A and the maker’s manual) should be carried out diligently and measurements recorded in E43 ‘Four stroke engine overhaul report’ and shared with the Vessel Manager. - Chief Engineer should be present and carry out (or closely supervise) the assembly and checking of the connecting rods, bearings and bolts. - Vessel should report daily on the overhauling progress, along with information regarding observations on the condition and any concerns, or unforeseen problems. Daily report to include following items as a minimum: - Measurements carried out concerning the bottom end assembly (conrod bore & ovality, crankpin diameter & ovality, bolts length, ridge wear etc.) - Photographs of crankpin, conrod bottom end halves (with serrations), bearing shells Assembly should be carried out only after receiving a green signal from office based on the assessment of Chief Engineer’s reports. After overhaul/repair - Before starting the AE, the Tech Director should be consulted regarding the critical items (including the observations/readings for bottom end and crankpin). - A go ahead for testing at each stage should be obtained from office based on CE’s feedbacks from the checks, assembly and crankcase inspections. |  |
| Turbocharger overhauling | Genuine spares to be used Where re-conditioning is required, maker authorised workshop to be engaged A spare overhauled cartridge to be maintained on board |  |
| Mechanical Overspeed Trip Device | Mechanical overspeed trip device to be renewed every 5 years (instead of bearing renewal) Overspeed trip setting to be confirmed to be conforming to makers’ specifications |  |
| Connecting Rods re- conditioning | Connecting rods re-conditioning to be carried out only by maker approved workshops |  |
| Flywheel gear and start motor pinion teeth inspection | Inspection routine should be covered in PMS at maker recommended interval or quarterly (in case no periodicity is recommended in maker’s manual) |  |

## WIRE ROPE INTEGRITY MANAGEMENT PLAN

The Company has developed wire rope integrity management plans. The following plans must be referred to for documentation, procurement, maintenance, inspection, evaluation and replacement:

# FOM 5.5 Wire rope integrity management plan

## SOFTWARE MAINTENANCE OF SHIPBOARD EQUIPMENT

All vessels shall maintain a log of onboard software for the following systems:

- Power generation systems

- Propulsion systems

- Control and alarm systems

- Navigation and communication systems

- Steering control systems

- Ship management systems

- Performance systems

- Fire and water mist systems

  - Auxiliary systems

  - The log is a record of the previous and current software versions (for the abovementioned systems) including all related service report documents.

  - The log shall be updated whenever updates, in accordance with the maker, have been made.

  - Updates may be classified as the following:

    - Bug fix (resolving software bugs).

    - Feature release (adding additional functionality).

    - Compliance update (maintaining conformity with regulations).

    - Security update (protecting against cybersecurity threats).

    - Obsolescence update (addressing software and/or hardware that is no longer supported).

  - Send Log to the Vessel Manager whenever changes are made.

  - A record of shipboard IT systems is maintained by Group IT separately and need not be captured on the log.

  - Newly taken over vessels shall complete the log prior to the first internal audit.

Note:

Updates, as defined above, shall only be made to shipboard systems in accordance with maker’s

instructions or guidance or as per the planned maintenance system where relevant.