It is recommended that Masters verify checks as guided below at least 96 hours prior arrival for smooth implementation.

# PUBLICATIONS

| No. | Publications required to be on board: |  | International Requirement |  | U.S. Requirement |  | Applicability and comments |  | On board |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Yes |  | No |  |
| 1. | Current and Corrected Charts |  | SOLAS V/19.2.1.44 |  | 33 CFR 164.33 |  | If using BA charts you must also carry Cumulative Notices, Annual Notices and Admiralty Chart Catalogue. |  |  |  |  |  |
| 2 | U.S. Coast Pilot |  | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 3 | Sailing directions |  | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 4 | Coast Guard Light List |  | N/A |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 5 | Tide Tables |  | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 6 | Tidal Current Tables |  | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 7 | International Rules of the Road |  | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 8 | Inland Rules of the Road |  |  |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 9 | International Code of Signals |  | SOLAS V/21 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| 10 | Current edition of 33 CFR Parts 1-169 |  |  |  |  |  | Available as part of combined USCG Publication #515 (33 CFR and 46 CFR) |  |  |  |  |  |
| No. | Publications required to be on board: | International Requirement |  | U.S. Requirement |  | Applicability and comments |  | On board |  |  |  |
|  |  |  |  |  |  |  |  | Yes |  | No |  |
| 11 | Current edition of 46 CFR Parts 1-149 | N/A |  |  |  | Available as part of combined USCG Publication #515 (33 CFR and 46 CFR |  |  |  |  |  |
| 12 | Current edition of 46 CFR Parts 150 and 153 | N/A |  | 46 CFR 153.905 |  | Tank Vessels only, in US waters |  |  |  |  |  |
| 13 | Current edition of 49 CFR 172 and 176 | N/A |  | 49 CFR 176.1 |  | US only, applies only to vessels carrying dangerous cargoes in bulk |  |  |  |  |  |
| 14 | Notice to Mariners | SOLAS V/27 |  | 33 CFR 164.33 |  |  |  |  |  |  |  |
| NOTE Some flags require vessels to carry SOLAS, MARPOL, IMO Ship’s Routing Guide, ColRegs, GMDSS Handbook, IMDG, IAMSAR, etc. while others just recommend it. |  |  |  |  |  |  |  |  |  |  |  |

- MANUALS

| No. | Publications required to be on board: | International Requirement | U.S. Requirement | Applicability and comments | On board |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Yes | No |
| 1. | Cargo & Ballast Information |  | 33 CFR 157.23 |  |  |  |
| 2. | Dedicated Clean Ballast Operations |  | 33 CFR 157.208 |  |  |  |
| 3. | SOPEP / SMPEP | MARPOL AxI /26.1 | 33 CFR 151.26 | IMO Coastal Contact List changes quarterly |  |  |
| 4. | NTVRP | N/A | 33 CFR 155.1030 |  |  |  |
| 5. | Bridge Log |  | 33 CFR 164.25 |  |  |  |
| 6. | Muster list & emergency instructions | SOLAS III/8 |  |  |  |  |
| 7. | Fire control plan | SOLAS II-2/15, 2.4 |  |  |  |  |
| 8. | Garbage Management Plan | MARPOL Ax V/9 | 33 CFR 151.57 |  |  |  |
| No. | Publications required to be on board: | International Requirement | U.S. Requirement | Applicability and comments | On board |  |
|  |  |  |  |  | Yes | No |
| 9. | Intact Stability booklet | SOLAS II-2/22 |  |  |  |  |
| 10. | Damage Control Plan | SOLAS II-2/23 | N/A |  |  |  |
| 11. | Oil Transfer Procedures |  | 33 CFR 155.720 | Specific additional US requirements |  |  |
| 12 | Procedures and Arrangements Manual | MARPOL 73/78, Annex II/ Resolution 5 & 8 | 46 CFR 153.490 | Chemical carriers only, Resolution MEPC.18(22) |  |  |
| 13 | Ballast Water Management Plan | International Convention for the Control and Management of Ship’s Ballast Water and Sediments | 33 CFR 151.2035 (a)(7) | Not yet ratified. BWMP’s required in US and Canada (Canada requires class approval) |  |  |
| 14 | Ship Security Plan | SOLAS XI-2 A/9.1 | 33 CFR 104.400 |  |  |  |
| 15 | Life Saving Equipment Training Manual | SOLAS III/35.2 |  |  |  |  |
| 16 | Fire Training Manual | SOLAS II-2/15, 2.3.1 |  | Available in each crew mess and/or recreation room |  |  |
| 17 | Grain Loading Manual |  |  | (Bulk carriers only) requires DOA – Document of Authorization. Class approved and in compliance with US Dept. of Agriculture and the International Grain Code. |  |  |
| 18 | Cargo Securing Manual | SOLAS 74, Reg VI/5 |  | MSC/Circ. 745 |  |  |

- PLACARDS REQUIRED ONBOARD VESSELS WHEN OPERATING IN US WATERS

| No. | Type of Placard | Location Required On Board | Reference: | On board |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Yes | No |
| 1 | Discharge of Oil Prohibited | Prominently posted in engineering spaces, engine control room, bunkering stations, and the bilge pumping station, etc. Specific wording is listed in reference citation. | 33 CFR 155.450 |  |  |
| 2 | MARPOL Annex V Garbage Discharge Placard | Posted in galley, mess rooms, and common spaces throughout the ship. Specific wording is listed in the reference citation | 33 CFR 151.59 MARPOL Ax. V/9 |  |  |
| 3 | Oil Transfer Procedures | Permanently posted or available at a place where the procedures can be easily seen and used by members of the crew when engaged in transfer operations. | 33 CFR 155.720 33 CFR 155.740 33 CFR 155.750 |  |  |
| 4 | Steering Gear Instructions, Block Diagram for Changeover | Simple operating instructions and a block diagram showing the change-over procedures for remote control systems. This must be posted at the steering station on the bridge and in the steering gear room. | 33 CFR 164.35 |  |  |
| 5 | Manoeuvring Diagram | A manoeuvring diagram must be prominently posted in the wheelhouse. This includes a turning diagram, manoeuvring information, and a warning at the bottom of the fact sheet. Specific information which must be included is listed in the citation. | 33 CFR 164.35 |  |  |
| 6 | Marine Sanitation Device | Each MSD must have a placard on which the operating instructions, safety precautions, and warnings pertinent to the device. The device must also have a placard which provides the information required by 33 CFR 159.55 | 33 CFR 159.59 33 CFR 159.55 |  |  |

- LOGS

| No. | Publications required to be on board: | International Requirement | U.S. Requirement | Applicability and comments | On board |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Yes | No |
| 1. | Lifesaving equipment maintenance | SOLAS 74/78 III/19 |  |  |  |  |
| 2 | Emergency training and drills | SOLAS 74/78 III/18 |  |  |  |  |
| 3 | Declaration of Inspection | N/A | 33 CFR 156.150(F) | US requirement only |  |  |
| 4 | Proof of transfer hose tests | N/A | 33 CFR 155.820 | Must be conducted annually |  |  |
| 5 | Proof of hydrostatic tests of fixed bunker lines | N/A | 33 CFR 156.170 | Must be conducted annually |  |  |
| 6 | Bridge log | STCW 95 I/14 | 33 CFR 164.25 | SOLAS V/28 |  |  |
| 7 | Ship Security Log/Records | SOLAS XI-2 A/10.1 | 33 CFR 104.235 |  |  |  |
| 8 | Cargo Record Book | MARPOL 73/78 | 33 CFR 151.25 |  |  |  |
| 9 | Oil Record Book | MARPOL AxI/20 | 33 CFR 151.25 |  |  |  |
| 10 | Ballast Water Management Log | International Convention for the Control and Management of Ship’s Ballast Water and Sediments | 33CFR 151.2045 | Not yet ratified. BWM log’s required in US and Canada |  |  |
| 11 | Garbage Record Book | MARPOL 73/78 Annex V, Reg 9 | 33CFR 151.55 (b) |  |  |  |
| 12 | Drug & Alcohol Testing Log | N/A | 46 CFR 16.260 | Owners must maintain records of chemical tests and results |  |  |
| 13 | Pre-Arrival tests of emergency equipment | N/A | 33 CFR 164.25 | US only |  |  |
| 14 | National Pollutant Discharge Elimination System (NPDES ) Weekly Checklist | N/A | 40 CFR 9, 122,142 VGP 4.2 (9) | US only |  |  |
| No. | Publications required to be on board: | International Requirement | U.S. Requirement | Applicability and comments | On board |  |
|  |  |  |  |  | Yes | No |
| 15 | Grey water estimated discharge log | N/A | VGP 4.2 (8.ix) | US only, need not be kept as an actual log so long as the records are on board |  |  |
| 16 | Record of Hazardous Material Training for the crew | N/A | 49 CFR 172.704 | US only, bulk/container vessels carrying dangerous materials in bulk |  |  |