| Inspection place | Check-out point maintenance item | Company maintenance interval | Inspection date | Remark |
| --- | --- | --- | --- | --- |
| 1 - Scrubber |  |  |  |  |
| High level alarm/float | Press down the level switch and check for alarm After checking level switch must be release at original position | Pre-discharge |  |  |
| Cooling water Alarm lamp of flow switch | To check alarm at I.G. cooling water pumps stopped & IG Blower stop | Pre-discharge |  |  |
| Scrubber cooling water | Check cooling water pressure | Pre-discharge |  |  |
| Scrubber inside lining | Condition of the coating / lining. Any corrosion. | 6 months |  |  |
| Scrubber inside spray nozzle | Run Scrubber pump, check spray is proper and nozzles clogged | 6 months |  |  |
| Scrubber inside sludge | Check the extent of sludge accumulation. Removal of sludge. | 6 months |  |  |

| Scrubber outside | Check for any damage and welding condition | 6 months |  |  |
| --- | --- | --- | --- | --- |
| Scrubber sight glass | Check level & clarity the sight glass (not fouled) | During every operation |  |  |
| U-seal part of scrubber drain | To check after taking out U-seal blind flange | 6 months |  |  |
| 2 - Demister |  |  |  |  |
| Inside condition | Wash with fresh water, and check wire mesh filter for wastage. Ensure demister covers the entire area and not broken. | 3 months |  |  |
| 3 - Piping arrangement |  |  |  |  |
| Isolating valve | Valve overhaul | Dry docking |  |  |
| IG Non-return valve- Mechanical | Internal inspection of flap and metal seat ring. | 6 months |  |  |
| IG non-return valve- Mechanical | Move & lubricate the valve | 1 week |  |  |
| Main line | Leakage, corrosion | During every operation |  |  |
| Branch lines | Leakage, corrosion | During every operation |  |  |
| 4 - Deck seal unit |  |  |  |  |
| Seal water | Sealing water flow | During every operation |  |  |
| Low/High level alarm | Check alarm functioning | Pre-discharge |  |  |
| Float | Check condition of float | 3 months |  |  |
| Inside coating | Check internal coating | 6 months |  |  |
| Water tank inside sludge | Check for sludge accumulation | 6 months |  |  |
| Anode condition | Check wastage of anode, if required renew | 6 months |  |  |
| Steam pipe | Check condition of steam piping for corrosion / leakage. | 6 months |  |  |

| 5 - Fan unit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Impeller | Water washing and to check condition | Every voyage / operation |  |  |
| IG blower casing drain | Ensure drain is not clogged and water is drained out | Every voyage / operation |  |  |
| Inside coating | to check coating condition | Every voyage / operation |  |  |
| Bearing temperature | Maximum 90°C | During every operation |  |  |
| Bearing vibration / condition | Maximum 30/1000mm | During every operation |  |  |
| Grease up | Greasing of bearings as applicable | 3 months |  |  |
| Dismantling full | Dismantling for bearing, shaft tightening and other necessary work. (Impeller Balancing) | Dry docking |  |  |
| 6 - Boiler up-take valve |  |  |  |  |
| Spindle grease up | Greasing / lubrication of spindle | Every voyage / operation |  |  |
| Gland packing | To check condition & renew if required | Every voyage / operation |  |  |
| Valve | To check condition - smooth opening closing, steam / air blow (if arrangement provided) | Every voyage / operation |  |  |
| Soot Blower Interlock | Check Soot blower interlock for IG is operational (if given) | Monthly |  |  |
| 7 - Control valve |  |  |  |  |
| Lubricator | Check for proper functioning | Monthly |  |  |
| Gland packing | Check for leaks | Monthly |  |  |
| Inside rubber seat | Check condition of seat for any damage | Dry docking |  |  |
| 8 - Scrubber OVBD water valve |  |  |  |  |
| Limit Switch | Check interlock is working | Monthly |  |  |
| Gland packing | Check for leaks | Every voyage / operation |  |  |
| Inside rubber seat | Check condition of seat for any damage | Dry docking |  |  |
| 9 - Control air system |  |  |  |  |
| Panel header | Drain out from header | Every voyage / operation |  |  |
| Air filter | Air filter to clean | Monthly |  |  |
| Regulator | Check regulator is working | Monthly |  |  |
| Lubricator | Check for proper functioning | Monthly |  |  |
| 10 - Fixed oxygen analyser |  |  |  |  |
| Sampling Gas pipe | To check pipe condition – pipe is clear and not clogged | Every voyage / operation |  |  |
| Calibration by standard gas | Calibration | Before every voyage / operation |  |  |
| Cleaning | Check & clean filters and sampling gas pipe | After every operation |  |  |
| Check oxygen % readings | Check EEC, CCR & Bridge Oxygen % Readings match | Every voyage / operation |  |  |
| Standby oxygen analyser | Vessels provided with STDBY oxygen analyser to check and calibrate it. | Every voyage / operation |  |  |
| 11 - Inert gas generator |  |  |  |  |
| Main & Pilot Burner | Pilot Burner – Clean atomizing tip & check for nozzle wear. Main Burner – Check the burner is clean and atomising tip is clear. Check rotary burner belt condition | Every voyage / operation |  |  |
| Blower failure and flame failure | Check alarm and closing of valve | Pre-discharge |  |  |
| FD Fan | If applicable - Check v-belt tension and condition, apply v- belt dressing | Monthly |  |  |
| Combustion chamber | Check condition for any rust or wear | Monthly |  |  |
| Burner Interlocks | Check the interlocks are functional. | Pre-discharge |  |  |
| Dew point temperature | Check that required dew point temperature is achievable (For LPG & LNG Tankers, will depend on cargo carried). | Every voyage / operation |  |  |
| 12 - Pressure vacuum valves & high velocity valves |  |  |  |  |
| Operation valves | Lubricate and operate the valves for free movement | 6 months |  |  |
| Overhaul | Opening for complete overhaul and inspection | 1 year |  |  |
| 13 - Liquid filled pressure vacuum breaker |  |  |  |  |
| Check liquid level | Check level of Glycol at atmospheric pressure | As and when required & every 6 months |  |  |
| 14 – Automation & control system check |  |  |  |  |
| Automation and control system | Check automation & control system servicing by technician | Dry docking |  |  |

Note:

- Refer to Manufacturer’s Instructions.

- The valve settings and accessories may vary on different ships, depending on the design of the insulation.

- If any point is not applicable to your vessel write Not Applicable or N/A.

Following Pictures to be taken at the time of maintenance:

- IG Blower 1 & 2 condition of internals and fans

- Scrubber –

  - Spray nozzles

  - Demister

  - Interior lining condition

  - Float

  - External

- Uptake valve

  - Open and shut condition showing indicators

- Deck non-return valve

- Deck isolation valve

- Deck seal

  - Demister

  - Internal condition

  - Float

  - External from two directions