| Ship Name | : |  |
| --- | --- | --- |
| IMO Number | : |  |
| International Call Sign | : |  |
| Flag State | : |  |
| Port of registry | : |  |
| Date of Register with the Flag State | : |  |
| Ship Type | : |  |
| Shipyard and Hull No | : |  |
| Class ID | : |  |
| Ship Owner Name and Address | : |  |
| Date of delivery | : |  |

Revision history

| Current Version | Created/Amended by | Date | Description of Change |
| --- | --- | --- | --- |

1	INTRODUCTION

The objectives of the inventory are to provide vessel-specific information on the hazardous materials present on board, to protect the health and safety of personnel onboard and to allow for planning of waste disposal by ship recycling facilities to prevent environmental pollution.

During the lifetime of a vessel, the vessel will undergo various changes due to repairs, maintenance, additions, removals and modifications to the vessel. To have an accurate inventory at the end of the lifetime of a vessel, it is important to establish a system to track the changes to the vessel that affect the inventory.

The inventory required under the Hong Kong Convention comprises of 3 parts. Only Part I of the Inventory, which covers materials containing in the vessel’s structure or equipment, need to be developed and maintained throughout the lifetime of the vessel. The maintenance of Part I of the Inventory, as required by the guidelines, is to be through supplier’s declarations. The focus of the system to maintain the inventory is therefore on obtaining and screening Material declarations (MDs) and Supplier’s Statement of Conformity (SDoCs) for hazardous materials for all purchase and use of spare parts and consumables for repair and maintenance of vessel’s structure equipment.

In addition to establishing the above monitoring system, this maintenance manual is also designed to provide records of the designated person in charge of the monitoring system, records of material declarations (from new installations, purchases of spare parts and consumables for repair and maintenance of structure and equipment) where hazardous material above their respective threshold values and reported and records of changes to the inventory (additional/deleted entries, dates of changes and signature of designated person) for ease of verification. As many of the hazardous materials have been banned by international Regulations/Conventions, it is expected that there should be minimal entries into the maintenance record and changes to the inventory.

2	SYSTEM FOR MAINTAINING THE INVENTORY

- Designated Person.

- Chief Engineer shall be the designated person to oversee the inventory and its maintenance.

3	ESTABLISHMENT OF MANAGEMENT SYSTEM TO MAINTAIN THE INVENTORY

The DP establishes and supervises a system as per below procedures in order to ensure that the Inventory onboard is properly updated and maintained.

4	PROCESS FLOW

- Purchasing of Materials

  - As the maintenance of the inventory is through supplier’s declarations, the purchase contact for the vessel shall be informed to always include a request for MDs and SDOCs in their Purchase Orders (POs) for all purchases of equipment, materials for vessel’s structure and spare parts / consumables for the structure and equipment. A copy of

POs for the above purchases shall be provided to the designated person to allow tracking of receipt of MDs and SDOCs for these purchases.

- Screening and Storage of MDs

  - The MD and SDoC received along with the delivery of the purchase shall be provided to the designated person. The designated person will then check that each MD is accompanied by a SDoC, the information on the MD is complete and screen each MD for hazardous material above the threshold value.

  - If MDs show no hazardous materials present above the respective threshold values, the MDs shall be scanned and filled. The process ends here.

  - If MDs shows hazardous materials above the respective threshold values, the designated person is to check if the hazardous materials and its quantity are allowed to be installed onboard. If it is allowed, move on to (3). If it is not allowed, report to both vessel manager and purchaser.

  - All the MDs, SDoCs and POs collected shall be filled accordingly.

- Handling of MDs that contain hazardous materials

  - If there are MDs that shows hazardous materials present above the respective threshold values ad are permitted to be installed onboard, the designated person shall follow up to track the installation/use onboard the vessel and update this maintenance record and the inventory MDs shall be scanned and filled.

5	RECORDS

Designated Person - Chief Engineer

| S. No. | Name of Chief Engineer | Period in charge (dd/mm/yyyy) |  | Signature |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |

Record of all new installations, repairs, maintenance, modifications to the inventory

| Date of installation | MD No. | SDoCs | POs | For Equipment/Structure/Spare part/Consumables | Name of Person/Position | Signature of designated person |
| --- | --- | --- | --- | --- | --- | --- |

Record of changes to IHM

| Date | Description (change to the IHM) | Name of Person/Position | Signature of designated person |
| --- | --- | --- | --- |